EC25&EC21

AT Commands Manual

LTE Module Series

Rev. EC25&EC21_AT_Commands_Manual_V1.3 Date: 2018-09-20

Status: Released

www.quectel.com

Our aim is to provide customers with timely and comprehensive service. For any assistance, please contact our company headquarters:

Quectel Wireless Solutions Co., Ltd.

7th Floor, Hongye Building, No.1801 Hongmei Road, Xuhui District, Shanghai 200233, China Tel: +86 21 5108 6236

Email: info@quectel.com

Or our local office. For more information, please visit:

http://www.quectel.com/support/sales.htm

For technical support, or to report documentation errors, please visit:

http://www.quectel.com/support/technical.htm Or email to: support@quectel.com

GENERAL NOTES

QUECTEL OFFERS THE INFORMATION AS A SERVICE TO ITS CUSTOMERS. THE INFORMATION PROVIDED IS BASED UPON CUSTOMERS‟ REQUIREMENTS. QUECTEL MAKES EVERY EFFORT TO ENSURE THE QUALITY OF THE INFORMATION IT MAKES AVAILABLE. QUECTEL DOES NOT MAKE ANY WARRANTY AS TO THE INFORMATION CONTAINED HEREIN, AND DOES NOT ACCEPT ANY LIABILITY FOR ANY INJURY, LOSS OR DAMAGE OF ANY KIND INCURRED BY USE OF OR RELIANCE UPON THE INFORMATION. ALL INFORMATION SUPPLIED HEREIN IS SUBJECT TO CHANGE WITHOUT PRIOR NOTICE.

COPYRIGHT

THE INFORMATION CONTAINED HERE IS PROPRIETARY TECHNICAL INFORMATION OF QUECTEL WIRELESS SOLUTIONS CO., LTD. TRANSMITTING, REPRODUCTION, DISSEMINATION AND EDITING OF THIS DOCUMENT AS WELL AS UTILIZATION OF THE CONTENT ARE FORBIDDEN WITHOUT PERMISSION. OFFENDERS WILL BE HELD LIABLE FOR PAYMENT OF DAMAGES. ALL RIGHTS ARE RESERVED IN THE EVENT OF A PATENT GRANT OR REGISTRATION OF A UTILITY MODEL OR DESIGN.

Copyright © Quectel Wireless Solutions Co., Ltd. 2018. All rights reserved.

# About the Document

History

Revision	Date	Author	Description

1.0	2016-05-30

1.1	2017-01-18

James CAI/ Bonnie ZHAO

Ivan ZHANG/ Sophie ZHU

Initial

- Added the following AT commands:

+QRIR/+QINDCFG/+QINISTAT/+QLTS/+QSPN/

+QHUP/+QEEC/+QSIDET/+QMIC/+QRXGAIN/

+QIIC/+QADC

- Added AT+QCFG commands shown as below: "urc/ri/ring"/"urc/ri/smsincoming"/"urc/ri/other"/"risig naltype"/"urc/delay"/"urc/cache"

- Updated the description for <Act> parameter in AT+COPS command: added the value 100 which indicates CDMA network

- Added commands AT+QTONEDET, AT+QLDTMF, AT+QLTONE and AT+QCFG=“tone/incoming”

- Updated the description of parameter <ci> in commands AT+CREG, AT+CGREG and AT+CEREG

- Deleted command AT+QCFG="tdscsq" and the description    of    +QURCCFG?    in     command

AT+QURCCFG

1.2	2017-11-14	Jessica GENG

Alessa TANG/

- Updated the description of commands AT+QUCCFG, AT+CTZU, AT+QDAI, AT+QEEC, AT+QIIC and AT+QCFG="band"

- Updated the examples of commands AT+QIIC and AT+COPS

- Deleted some related information about CDMA

- Updated the supported baud rates of command AT+IPR

- Added Chapters 14~20.

1.3	2018-09-20

Jessica GENG/

- Added "ccinfo" as a supported value of <urctype>

Demi QU	in AT+QINDCFG.

- Updated the range of <index> in AT+QEEC.

- Updated the default value description of <st_gain> in AT+QSIDET.

Contents

About the Document	2

Contents	3

Table Index	8

- Introduction	9

  - Scope of the Document	9

  - AT Command Syntax	9

  - Supported Character Sets	10

  - AT Command Interface	10

  - Unsolicited Result Code	10

  - Turn off Procedure	11

- General Commands	12

  - ATI	Display Product Identification Information	12

  - AT+GMI	Request Manufacturer Identification	13

  - AT+GMM	Request TA Model Identification	13

  - AT+GMR	Request TA Revision Identification of Software Release	14

  - AT+CGMI	Request Manufacturer Identification	14

  - AT+CGMM	Request Model Identification	15

  - AT+CGMR	Request TA Revision Identification of Software Release	15

  - AT+GSN	Request International Mobile Equipment Identity (IMEI)	16

  - AT+CGSN	Request Product Serial Number Identification	17

  - AT&F	Set all Current Parameters to Manufacturer Defaults	17

  - AT&V	Display Current Configuration	18

  - AT&W	Store Current Parameters to User Defined Profile	19

  - ATZ	Set all Current Parameters to User Defined Profile	19

  - ATQ	Set Result Code Presentation Mode	20

  - ATV	TA Response Format	20

  - ATE	Set Command Echo Mode	22

  - A/	Repeat Previous Command Line	22

  - ATS3	Set Command Line Termination Character	23

  - ATS4	Set Response Formatting Character	24

  - ATS5	Set Command Line Editing Character	24

  - ATX	Set CONNECT Result Code Format and Monitor Call Progress	25

  - AT+CFUN	Set Phone Functionality	26

  - AT+CMEE	Error Message Format	27

  - AT+CSCS	Select TE Character Set	28

  - AT+QURCCFG	Configure URC Indication Option	29

- Serial Interface Control Commands	31

  - AT&C	Set DCD Function Mode	31

  - AT&D	Set DTR Function Mode	31

  - AT+IFC	Set TE-TA Local Data Flow Control	32

  - AT+ICF	Set TE-TA Control Character Framing	33

  - AT+IPR	Set TE-TA Fixed Local Rate	34

  - AT+QRIR	Restore RI Behavior to Inactive	35

- Status Control Commands	37

  - AT+CPAS	Mobile Equipment Activity Status	37

  - AT+CEER	Extended Error Report	38

  - AT+QCFG	Extended Configuration Settings	39

    - AT+QCFG="gprsattach"	GPRS Attach Mode Configuration	40

    - AT+QCFG="nwscanmode"	Network Search Mode Configuration	41

    - AT+QCFG="nwscanseq"	Network Searching Sequence Configuration	42

    - AT+QCFG="roamservice"	Roam Service Configuration	43

    - AT+QCFG="servicedomain"	Service Domain Configuration	43

    - AT+QCFG="band"	Band Configuration	44

    - AT+QCFG="hsdpacat"	HSDPA Category Configuration	46

    - AT+QCFG="hsupacat"	HSUPA Category Configuration	46

    - AT+QCFG="rrc"	RRC Release Version Configuration	47

    - AT+QCFG="sgsn"	UE SGSN Release Version Configuration	48

    - AT+QCFG="msc"	UE MSC Release Version Configuration	48

    - AT+QCFG="PDP/duplicatechk"	Establish Multi PDNs with the Same APN	49

    - AT+QCFG="urc/ri/ring"	RI Behavior When RING URC is Presented	50

    - AT+QCFG="urc/ri/smsincoming"	RI  Behavior  When  Incoming  SMS  URCs  are Presented		52

    - AT+QCFG="urc/ri/other"	RI Behavior When Other URCs are Presented	53

    - AT+QCFG="risignaltype"	RI Signal Output Carrier	53

    - AT+QCFG="urc/delay"	Delay URC Indication	54

    - AT+QCFG="urc/cache"	URC Cache Function	55

    - AT+QCFG="tone/incoming"	Ring tone Function	56

  - AT+QINDCFG	URC Indication Configuration	57

- (U)SIM Related Commands	60

  - AT+CIMI	Request International Mobile Subscriber Identity (IMSI)	60

  - AT+CLCK	Facility Lock	61

  - AT+CPIN	Enter PIN	63

  - AT+CPWD	Change Password	65

  - AT+CSIM	Generic (U)SIM Access	66

  - AT+CRSM	Restricted (U)SIM Access	67

  - AT+QCCID	Show ICCID	68

  - AT+QPINC	Display PIN Remainder Counter	69

  - AT+QINISTAT	Query Initialization Status of (U)SIM Card	70

  - AT+QSIMDET	(U)SIM Card Detection	70

  - AT+QSIMSTAT	(U)SIM Card Insertion Status Report	71

- Network Service Commands	74

  - AT+COPS	Operator Selection	74

  - AT+CREG	Network Registration	76

  - AT+CSQ	Signal Quality Report	78

  - AT+CPOL   Preferred Operator List	79

  - AT+COPN	Read Operator Names	81

  - AT+CTZU	Automatic Time Zone Update	81

  - AT+CTZR	Time Zone Reporting	82

  - AT+QLTS	Obtain the Latest Time Synchronized Through Network	84

  - AT+QNWINFO	Query Network Information	85

  - AT+QSPN	Display the Name of Registered Network	87

- Call Related Commands	89

  - ATA   Answer an Incoming Call	89

  - ATD	Mobile Originated Call to Dial a Number	90

  - ATH	Disconnect Existing Connection	92

  - AT+CVHU	Voice Hang up Control	92

  - AT+CHUP	Hang up Call	93

7.6.	+++	Switch from Data Mode to Command Mode	93

  - ATO	Switch from Command Mode to Data Mode	94

  - ATS0	Set Number of Rings before Automatically Answering Call	95

  - ATS6	Set Pause before Blind Dialing	96

  - ATS7	Set Time to Wait for Connection Completion	96

  - ATS8	Set the Time to Wait for Comma Dial Modifier	97

  - ATS10	Set Disconnection Delay after Indicating the Absence of Data Carrier	98

  - AT+CBST	Select Bearer Service Type	98

  - AT+CSTA   Select Type of Address	101

  - AT+CLCC	List Current Calls of ME	101

  - AT+CR	Service Reporting Control	103

  - AT+CRC	Set Cellular Result Codes for Incoming Call Indication	104

  - AT+CRLP	Select Radio Link Protocol Parameter	105

  - AT+QECCNUM	Configure Emergency Call Numbers	106

  - AT+QHUP	Hang up Call with a Specific Release Cause	108

- Phonebook Commands	110

  - AT+CNUM	Subscriber Number	110

  - AT+CPBF	Find Phonebook Entries	111

  - AT+CPBR	Read Phonebook Entries	112

  - AT+CPBS	Select Phonebook Memory Storage	113

  - AT+CPBW	Write Phonebook Entry	114

- Short Message Service Commands	116

  - AT+CSMS	Select Message Service	116

  - AT+CMGF	Message Format	117

  - AT+CSCA   Service Center Address	118

  - AT+CPMS	Preferred Message Storage	119

  - AT+CMGD	Delete Message	121

  - AT+CMGL   List Messages	122

  - AT+CMGR	Read Message	125

  - AT+CMGS	Send Message	129

  - AT+CMMS	More Messages to Send	131

  - AT+CMGW	Write Message to Memory	132

  - AT+CMSS	Send Message from Storage	134

  - AT+CNMA   New Message Acknowledgement to UE/TE	135

  - AT+CNMI	SMS Event Reporting Configuration	137

  - AT+CSCB	Select Cell Broadcast Message Types	139

  - AT+CSDH	Show SMS Text Mode Parameters	140

  - AT+CSMP	Set SMS Text Mode Parameters	141

  - AT+QCMGS	Send Concatenated Messages	142

  - AT+QCMGR	Read Concatenated Messages	144

- Packet Domain Commands	146

  - AT+CGATT	Attachment or Detachment of PS	146

  - AT+CGDCONT	Define PDP Context	147

  - AT+CGQREQ	Quality of Service Profile (Requested)	148

  - AT+CGQMIN	Quality of Service Profile (Minimum Acceptable)	151

  - AT+CGEQREQ	3G Quality of Service Profile (Requested)	154

  - AT+CGEQMIN	3G Quality of Service Profile (Minimum Acceptable)	158

  - AT+CGACT	Activate or Deactivate PDP Context	162

  - AT+CGDATA   Enter Data State	163

  - AT+CGPADDR	Show PDP Address	164

  - AT+CGCLASS	GPRS Mobile Station Class	165

  - AT+CGREG	Network Registration Status	166

  - AT+CGEREP	Packet Domain Event Reporting	168

  - AT+CGSMS	Select Service for MO SMS Messages	170

  - AT+CEREG	EPS Network Registration Status	171

  - AT+QGDCNT	Packet Data Counter	172

  - AT+QAUGDCNT	Auto Save Packet Data Counter	173

- Supplementary Service Commands	175

  - AT+CCFC	Call Forwarding Number and Conditions Control	175

  - AT+CCWA   Call Waiting Control	177

  - AT+CHLD	Call Related Supplementary Services	179

  - AT+CLIP	Calling Line Identification Presentation	181

  - AT+CLIR	Calling Line Identification Restriction	182

  - AT+COLP	Connected Line Identification Presentation	183

  - AT+CSSN	Supplementary Service Notifications	185

  - AT+CUSD	Unstructured Supplementary Service Data	186

- Audio Commands	188

  - AT+CLVL   Loudspeaker Volume Level Selection	188

  - AT+CMUT	Mute Control	189

  - AT+QAUDLOOP	Enable/Disable Audio Loop Test	189

  - AT+VTS	DTMF and Tone Generation	190

  - AT+VTD	Set Tone Duration	191

  - AT+QAUDMOD	Set Audio Mode	192

  - AT+QDAI	Digital Audio Interface Configuration	193

  - AT+QEEC	Set Echo Cancellation Parameters	195

  - AT+QSIDET	Set the Side Tone Gain in Current Mode	196

  - AT+QMIC	Set Uplink Gains of MIC	197

  - AT+QRXGAIN	Set Downlink Gains of RX	198

  - AT+QIIC	IIC Read & Write	199

  - AT+QTONEDET	Enable/Disable DTMF Detection	200

  - AT+QLDTMF	Play Local DTMF	201

  - AT+QLTONE	Play a Local Customized Tone	202

- Hardware Related Commands	204

  - AT+QPOWD	Power off	204

  - AT+CCLK	Clock	204

  - AT+CBC	Battery Charge	205

  - AT+QADC	Read ADC Value	206

  - AT+QSCLK	Enable/Disable Entering into Sleep Mode	207

- DFOTA Related AT Command	208

- FTP(S) Related AT Commands	209

- HTTP(S) Related AT Commands	210

- MMS Related AT Commands	211

- SMTP Related AT Commands	212

- TCP(IP) Related AT Commands	213

- GNSS Related AT Commands	214

- Appendix	215

  - References	215

  - Factory Default Settings Restorable with AT&F	217

  - AT Command Settings Storable with AT&W	219

  - AT Command Settings Storable with ATZ	219

  - Summary of CME ERROR Codes	220

  - Summary of CMS ERROR Codes	222

  - Summary of URC	224

  - SMS Character Sets Conversions	226

  - Release Cause Text List of AT+CEER	232

Table Index

TABLE 1: TYPES OF AT COMMANDS AND RESPONSES	10

TABLE 2: AT&V RESPONSE	18

TABLE 3: ATV0&ATV1 RESULT CODES NUMERIC EQUIVALENTS AND BRIEF DESCRIPTION	21

TABLE 4: PARAMETER CONFIGURATIONS SUPPORTED BY AT+CBST	100

TABLE 5: DELAY CLASS	151

TABLE 6: DFOTA RELATED AT COMMAND	208

TABLE 7: FTP(S) RELATED AT COMMANDS	209

TABLE 8: HTTP(S) RELATED AT COMMANDS	210

TABLE 9: MMS RELATED COMMANDS	211

TABLE 10: SMTP RELATED AT COMMANDS	212

TABLE 11: TCP(IP) RELATED AT COMMANDS	213

TABLE 12: GNSS RELATED AT COMMANDS	214

TABLE 13: RELATED DOCUMENTS	215

TABLE 14: TERMS AND ABBREVIATIONS	216

TABLE 15: FACTORY DEFAULT SETTINGS RESTORABLE WITH AT&F	217

TABLE 16: AT COMMAND SETTINGS STORABLE WITH AT&W	219

TABLE 17: AT COMMAND SETTINGS STORABLE WITH ATZ	219

TABLE 18: DIFFERENT CODING SCHEMES OF +CME ERROR: <ERR>	220

TABLE 19: DIFFERENT CODING SCHEMES OF +CMS ERROR: <ERR>	222

TABLE 20: SUMMARY OF URC	224

TABLE 21: THE WAY OF SMS TEXT INPUT OR OUTPUT	226

TABLE 22: THE INPUT CONVERSIONS TABLE (DCS=GSM 7 BIT AND AT+CSCS=“GSM”)	227

TABLE 23: THE OUTPUT CONVERSIONS TABLE (DCS=GSM 7 BIT AND AT+CSCS=“GSM”)	228

TABLE 24: GSM EXTENDED CHARACTERS	228

TABLE 25: THE INPUT CONVERSIONS TABLE (DCS=GSM 7 BIT AND AT+CSCS=“IRA”)	229

TABLE 26: IRA EXTENDED CHARACTERS	230

TABLE 27: THE OUTPUT CONVERSIONS TABLE (DCS=GSM 7 BIT AND AT+CSCS=“IRA”)	230

TABLE 28: GSM EXTENDED CHARACTERS	231

TABLE 29: RELEASE CAUSE TEXT LIST OF AT+CEER	232

# Introduction

## Scope of the Document

This document presents the AT Commands Set for Quectel cellular engines EC25 & EC21.

## AT Command Syntax

The “AT” or “at” prefix must be set at the beginning of each command line. To terminate a command line enter <CR>. Commands are usually followed by a response that includes “<CR><LF><response><CR><LF>”. Throughout this document, only the responses  are  presented, “<CR><LF>” are omitted intentionally.

The AT Commands Set implemented by EC25 & EC21 is a combination of 3GPP TS 27.007, 3GPP TS

27.005 and ITU-T recommendation V.25ter as well as the AT Commands developed by Quectel.

All these AT commands can be split into three categories syntactically: “basic”, “S parameter”, and “extended”. They are listed as follows:

#### Basic syntax

These AT commands have the format of “AT<x><n>”, or “AT&<x><n>”, where “<x>” is the command, and “<n>” is/are the argument(s) for that command. An example of this is “ATE<n>”, which tells the DCE whether received characters should be echoed back to the DTE according to the value of “<n>”. “<n>” is optional and a default will be used if it is missing.

#### S parameter syntax

These AT commands have the format of “ATS<n>=<m>”, where “<n>” is the index of the S register to set, and “<m>” is the value to assign to it.

#### Extended syntax

These commands can be operated in several modes, as following table:

#### Table 1: Types of AT Commands and Responses

| Test Command | This command returns the list of parameters and value ranges AT+<x>=? set by the corresponding Write Command or internal processes. |
| --- | --- |
| Read Command | This command returns the currently set value of the parameter AT+<x>? or parameters. |
| Write Command | AT+<x>=<…> This command sets the user-definable parameter values. |
| Execution Command | This command reads non-variable parameters affected by AT+<x> internal processes in the UE. |

## Supported Character Sets

EC25 & EC21 AT command interface defaults to the GSM character set. EC25 & EC21 modules support the following character sets:

- GSM format

- UCS2

- IRA

The character set can be configured and interrogated by using the AT+CSCS command (3GPP TS 27.007) and it is defined in 3GPP TS 27.005. The character set affects transmission and reception of SMS and SMS Cell Broadcast Messages, as well as the entry and display of phone book entries text field.

## AT Command Interface

EC25 & EC21 AT command interface includes two USB ports (USB MODEM port and USB AT port) and one main UART port. The main UART port and two USB ports support AT command communication and data transfer.

## Unsolicited Result Code

As an Unsolicited Result Code and a report message, URC is not issued as part of the response related to an executed AT command. URC is issued by the EC25 & EC21 without being requested by the TE and it is issued automatically when a certain event occurs. Typical events leading to URCs are incoming calls (RING), received short messages, high/low voltage alarm, high/low temperature alarm, etc.

## Turn off Procedure

It is recommended to execute AT+QPOWD command to turn off the module, as it is the safest and best way. This procedure is realized by letting the module log off from the network and allowing the software to enter into a secure and safe data state before disconnecting the power supply.

After sending AT+QPOWD, do not enter any other AT commands. The module outputs message, POWERED DOWN and sets the STATUS pin as low to enter into the shutdown state. In order to avoid data loss, it is suggested to wait for 1s to switch off the VBAT after the STATUS pin is set as low and the URC POWERED DOWN is outputted. If POWERED DOWN has not been received after 65s, the VBAT shall be switched off compulsorily.

# General	Commands

## ATI	Display Product Identification Information

The command delivers a product information text.

| ATI Display Product Identification Information |  |
| --- | --- |
| Execution Command ATI | Response TA issues product information text. Quectel EC2x Revision: <revision> OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<revision>	Identification text of product software version

Example

ATI

Quectel EC25

Revision: EC25EFAR02A09M4G

OK

## AT+GMI	Request Manufacturer Identification

The command returns a manufacturer identification text. See also AT+CGMI.

| AT+GMI Request Manufacturer Identification |  |
| --- | --- |
| Test Command AT+GMI=? | Response OK |
| Execution Command AT+GMI | Response TA reports one or more lines of information text which permit the user to identify the manufacturer. Quectel OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

## AT+GMM	Request TA Model Identification

The command returns a product model identification text. It is identical with AT+CGMM.

| AT+GMM Request TA Model Identification |  |
| --- | --- |
| Test Command AT+GMM=? | Response OK |
| Execution Command AT+GMM | Response TA returns a product model identification text. EC2x OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

## AT+GMR	Request TA Revision Identification of Software Release

The command delivers a product firmware version identification text. It is identical with AT+CGMR.

| AT+GMR Request TA Revision Identification of Software Release |  |
| --- | --- |
| Test Command AT+GMR=? | Response OK |
| Execution Command AT+GMR | Response TA reports one or more lines of information text which permit the user to identify the revision of software release. <revision> OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<revision>	Identification text of product software version

Example

AT+GMR EC25EFAR02A09M4G

OK

## AT+CGMI	Request Manufacturer Identification

The command returns a manufacturer identification text. See also AT+GMI.

| AT+CGMI Request Manufacturer Identification |  |
| --- | --- |
| Test Command AT+CGMI=? | Response OK |
| Execution Command AT+CGMI | Response TA returns manufacturer identification text. Quectel |

|  | OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

## AT+CGMM	Request Model Identification

The command returns a product model identification text. It is identical with AT+GMM.

| AT+CGMM Request Model Identification |  |
| --- | --- |
| Test Command AT+CGMM=? | Response OK |
| Execution Command AT+CGMM | Response TA returns product model identification text. EC2x OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

## AT+CGMR	Request TA Revision Identification of Software Release

The command delivers a product firmware version identification text. It is identical with AT+GMR.

| AT+CGMR Request TA Revision Identification of Software Release |  |
| --- | --- |
| Test Command AT+CGMR=? | Response OK |
| Execution Command AT+CGMR | Response TA returns identification text of product software version. <revision> OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<revision>	Identification text of product software version

## AT+GSN	Request International Mobile Equipment Identity (IMEI)

The command returns the International Mobile Equipment Identity (IMEI) number of ME. It is identical with

#### AT+CGSN.

| AT+GSN Request International Mobile Equipment Identity (IMEI) |  |
| --- | --- |
| Test Command AT+GSN=? | Response OK |
| Execution Command AT+GSN | Response TA reports the IMEI (International Mobile Equipment Identity) number in information text which permits the user to identify the individual ME device. <IMEI> OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<IMEI>	IMEI of the ME

The serial number (IMEI) varies with the individual ME device.

NOTE

## AT+CGSN	Request Product Serial Number Identification

The command returns International Mobile Equipment Identity (IMEI) number of ME. It is identical with

#### AT+GSN.

| AT+CGSN Request Product Serial Number Identification |  |
| --- | --- |
| Test Command AT+CGSN=? | Response OK |
| Execution Command AT+CGSN | Response <IMEI> OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<IMEI>	IMEI of the ME

The serial number (IMEI) varies with the individual ME device.

NOTE

## AT&F	Set all Current Parameters to Manufacturer Defaults

The command resets AT command settings to their factory default values.

| AT&F Set all Current Parameters to Manufacturer Defaults |  |
| --- | --- |
| Execution Command AT&F[<value>] | Response TA sets all current parameters to the manufacturer defined profile. See Table 8. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<value>	0	Set all TA parameters to manufacturer defaults

## AT&V	Display Current Configuration

The command displays the current settings of several AT command parameters, including the single-letter AT command parameters which are not readable otherwise.

| AT&V Display Current Configuration |  |
| --- | --- |
| Execution Command AT&V | Response TA returns the current parameter settings. See Table 2. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

#### Table 2: AT&V Response

AT&V

&C: 1

&D: 2

&F: 0

&W: 0

E: 1

Q: 0

V: 1

X: 4

Z: 0

S0: 0

S3: 13

S4: 10

S5: 8

S6: 2

S7: 0

S8: 2

S10: 15 OK

## AT&W	Store Current Parameters to User Defined Profile

The command stores the current AT command settings to a user defined profile in non-volatile memory.

| AT&W Store Current Parameters to User Defined Profile |  |
| --- | --- |
| Execution Command AT&W[<n>] | Response TA stores the current parameter settings in the user defined profile. See Table 9. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0	Profile number to store current parameters

## ATZ   Set all Current Parameters to User Defined Profile

The command restores the current AT command settings to the user defined profile in non-volatile memory, if one was stored with AT&W before. Any additional AT command on the same command line may be ignored.

| ATZ Set all Current Parameters to User Defined Profile |  |
| --- | --- |
| Execution Command ATZ[<value>] | Response TA sets all current parameters to the user defined profile. See Table 10. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<value>	0	Reset to profile number 0

## ATQ	Set Result Code Presentation Mode

The command controls whether the result code is transmitted to the TE. Other information text transmitted as response is not affected.

| ATQ Set Result Code Presentation Mode |  |
| --- | --- |
| Execution Command ATQ<n> | Response This parameter setting determines whether or not the TA transmits any result code to the TE. Information text transmitted in response is not affected by this setting. If <n>=0: OK If <n>=1: (none) |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0	TA transmits result code

1	Result codes are suppressed and not transmitted

## ATV	TA Response Format

The command determines the contents of header and trailer transmitted with AT command result codes and information responses.

The result codes, their numeric equivalents and brief descriptions of the use of each are listed in the following table.

| ATV TA Response Format |  |
| --- | --- |
| Execution Command ATV<value> | Response This parameter setting determines the contents of the header and trailer transmitted with result codes and information responses. When <value>=0 0 When <value>=1 OK |

| Maximum Response Time | 300ms |
| --- | --- |
| Reference V.25ter |  |

Parameter

<value>	0	Information response: <text><CR><LF>

Short result code format: <numeric code><CR>

1	Information response: <CR><LF><text><CR><LF>

Long result code format: <CR><LF><verbose code><CR><LF>

Example

ATV1

OK AT+CSQ

+CSQ: 30,99

//Set <value>=1

OK

ATV0 0

AT+CSQ

+CSQ: 30,99

0

//When <value>=1, the result code is OK.

//Set <value>=0

//When <value>=0, the result code is 0.

#### Table 3: ATV0&ATV1 Result Codes Numeric Equivalents and Brief Description

ATV1

ATV0

Description

OK	0	Acknowledges execution of a command

CONNECT	1

RING	2

NO CARRIER	3

ERROR	4

A connection has been established; the DCE is moving from command mode to data mode

The DCE has detected an incoming call signal from network

The connection has been terminated or the attempt to establish a connection failed

Command not recognized, command line maximum length exceeded, parameter value invalid, or other problem with processing the command line

NO DIALTONE	6	No dial tone detected

BUSY	7	Engaged (busy) signal detected

NO ANSWER	8

“@” (Wait for Quiet Answer) dial modifier was used, but remote ringing followed by five seconds of silence was not detected before expiration of the connection timer (S7)

## ATE	Set Command Echo Mode

The command controls whether or not the module echoes characters received from TE during AT command mode.

| ATE Set Command Echo Mode |  |
| --- | --- |
| Execution Command ATE<value> | Response This setting determines whether or not the TA echoes characters received from TE during command state. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<value>	0	Echo mode OFF

1	Echo mode ON

## A/	Repeat Previous Command Line

The command repeats previous AT command line, and “/” acts as the line terminating character.

| A/ Repeat Previous Command Line |  |
| --- | --- |
| Execution Command A/ | Response Repeat the previous command |
| Reference V.25ter |  |

Example

ATI

Quectel EC25

Revision: EC25EFAR02A09M4G

OK

A/ Quectel EC25

Revision: EC25EFAR02A09M4G

//Repeat the previous command

OK

## ATS3	Set Command Line Termination Character

The command determines the character recognized by the module to terminate an incoming command line. It is also generated for result codes and information text, along with character value set via ATS4.

| ATS3 Set Command Line Termination Character |  |
| --- | --- |
| Read Command ATS3? | Response <n> OK |
| Write Command ATS3=<n> | Response This parameter setting determines the character recognized by TA to terminate an incoming command line. The TA also returns this character in output. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0-13-127	Command line termination character (Default 13=<CR>)

## ATS4	Set Response Formatting Character

The command determines the character generated by the module for result code and information text, along with the command line termination character set via ATS3.

| ATS4 Set Response Formatting Character |  |
| --- | --- |
| Read Command ATS4? | Response <n> OK |
| Write Command ATS4=<n> | Response This parameter setting determines the character generated by the TA for result code and information text. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0-10-127	Response formatting character (Default 10=<LF>)

## ATS5	Set Command Line Editing Character

The command determines the character value used by the module to delete the immediately preceding character from the AT command line (i.e. equates to backspace key).

| ATS5 Set Command Line Editing Character |  |
| --- | --- |
| Read Command ATS5? | Response <n> OK |
| Write Command ATS5=<n> | Response This parameter setting determines the character recognized by TA as a request to delete the immediately preceding character from the command line. OK |
| Maximum Response Time | 300ms |

Reference

V.25ter

Parameter

<n>	0-8-127	Response editing character (Default 8=<Backspace>)

## ATX   Set CONNECT Result Code Format and Monitor Call Progress

The command determines whether or not the module transmits particular result codes to the TE. It also controls whether or not the module verifies the presence of a dial tone when it begins dialing, and whether or not engaged tone (busy signal) detection is enabled.

| ATX Set CONNECT Result Code Format and Monitor Call Progress |  |
| --- | --- |
| Execution Command ATX<value> | Response This parameter setting determines whether or not the TA detected the presence of dial tone and busy signal and whether or not TA transmits particular result codes. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<value>	0	CONNECT result code only returned, dial tone and busy detection are both disabled

- CONNECT <text> result code only returned, dial tone and busy detection are both disabled

- CONNECT <text> result code returned, dial tone detection is enabled, busy detection is disabled

- CONNECT <text> result code returned, dial tone detection is disabled, busy detection is enabled

- CONNECT <text> result code returned, dial tone and busy detection are both enabled

## AT+CFUN	Set Phone Functionality

The command controls the functionality level. It can also be used to reset the UE.

| AT+CFUN Set Phone Functionality |  |
| --- | --- |
| Test Command AT+CFUN=? | Response +CFUN: (list of supported <fun>s),(list of supported <rst>s) OK |
| Read Command AT+CFUN? | Response +CFUN: <fun> OK |
| Write Command AT+CFUN=<fun>[,<rst>] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 15s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<fun>	0	Minimum functionality

1	Full functionality (Default)

4	Disable the ME from both transmitting and receiving RF signals

<rst>	0	Do not reset the ME before setting it to <fun> functionality level.

This is the default setting when <rst> is not given.

1		Reset the ME. The device is fully functional after the reset. This value is available only for <fun>=1

Example

AT+CFUN=0

OK AT+COPS?

+COPS: 0

//Switch phone to minimum functionality

//No operator is registered

OK

AT+CPIN?

+CME ERROR: 13

AT+CFUN=1 OK

//(U)SIM failure

//Switch phone to full functionality

+CPIN: SIM PIN

AT+CPIN=1234 OK

+CPIN: READY

+QUSIM: 1

+QIND: PB DONE

+QIND: SMS DONE

AT+CPIN?

+CPIN: READY

OK

AT+COPS?

+COPS: 0,0,"CHINA MOBILE",7

//Operator is registered

OK

## AT+CMEE	Error Message Format

The command controls the format of error result codes: ERROR, error numbers or verbose messages as

#### +CME ERROR: <err> and +CMS ERROR: <err>.

| AT+CMEE Error Message Format |  |
| --- | --- |
| Test Command AT+CMEE=? | Response +CMEE: (list of supported <n>s) OK |
| Read Command AT+CMEE? | Response +CMEE: <n> OK |
| Write Command AT+CMEE=<n> | Response TA disables or enables the use of result code +CME ERROR: <err> as an indication of an error related to the functionality of the ME. |

|  | OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Disable result code

- Enable result code and use numeric values

- Enable result code and use verbose values

Example

AT+CMEE=0

OK AT+CPIN? ERROR AT+CMEE=1 OK AT+CPIN?

+CME ERROR: 10

AT+CMEE=2

//Disable result code

//Only ERROR will be displayed

//Enable error result code with numeric values

//Enable error result code with verbose (string) values

OK

AT+CPIN?

+CME ERROR: SIM not inserted

## AT+CSCS	Select TE Character Set

The Write Command informs the module which character set is used by the TE. This enables the UE to convert character strings correctly between TE and UE character sets.

| AT+CSCS Select TE Character Set |  |
| --- | --- |
| Test Command AT+CSCS=? | Response +CSCS: (list of supported <chset>s) OK |
| Read Command AT+CSCS? | Response +CSCS: <chset> |

|  | OK |
| --- | --- |
| Write Command AT+CSCS=<chset> | Response Set character set <chset> which is used by the TE. The TA can then convert character strings correctly between the TE and ME character sets. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<chset>	“GSM”	GSM default alphabet

“IRA”	International reference alphabet “UCS2”	UCS2 alphabet

Example

AT+CSCS?

+CSCS: “GSM”

//Query the current character set

OK

AT+CSCS=“UCS2” OK

AT+CSCS?

+CSCS: “UCS2”

//Set the character set to “UCS2”

OK

## AT+QURCCFG	Configure URC Indication Option

The command is used to configure the output port of URC.

| AT+QURCCFG Configure URC Indication Option |  |
| --- | --- |
| Test Command AT+QURCCFG=? | Response +QURCCFG: "urcport",("usbat","usbmodem","uart1") OK |
| Write Command AT+QURCCFG="urcport"[,<urcportv | If the configuration parameter <urcportvalue> is omitted, return current configuration: |

| alue>] | +QURCCFG: "urcport",<urcportvalue> OK If the configuration parameter <urcportvalue> is not omitted, response: OK ERROR |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<urcportvalue>	Set URC output port

"usbat"	USB AT port

"usbmodem"	USB modem port

"uart1"	Main UART

- Configuration of URC output port will be saved to NV immediately by default.

- After configuration of URC output port is set successfully, it will take effect immediately.

NOTES

Example

AT+QURCCFG=?

+QURCCFG: "urcport",("usbat","usbmodem","uart1")

OK

AT+QURCCFG="urcport"

+QURCCFG: "urcport","usbat"

OK

AT+QURCCFG="urcport","usbmodem" OK

AT+QURCCFG="urcport"

+QURCCFG: "urcport","usbmodem"

OK

# Serial	Interface	Control	Commands

## AT&C	Set DCD Function Mode

The command controls the behavior of the UE‟s DCD (data carrier detection) line.

| AT&C Set DCD Function Mode |  |
| --- | --- |
| Execution Command AT&C[<value>] | Response This parameter determines how the state of circuit 109(DCD) relates to the detection of received line signal from the distant end. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<value>	0	DCD line is always ON

1	DCD line is ON only in the presence of data carrier

## AT&D	Set DTR Function Mode

The command determines how the UE responds if DTR line is changed from low to high level during data mode.

| AT&D Set DTR Function Mode |  |
| --- | --- |
| Execution Command AT&D[<value>] | Response This parameter determines how the TA responds when circuit 108/2 (DTR) is changed from low to high level during data mode. OK |
| Maximum Response Time | 300ms |

Reference V.25ter

Parameter

<value>	0	TA ignores status on DTR

- LowHigh on DTR: Change to command mode while remaining the connected call.

- LowHigh on DTR: Disconnect data call, and change to command mode. When DTR is at high level, auto-answer function is disabled.

## AT+IFC	Set TE-TA Local Data Flow Control

The command determines the flow control behavior of the serial port.

| AT+IFC Set TE-TA Local Data Flow Control |  |
| --- | --- |
| Test Command AT+IFC=? | Response +IFC: (list of supported <dce_by_dte>s),(list of supported <dte_by_dce>s) OK |
| Read Command AT+IFC? | Response +IFC: <dce_by_dte>,<dte_by_dce> OK |
| Write Command AT+IFC=<dce_by_dte>,<dte_by_dce> | Response This parameter setting determines the data flow control on the serial interface for data mode. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<dce_by_dte>	Specifies the method that will be used by TE when receiving data from TA

0	None

2	RTS flow control

<dte_by_dce>	Specifies the method that will be used by TA when receiving data from TE

0	None

2	CTS flow control

Flow control is only applicable for data mode.

NOTE

Example

AT+IFC=2,2

OK AT+IFC?

+IFC: 2,2

//Open the hardware flow control

OK

## AT+ICF	Set TE-TA Control Character Framing

The command determines the serial interface character framing format and parity received by TA from TE.

| AT+ICF Set TE-TA Control Character Framing |  |  |  |
| --- | --- | --- | --- |
| Test Command AT+ICF=? | Response +ICF: (list of supported <format>s),(list <parity>s) OK | of | supported |
| Read Command AT+ICF? | Response +ICF: <format>,<parity> OK |  |  |
| Write Command AT+ICF=[<format>,[<parity>]] | Response This parameter setting determines the serial interface character framing format and parity received by TA from TE. OK |  |  |
| Maximum Response Time | 300ms |  |  |
| Reference V.25ter |  |  |  |

Parameter

<format>	3	8 data 0 parity 1 stop

<parity>	0	Odd

- Even

- Mark (1)

- Space (0)

- The command is applied for command state.

- The <parity> field is ignored if the <format> field specifies no parity.

NOTES

## AT+IPR	Set TE-TA Fixed Local Rate

The command is used to query and set the baud rate of the UART. The default baud rate value (<rate>) is 115200bps. The setting of <rate> will not be restored with AT&F.

| AT+IPR Set TE-TA Fixed Local Rate |  |
| --- | --- |
| Test Command AT+IPR=? | Response +IPR: (list of supported auto detectable <rate>s),(list of supported fixed-only <rate>s) OK |
| Read Command AT+IPR? | Response +IPR: <rate> OK |
| Write Command AT+IPR=<rate> | Response This parameter setting determines the data rate of the TA on the serial interface. After the delivery of any result code associated with the current command line, the rate of command takes effect. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<rate>	Baud rate per second

9600

19200

38400

57600

115200

230400

460800

921600

2900000

3000000

- If a fixed baud rate is set, make sure that both TE (DTE, usually external processor) and TA (DCE, Quectel module) are configured to the same rate.

- The value of AT+IPR cannot be restored with AT&F and ATZ; but it is still storable with AT&W.

- In multiplex mode, the baud rate cannot be changed by the Write Command AT+IPR=<rate>; and the setting is invalid and cannot be stored even if AT&W is executed after the Write Command.

- A selected baud rate takes effect after the Write Commands are executed and acknowledged by OK.

NOTES

Example

AT+IPR=115200

OK AT&W

//Set fixed baud rate to 115200bps

//Store current setting, that is, the serial communication

speed is 115200bps after restarting module

OK

AT+IPR?

+IPR: 115200

OK

AT+IPR=115200;&W OK

//Set fixed baud rate to 115200bps and store current setting

## AT+QRIR   Restore RI Behavior to Inactive

If the RI (ring indicator) behavior is "always", it can be restored to inactive by the Execution Command. The RI behavior is controlled by AT+QCFG. Please refer to AT+QCFG="urc/ri/ring", AT+QCFG="urc/ri/smsincoming" and AT+QCFG="urc/ri/other" for more details.

| AT+QRIR Restore RI Behavior to Inactive |  |
| --- | --- |
| Test Command AT+QRIR=? | Response OK |
| Execution Command AT+QRIR | Response OK |

|  | ERROR |
| --- | --- |
| Maximum Response Time | 300ms |

# Status	Control	Commands

## AT+CPAS	Mobile Equipment Activity Status

The Execution Command queries the module‟s activity status.

| AT+CPAS Mobile Equipment Activity Status |  |
| --- | --- |
| Test Command AT+CPAS=? | Response +CPAS: (list of supported <pas>s) OK |
| Execution Command AT+CPAS | Response TA returns the activity status of ME: +CPAS: <pas> OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<pas>	0	Ready

- Ringing

- Call in progress or call hold

Example

AT+CPAS

+CPAS: 0

//The module is idle

OK

RING

AT+CLCC

+CLCC: 1,1,4,0,0,“15695519173”,161

OK

AT+CPAS

+CPAS: 3

//The module is ringing

OK

AT+CLCC

+CLCC: 1,0,0,0,0,“10010”,129

OK

AT+CPAS

+CPAS: 4

//Call in progress

OK

## AT+CEER	Extended Error Report

The command is used to query an extended error and report the cause of the last failed operation, such as:

- the failure to release a call

- the failure to set up a call (both mobile originated or terminated)

- the failure to modify a call by using supplementary services

- the failure to activate, register, query, deactivate or deregister a supplementary service

- the failure to attach GPRS or the failure to activate a PDP context

- the failure to detach GPRS or the failure to deactivate a PDP context

The release cause <text> is a text to describe the cause information given by the network.

| AT+CEER Extended Error Report |  |
| --- | --- |
| Test command AT+CEER=? | Response OK |
| Execution command AT+CEER | Response +CEER: <text> OK ERROR If error is related to ME functionality: |

|  | +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<text>	Release cause text. Reason for the last call failure to setup or release (listed in Chapter 14.9). Both CS and PS domain call types are reported. Cause data is captured from Call Manager events and cached locally for later use by this command.

## AT+QCFG	Extended Configuration Settings

The command is used to query and configure various settings of UE.

| AT+QCFG Extended Configuration Settings |  |
| --- | --- |
| Test Command AT+QCFG=? | Response +QCFG: “gprsattach”, (list of supported <attachmode>s) +QCFG: “nwscanmode”, (list of supported <scanmode>s), (list of supported <effect>s) +QCFG: “nwscanseq”, (list of supported <scanseq>s), (list of supported <effect>s) +QCFG: “roamservice”, (list of supported <roammode>s), (list of supported <effect>s) +QCFG: “servicedomain”, (list of supported <service>s), (list of supported <effect>s) +QCFG: “band”, (list of supported <bandval>s), (list of supported <ltebandval>s) (list of supported <effect>s) +QCFG: “hsdpacat”, (list of supported <cat>s) +QCFG: “hsupacat”, (list of supported <cat>s) +QCFG: “rrc”, (list of supported <rrcr>s) |

|  | +QCFG: “sgsn”, (list of supported <sgsnr>s) +QCFG: “msc”,(list of supported <mscr>s) +QCFG: "pdp/duplicatechk",(list of supported <enable>s) +QCFG: "tdscsq",(list of supported <value>s) +QCFG: "urc/ri/ring",(list of supported <typeri>s), (list of supported <pulseduration>s), (list of supported <activeduration>s), (list of supported <inactiveduration>s), (list of supported <ringnodisturbing>s) +QCFG: "urc/ri/smsincoming", (list of supported <typeri>s), (list of supported <pulseduration>s) +QCFG: "urc/ri/other", (list of supported <typeri>s), (list of supported <pulseduration>s) +QCFG: "risignaltype", (list of supported <risignatype>s) +QCFG: "urc/cache", (list of supported <value>s) +QCFG: "tone/incoming" (list of supported <value>s) OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference |  |

### AT+QCFG="gprsattach"	GPRS Attach Mode Configuration

The command specifies the mode to attach GPRS when UE is powered on. This configuration is valid only after the module is restarted.

| AT+QCFG="gprsattach" GPRS Attach Mode Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“gprsattach”[,<attachmode >] | Response If <attachmode> is omitted, return current configuration: +QCFG: “gprsattach”,<attachmode> OK If the configuration parameter <attachmode> is not omitted, configure the GPRS attach mode: OK |

|  | ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<attachmode>	Number format, the mode to attach GRPS when UE is powered on

0	Manual attach

1	Auto attach

### AT+QCFG="nwscanmode"	Network Search Mode Configuration

The command specifies the network mode to be serached. If <effect> is omitted, the configuration will take effect immediately.

| AT+QCFG="nwscanmode" Network Search Mode Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“nwscanmode”[,<scanmod e>[,<effect>]] | Response If <scanmode> and <effect> are both omitted, return the current configuration: +QCFG: “nwscanmode”,<scanmode> OK If <scanmode> and <effect> are not omitted, set the network mode to be searched: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<scanmode>	Number format, network search mode

0	AUTO

- GSM only

- WCDMA only

- LTE only

- TD-SCDMA only

- UMTS only

- CDMA only

- HDR only

- CDMA and HDR only

<effect>	Number format. When to take effect

0	Take effect after UE reboots

1	Take effect immediately

### AT+QCFG="nwscanseq"	Network Searching Sequence Configuration

The command specifies the sequence of searching network. This configuration is valid only after the module is restarted.

| AT+QCFG="nwscanseq" Network Searching Sequence Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“nwscanseq”[,<scanseq>] | Response If <scanseq> is omitted, return current configuration: +QCFG: “nwscanseq”,<scanseq> OK If <scanseq> is omitted, specify the network searching sequence: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<scanseq>	Number format. Network search sequence

(e.g.: 04030201(LTE/WCDMA/TD-SCDMA/GSM))

- Automatic (LTE/ WCDMA/TD-SCDMA/GSM)

- GSM

- TD-SCDMA

- WCDMA

- LTE

- CDMA

### AT+QCFG="roamservice"	Roam Service Configuration

The command is used to enable or disable the roam service. If <effect> is omitted, the configuration will take effect immediately.

| AT+QCFG="roamservice" Roam Service Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“roamservice”[,<roammod e>[,<effect>]] | Response If <roammode> and <effect> are both omitted, return the current configuration: +QCFG: "roamservice",<roammode> OK If <roammode> and <effect> are not omitted, configure the mode of roam service : OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<roammode>	Number format. The mode of roam service

      - Disable roam service

      - Enable roam service

255	AUTO

<effect>	Number format. When to take effect

0	Take effect after UE reboots

1	Take effect immediately

### AT+QCFG="servicedomain"	Service Domain Configuration

The command specifies the registered service domain. If <effect> is omitted, the configuration will take effect immediately.

| AT+QCFG="servicedomain" Service Domain Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“servicedomain”[,<service >[,<effect>]] | Response If <service> and <effect> are both omitted, return the current configuration: |

|  | +QCFG: "servicedomain",<service> OK If <service> and <effect> are not omitted, configure the service domain of UE: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<service>	Service domain of UE

0	CS only

- PS only

- CS & PS

<effect>	Number format. When to take effect

0	Take effect after UE reboots

1	Take effect immediately

### AT+QCFG="band"	Band Configuration

The command specifies the preferred frequency bands to be searched of UE. If <effect> is omitted, the configuration will take effect immediately.

| AT+QCFG="band" Band Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“band”[,<bandval>,<lteban dval>,<tdsbandval>[,<effect>]] | Response If configuration parameters are omitted (that is, only execute AT+QCFG="band"), return current configuration: +QCFG: “band”,<bandval>,<ltebandval>,<tdsbandval> OK If configuration parameters are all entered, configure the preferred frequency bands to be searched: OK ERROR If there is any error related to ME functionality: |

|  | +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<bandval>	A hexadecimal value that specifies the GSM and WCDMA frequency band.  If it is set to 0, it means not to change GSM and WCDMA frequency band. (e.g.: 00000013=00000001(GSM900)+00000002(GSM1800)+00000010(WCDMA

2100))

00000000	No change

00000001	GSM900

00000002	GSM1800

00000004	GSM850

00000008	GSM1900

00000010	WCDMA 2100

00000020	WCDMA 1900

00000040	WCDMA 850

00000080	WCDMA 900

00000100	WCDMA 800

00000200	WCDMA 1700

0000FFFF	Any frequency band

<ltebandval>	A hexadecimal value that specifies the LTE frequency band. If it is set to 0 or 0x40000000, it means not to change LTE frequency band. (e.g.: 0x15=0x1(LTE B1)+0x4(LTE B3)+0x10(LTE B5))

0x1 (CM_BAND_PREF_LTE_EUTRAN_BAND1)	LTE B1

0x4 (CM_BAND_PREF_LTE_EUTRAN_BAND3)	LTE B3

0x10 (CM_BAND_PREF_LTE_EUTRAN_BAND5)	LTE B5

0x40 (CM_BAND_PREF_LTE_EUTRAN_BAND7)	LTE B7

0x80 (CM_BAND_PREF_LTE_EUTRAN_BAND8)	LTE B8

0x80000(CM_BAND_PREF_LTE_EUTRAN_BAND20)	LTE B20

0x7FFFFFFFFFFFFFFF(CM_BAND_PREF_ANY)	Any frequency band

<tdsbandval>		A hexadecimal value that specifies the TD-SCDMA frequency band. If it is set to 0 or 0x40000000, it means not to change TD-SCDMA frequency band. (e.g.: 0x21=0x1(TDS BCA)+0x20(TDS BCF))

0x1 (CM_BAND_PREF_TDS_BANDA)	TDS BCA

0x2 (CM_BAND_PREF_TDS_BANDB)	TDS BCB

0x4 (CM_BAND_PREF_TDS_BANDC)	TDS BCC

0x8 (CM_BAND_PREF_TDS_BANDD)	TDS BCD

0x10 (CM_BAND_PREF_TDS_BANDE)	TDS BCE

0x20 (CM_BAND_PREF_TDS_BANDF)	TDS BCF

<effect>	When to take effect

0	Take effect after UE reboots

1	Take effect immediately

### AT+QCFG="hsdpacat"	HSDPA Category Configuration

The command specifies the HSDPA category. This configuration is valid only after the module is restarted.

| AT+QCFG="hsdpacat" HSDPA Category Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“hsdpacat”[,<cat>] | Response If <cat> is omitted, return the current configuration: +QCFG: "hsdpacat",<cat> OK If <cat> is not omitted, configure the HSDPA category: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<cat>	HSDPA category

- Category 6

8	Category 8

10	Category 10

12	Category 12

14	Category 14

18	Category 18

20	Category 20

24	Category 24

### AT+QCFG="hsupacat"	HSUPA Category Configuration

The command specifies the HSUPA category. This configuration is valid only after the module is restarted.

| AT+QCFG="hsupacat" HSUPA Category Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“hsupacat”[,<cat>] | Response If <cat> is omitted, return the current configuration: +QCFG: "hsupacat",<cat> |

|  | OK If <cat> is not omitted, configure the HSUPA category: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<cat>	HSUPA category

- Category 5

- Category 6

### AT+QCFG="rrc"	RRC Release Version Configuration

The command specifies the RRC release version. This configuration is valid only after the module is restarted.

| AT+QCFG="rrc" RRC Release Version Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“rrc”[,<rrcr>] | Response If <rrcr> is omitted, return the current configuration: +QCFG: “rrc”,<rrcr> OK If <rrcr> is not omitted, configure the RRC release version: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<rrcr>	RRC release version.

0	R99

- R5

- R6

- R7

- R8

### AT+QCFG="sgsn"	UE SGSN Release Version Configuration

The command specifies the UE SGSN release version. This configuration is valid only after the module is restarted.

| AT+QCFG="sgsn" UE SGSN Release Version Configuration |  |
| --- | --- |
| Write Command AT+QCFG=“sgsn”[,<sgsnr>] | Response If <sgsnr> is omitted, return the current configuration: +QCFG: “sgsn”,<sgsnr> OK If <sgsnr> is not omitted, configure the SGSN release version: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<sgsnr>	SGSN release version 0	R97

1	R99

2	Dynamic

### AT+QCFG="msc"	UE MSC Release Version Configuration

The command specifies the UE MSC release version. This configuration is valid only after the module is restarted.

| AT+QCFG="msc" UE MSC Release Version Configuration |  |
| --- | --- |
| Write Command AT+QCFG="msc"[,<mscr>] | Response If <mscr> is omitted, return the current configuration: +QCFG: "msc",<mscr> |

|  | OK If <mscr> is not omitted, configure the MSC release version: OK ERROR If there is an error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<mscr>		MSC release version 0	R97

1	R99

2	Dynamic

### AT+QCFG="PDP/duplicatechk"	Establish Multi PDNs with the Same APN

The command allows/refuses establishing multi PDNs with the same APN profile. The configuration will take effect immediately.

| AT+QCFG="pdp/duplicatechk" Establish Multi PDNs with the Same APN |  |
| --- | --- |
| Write Command AT+QCFG=“pdp/duplicatechk”[,<enabl e>] | Response If <enable> is omitted, return the current configuration: +QCFG: “pdp/duplicatechk”,<enable> OK If <enable> is not omitted, allow/refuse establishing multiple PDNs with the same APN profile: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

| Parameter |  |  |
| --- | --- | --- |
| <enable> | 0 | Refused to establish multi PDNs with the same APN profile |
|  | 1 | Allowed to establish multi PDNs with the same APN profile |

### AT+QCFG="urc/ri/ring"   RI Behavior When RING URC is Presented

AT+QCFG="urc/ri/ring", AT+QCFG="urc/ri/smsincoming" and AT+QCFG="urc/ri/other" control the RI (ring indicator) behavior when a URC is reported. These configurations will be stored into NV automatically. The ring indicator is active low. AT+QCFG="urc/ri/ring" specifies the RI behavior when URC RING is presented to indicate an incoming call.

The sum of parameter <activeduration> and <inactiveduration> determines the interval time of “RING” indications when a call is coming.

| AT+QCFG="urc/ri/ring" RI Behavior When RING URC is Presented |  |
| --- | --- |
| Write Command AT+QCFG="urc/ri/ring"[,<typeri>[,<pul seduration>[,<activeduration>[,<inacti veduration>[,<ringnodisturbing>]]]]] | Response If <typeri>, <pulseduration>, <activeduration>, <inactiveduration> and <ringnodisturbing> are omitted, return the current configuration: +QCFG: "urc/ri/ring",<typeri>,<pulseduration>,<activeduration>, <inactiveduration>,<ringnodisturbing>,<pulsecount> OK If all configuration parameters are entered, set the RI behavior when RING URC is presented: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<typeri>	RI behavior when URCs are presented

"off"	No change. Ring indicator keeps inactive.

"pulse"	Pulse. Pulse width determined by <pulseduration>.

"always"	Change to active. RI behavior can be restored to inactive by

#### AT+QRIR.

"auto"	When “RING” is presented to indicate an incoming call, the ring indicator changes to and keeps active. When ring of the incoming call ends, either answering or hanging up the incoming call, the ring indicator will change to inactive.

"wave"	When RING is presented to indicate an incoming call. The ring indicator outputs a square wave. Both <activeduration> and

<inactiveduration> are used to set parameters of the square wave. When the ring of incoming call ends, either answering or hanging up the incoming call, the ring indicator will change to inactive.

<pulseduration>	Set the width of pulse. The value ranges from 1 to 2000ms and the default is 120ms. This parameter is only meaningful when <typeri> is “pulse”. If this parameter is not needed, it can be set as null.

<activeduration>		The active duration of the square wave. The value ranges from 1 to 10000ms, and the default is 1000ms. This parameter is only meaningful when <typeri> is “wave”.

<inactiveduration>	Set the inactive duration of the square wave. The value ranges from 1 to 10000ms, and the default is 5000ms. This parameter is only meaningful when

<typeri> is “wave”.

<ringnodisturbing>	Set whether the ring indicator behavior could be disturbed. This parameter is only meaningful when <typeri> is configured to “auto” or “wave”. For example, when <typeri> is configured to “wave”, if the square wave does not need to be disturbed by other URCs (including SMS related URCs), then

<ringnodisturbing> should be set to "on".

"off"	RI behavior can be disturbed by other URCs when the behavior is caused by an incoming call ringing.

"on"	RI behavior cannot be disturbed by other URCs when the behavior is caused by an incoming call ringing.

<pulsecount>	The count of pulse. This parameter is only meaningful when  <typeri> is “pulse”. The value ranges from 1 to 5 and the default is 1. The interval time between two pulses is equal to <pulseduration>.

### AT+QCFG="urc/ri/smsincoming"	RI Behavior When Incoming SMS URCs are Presented

The command specifies the RI (ring indicator) behavior when related incoming message URCs are presented. Related incoming message URCs list: +CMTI, +CMT, +CDS and +CBM.

| AT+QCFG="urc/ri/smsincoming" RI Behavior When Incoming SMS URCs are Presented |  |
| --- | --- |
| Write Command AT+QCFG="urc/ri/smsincoming"[,<typ eri>[,<pulseduration>]] | Response If <typeri> and <pulseduration> are omitted, return the current configuration: +QCFG: "urc/ri/smsincoming",<typeri>,<pulseduration>,<pulsec ount> OK If <typeri> and <pulseduration> are not omitted, set the RI behavior when incoming SMS URCs are presented: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<typeri>	RI behavior when URCs are presented

"off"	No change. Ring indicator keeps inactive.

"pulse"	Pulse. Pulse width determined by <pulseduration>.

"always"	Change  to  active.  RI  behavior  can  be  restored  to  inactive  by

#### AT+QRIR.

<pulseduration>	Set the width of pulse. Value ranges from 1 to 2000ms and the default is 120ms.

This parameter is only valid when <typeri> is “pulse”.

<pulsecount>		The count of pulse. This parameter is only meaningful when <typeri> is “pulse”. The value ranges from 1 to 5 and the default is 1. The interval time between two pulses is equal to <pulseduration>.

### AT+QCFG="urc/ri/other"	RI Behavior When Other URCs are Presented

The command specifies the RI (ring indicator) behavior when other URCs are presented.

| AT+QCFG="urc/ri/other" RI Behavior When Other URCs are Presented |  |
| --- | --- |
| Write Command AT+QCFG="urc/ri/other"[,<typeri>[,<p ulseduration>]] | Response If <typeri> and <pulseduration> are omitted, return the current configuration: +QCFG: "urc/ri/other",<typeri>,<pulseduration>,<pulsecount> OK If <typeri> and <pulseduration> are not omitted, set the RI behavior when other URCs are presented: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<typeri>	RI behavior when URCs are presented

"off"	No change. Ring indicator keeps inactive.

"pulse"	Pulse. Pulse width determined by <pulseduration>.

<pulseduration>	Set the width of pulse. Value ranges from 1 to 2000ms and the default is 120ms.

This parameter is effect only when <typeri> is “pulse”.

<pulsecount>		The count of pulse. This parameter is only meaningful when <typeri> is “pulse”. The value ranges from 1 to 5 and the default is 1. The interval time between two pulses is equal to <pulseduration>.

### AT+QCFG="risignaltype"	RI Signal Output Carrier

The command specifies the RI (ring indicator) signal output carrier.

| AT+QCFG="risignaltype" RI Signal Output Carrier |  |
| --- | --- |
| Write Command AT+QCFG="risignaltype",[<risignatyp e>] | Response If <risignatype> is omitted, return the current configuration: +QCFG: "risignaltype",<risignatype> |

|  | OK If <risignatype> is not omitted, configure the RI signal output carrier: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<risignaltype>	RI signal output carrier.

"respective"	The  ring  indicator  behaves  on  the  port  where  URC  is presented.

For example, if a URC is presented on UART port, it is physical ring indicator. If the URC is presented on USB port, it is virtual ring indicator. If the URC is presented on USB AT port, and the port does not support ring indicator, then there will be no ring indicator. AT+QURCCFG=“urcport” can get the port on which URC is presented.

"physical"	No  matter  which  port  URC  is  presented  on,  URC  only causes the behavior of physical ring indicator.

### AT+QCFG="urc/delay"	Delay URC Indication

The command can delay the output of URC indication until ring indicator pulse ends.

| AT+QCFG="urc/delay" Delay URC Indication |  |
| --- | --- |
| Write Command AT+QCFG="urc/delay"[,<enable>] | Response If <enable> is omitted, return the current configuration : +QCFG: "urc/delay",<enable> OK If <enable> is not omitted, set when the URC indication will be outputted: OK ERROR If there is any error related to ME functionality: |

|  | +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<enable>	0	URC indication will be outputted when ring indicator pulse starts.

1	URC indication will be outputted when ring indicator pulse ends (only effective when the type of ring indicator is “pulse”. Please refer to AT+QCFG="urc/ri/ring",AT+QCFG="urc/ri/smsincoming"and AT+QCFG="urc/ri/other" for more details).

### AT+QCFG="urc/cache"	URC Cache Function

| AT+QCFG="urc/cache" URC Cache Function |  |
| --- | --- |
| Write Command AT+QCFG="urc/cache",<enable> | Response If <enable> is omitted, return the current configuration: +QCFG: "urc/cache",<enable> OK If <enable> is not omitted, enable/disable URC cache function: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

| Parameter |  |  |
| --- | --- | --- |
| <enable> | 0 | Disable URC cache |
|  | 1 | Enable URC cache |

The settings of the command will take effect immediately and will not be saved after power off.

NOTE

Example

AT+QCFG="urc/cache"

+QCFG: "urc/cache",0

//URC cache function is disabled

OK

AT+QCFG="urc/cache",1 OK

AT+QCFG="urc/cache"

+QCFG: "urc/cache",1

//Enable URC cache

OK

//Make a call and send two messages to the module

AT+QCFG="urc/cache",0

OK

//Disable URC cache

RING

//Output cached URC

NO CARRIER

//Output cached URC

+CMTI: "ME",0

//Output cached URC

+CMTI: "ME",1

AT+QCFG="urc/cache"

+QCFG: "urc/cache",0

//Output cached URC

//URC cache function is disabled

OK

### AT+QCFG="tone/incoming"	Ring tone Function

| AT+QCFG="tone/incoming" Ring Tone Function |  |
| --- | --- |
| Write Command AT+QCFG="tone/incoming",<enable> | Response If <enable> is omitted, return the current configuration: +QCFG: "tone/incoming",<enable> OK If <enable> is not omitted, enable/disable ring tone function: OK ERROR |

|  | If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Reference |  |

| Parameter |  |  |
| --- | --- | --- |
| <enable> | 0 | Disable ring tone |
|  | 1 | Enable Nokia ring tone |
|  | 2 | Enable ring tone |

The settings of the command will take effect immediately, and will be saved after power off.

NOTE

Example

AT+QCFG="tone/incoming"

+QCFG: "tone/incoming",0

//Ring tone function is disabled

OK

AT+QCFG="tone/incoming",1 OK

AT+QCFG="tone/incoming"

+QCFG: "tone/incoming",1

//Enable ring tone

OK

## AT+QINDCFG	URC Indication Configuration

The command is used to control URC indication.

| AT+QINDCFG URC Indication Configuration |  |
| --- | --- |
| Test command AT+QINDCFG=? | Response +QINDCFG: "all",(0,1),(0,1) +QINDCFG: "csq",(0,1),(0,1) +QINDCFG: "smsfull",(0,1),(0,1) +QINDCFG: "ring",(0,1),(0,1) +QINDCFG: "smsincoming",(0,1),(0,1) +QINDCFG: "act",(0,1),(0,1) |

|  | +QINDCFG: "ccinfo",(0,1),(0,1) OK |
| --- | --- |
| Write command AT+QINDCFG=<urctype>[,<enable>[,< savetonvram>]] | Response If <enable> and <savetonvram> are omitted, the current configuration will be returned: +QINDCFG: <urctype>,<enable> OK If <enable> and <savetonvram> are not omitted, set the URC indication configurations: OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<urctype>	URC type

"all"	Main switch of all URCs. Default is ON.

"csq"	Indication of signal strength and channel bit error rate change (similar to AT+CSQ). Default is OFF. If this configuration is ON, present:

#### +QIND: “csq”,<rssi>,<ber>

"smsfull"	SMS storage full indication. Default is  OFF. If this configuration is ON, present:

#### +QIND: “smsfull”,<storage>

"ring"	“RING” indication. Default is ON. "smsincoming"	Incoming message indication. Default is ON.

Related URCs list:

+CMTI, +CMT, +CDS

"act"	Indication  of  network  access  technology  change.

Default is OFF. If this configuration is ON, present:

#### +QIND: "act",<actvalue>

<actvalue> is string format. The values are as below: “GSM”

“EGPRS” “WCDMA” “HSDPA” “HSUPA”

“HSDPA&HSUPA” “LTE”

“TD-SCDMA” “CDMA”

“HDR”

“EVDO” “UNKNOWN”

The examples of URC are as below:

#### +QIND: "act","HSDPA&HSUPA"

#### +QIND: "act","UNKNOWN"

The description of “act” is as below:

    - If  module  does  not  register  on  network,  the

<actvalue> would be “UNKNOWN”.

    - If this configuration is ON, the URC of "act" will be reported immediately. Only when the network access technology changes, a new URC will be reported.

"ccinfo"	Indication of voice call state change.

<enable>	URC indication is ON or OFF

0	OFF

1	ON

<savetonvram>	Whether to save configuration into NV. Not saved by default.

0	Not save

1	Save

# (U)SIM Related Commands

## AT+CIMI   Request International Mobile Subscriber Identity (IMSI)

The command requests the International Mobile Subscriber Identity (IMSI) which is intended to permit the TE to identify the individual SIM card or active application in the UICC (GSM or USIM) that is attached to MT.

| AT+CIMI Request International Mobile Subscriber Identity (IMSI) |  |
| --- | --- |
| Test Command AT+CIMI=? | Response OK |
| Execution Command AT+CIMI | Response TA returns <IMSI> for identifying the individual (U)SIM which is attached to ME. <IMSI> OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<IMSI>	International Mobile Subscriber Identity (string without double quotes)

Example

AT+CIMI

460023210226023

//Query IMSI number of (U)SIM which is attached to ME

OK

## AT+CLCK   Facility Lock

The command is used to lock, unlock or interrogate a MT or a network facility <fac>. It can be aborted when network facilities are being set or interrogated. The factory default password of PF, PN, PU, PP and PC lock is “12341234”.

| AT+CLCK Facility Lock |  |
| --- | --- |
| Test Command AT+CLCK=? | Response +CLCK: (list of supported <fac>s) OK |
| Write Command AT+CLCK=<fac>,<mode>[,<passwd>[ ,<class>]] | Response This command is used to lock, unlock or interrogate the ME or network facility <fac>. Password is normally needed to do such actions. When querying the status of network service (<mode>=2) the response line for „not active‟ case (<status>=0) should be returned only if service is not active for any <class>. If <mode> is not equal to 2 and the command is set successfully: OK If <mode>=2 and command is set successfully: +CLCK: <status>[,<class>] [+CLCK: <status>[,<class>]] […] OK |
| Maximum Response Time | 5s |
| Reference 3GPP TS 27.007 |  |

Parameter

<fac>	“SC”	(U)SIM  (lock  SIM/UICC  card  installed  in  the  currently  selected  card  slot) (SIM/UICC asks password in MT power-up and when this lock command issued).

“AO”	BAOC (Bar All Outgoing Calls) (refer to 3GPP TS 22.088 clause 1).

“OI”	BOIC (Bar Outgoing International Calls) (refer to 3GPP TS 22.088 clause 1). “OX”	BOIC-exHC (Bar Outgoing International Calls except to Home Country) (refer to

3GPP TS 22.088 clause 1).

“AI”	BAIC (Bar All Incoming Calls) (refer to 3GPP TS 22.088 clause 2).

|  | “IR” | BIC-Roam (Bar Incoming Calls when Roaming outside the home country) (refer |
| --- | --- | --- |
|  |  | to 3GPP TS 22.088 clause 2). |
|  | “AB” | All Barring services (refer to 3GPP TS 22.030) (applicable only for <mode>=0). |
|  | “AG” | All outgoing barring services (refer to 3GPP TS 22.030) (applicable only for |
|  |  | <mode>=0). |
|  | “AC” | All incoming barring services (refer to 3GPP TS 22.030) (applicable only for |
|  |  | <mode>=0). |
|  | “FD” | SIM card or active application in the UICC (GSM or USIM) fixed dialing memory |
|  |  | feature (if PIN2 authentication has not been done during the current session, |
|  |  | PIN2 is required as <passwd>). |
|  | “PF” | Lock Phone to the very first inserted SIM/UICC card (also referred in the present |
|  |  | document as PH-FSIM) (MT asks password when other SIM/UICC cards are |
|  |  | inserted). |
|  | “PN” | Network Personalization (refer to 3GPP TS 22.022) |
|  | “PU” | Network Subset Personalization (refer to 3GPP TS 22.022) |
|  | “PP” | Service Provider Personalization (refer to 3GPP TS 22.022) |
|  | “PC” | Corporate Personalization (refer to 3GPP TS 22.022) |
| <mode> | 0 | Unlock |
|  | 1 | Lock |
|  | 2 | Query status |
| <passwd> Password |  |  |
| <class> | 1 | Voice |
|  | 2 | Data |
|  | 4 | FAX |
|  | 7 | All telephony except SMS (Default) |
|  | 8 | Short message service |
|  | 16 | Data circuit synchronization |
|  | 32 | Data circuit asynchronization |
| <status> | 0 | Off |
|  | 1 | On |

Example

AT+CLCK="SC",2	//Query the status of (U)SIM card

+CLCK: 0	//The (U)SIM card is unlocked (OFF)

#### OK

AT+CLCK="SC",1,"1234"	//Lock (U)SIM card, and the password is 1234

#### OK

AT+CLCK="SC",2	//Query the status of (U)SIM card

+CLCK: 1	//The (U)SIM card is locked (ON)

#### OK

AT+CLCK="SC",0,"1234"	//Unlock (U)SIM card

#### OK

## AT+CPIN   Enter PIN

The command is used to enter a password or query whether or not the module requires a password which is necessary before it can be operated. The password may be (U)SIM PIN, (U)SIM PUK, PH-SIM PIN, etc.

| AT+CPIN Enter PIN |  |
| --- | --- |
| Test Command AT+CPIN=? | Response OK |
| Read Command AT+CPIN? | Response TA returns an alphanumeric string indicating whether or not some password is required. +CPIN: <code> OK |
| Write Command AT+CPIN=<pin>[,<new_pin>] | Response TA stores a password, such as (U)SIM PIN, (U)SIM PUK, etc., which is necessary before it can be operated. If the PIN is to be entered twice, the TA shall automatically repeat the PIN. If no PIN request is pending, no action is taken and an error message +CME ERROR is returned to TE. If the PIN required is (U)SIM PUK or (U)SIM PUK2, the second pin is required. This second pin <new_pin> is used to replace the old pin in the (U)SIM. OK |
| Maximum Response Time | 5s |
| Reference 3GPP TS 27.007 |  |

| Parameter |  |  |
| --- | --- | --- |
| <code> | READY | MT is not pending for any password |
|  | SIM PIN | MT is waiting for SIM PIN to be given |
|  | SIM PUK | MT is waiting for SIM PUK to be given |
|  | SIM PIN2 | MT is waiting for SIM PIN2 to be given |
|  | SIM PUK2 | MT is waiting for SIM PUK2 to be given |
|  | PH-NET PIN | MT is waiting for network personalization password to be given |
|  | PH-NET PUK | MT is waiting for network personalization unblocking password |

to be given

PH-NETSUB PIN	MT is waiting for network subset personalization password to be given

PH-NETSUB PUK	MT is waiting for network subset personalization unblocking password to be given

PH-SP PIN	MT is waiting for service provider personalization password to be given

PH-SP PUK	MT is waiting for service provider personalization unblocking password to be given

PH-CORP PIN		MT is  waiting  for  corporate  personalization password  to  be given

PH-CORP PUK		MT	is	waiting	for	corporate	 personalization	unblocking password to be given

<pin>	String type. Password. If the requested password was a PUK, such as (U)SIM PUK1, PH-FSIM PUK or another passwords, then <pin> must be followed by <new_pin>.

<new_pin>	String type. New password required if the requested code was a PUK.

Example

//Enter PIN

AT+CPIN?

+CPIN: SIM PIN

//Queried PIN code is locked

OK

AT+CPIN=1234 OK

//Enter PIN

+CPIN: READY

AT+CPIN?

+CPIN: READY

//PIN has already been entered

OK

//Enter PUK and PIN

AT+CPIN?

+CPIN: SIM PUK

//Queried PUK code is locked

OK

AT+CPIN="26601934","1234" OK

//Enter PUK and new PIN password

+CPIN: READY

AT+CPIN?

+CPIN: READY

//PUK has already been entered

OK

## AT+CPWD	Change Password

The command sets a new password for the facility lock function defined by AT+CLCK.

| AT+CPWD Change Password |  |
| --- | --- |
| Test Command AT+CPWD=? | Response TA returns a list of pairs which present the available facilities and the maximum length of their password. +CPWD: (list of supported <fac>s),(<pwdlength>s) OK |
| Write Command AT+CPWD=<fac>,<oldpwd>,<newpwd > | Response TA sets a new password for the facility lock function. OK |
| Maximum Response Time | 5s |
| Reference 3GPP TS 27.007 |  |

| Parameter |  |  |
| --- | --- | --- |
| <fac> | “SC” | (U)SIM (lock SIM/UICC card) (SIM/UICC asks password in MT power-up and |
|  |  | when this lock command is issued) |
|  | “AO” | BAOC (Bar All Outgoing Calls, refer to 3GPP TS 22.088 clause 1) |
|  | “OI” | BOIC (Bar Outgoing International Calls, refer to 3GPP TS 22.088 clause 1) |
|  | “OX” | BOIC-exHC (Bar Outgoing International Calls except to Home Country, refer to |
|  |  | 3GPP TS 22.088 clause 1) |
|  | “AI” | BAIC (Bar All Incoming Calls, refer to 3GPP TS 22.088 clause 2) |
|  | “IR” | BIC-Roam (Bar Incoming Calls when Roaming outside the home country, refer |
|  |  | to 3GPP TS 22.088 clause 2) |
|  | “AB” | All barring services (refer to 3GPP TS 22.030, applicable only for <mode>=0) |
|  | “AG” | All outgoing barring services (refer to 3GPP TS 22.030, applicable only for |
|  |  | <mode>=0) |
|  | “AC | All incoming barring services (refer to 3GPP TS 22.030, applicable only for |
|  |  | <mode>=0) |
|  | “P2” | (U)SIM PIN2 |

<pwdlength>	Integer type. Maximum length of password.

<oldpwd>	Password specified for the facility from the user interface or with command.

<newpwd>	New password

Example

AT+CPIN?

+CPIN: READY

OK

AT+CPWD="SC","1234","4321" OK

//Change (U)SIM card password to “4321”

//Restart module or re-activate the SIM card

AT+CPIN?	//Query PIN code is locked

+CPIN: SIM PIN

OK

AT+CPIN="4321" OK

//PIN must be entered to define a new password “4321”

+CPIN: READY

## AT+CSIM   Generic (U)SIM Access

The command allows a direct control of the (U)SIM that is installed in the currently selected card slot by a distant application on the TE. The TE shall then keep the processing of (U)SIM information within the frame specified by GSM/UMTS.

| AT+CSIM Generic (U)SIM Access |  |
| --- | --- |
| Test Command AT+CSIM=? | Response OK |
| Write Command AT+CSIM=<length>,<command> | Response +CSIM: <length>,<response> OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference |  |

3GPP TS 27.007

Parameter

<length>	Integer type. Length of <command> or <response> string.

<command>	Command  transferred  by  the  MT  to  the  (U)SIM  in  the  format  as  described  in

3GPP TS 51.011.

<response>	Response to the command transferred by the (U)SIM to the MT in the format as described in 3GPP TS 51.011.

## AT+CRSM	Restricted (U)SIM Access

The command offers easy and limited access to the (U)SIM database. It transmits the (U)SIM command number <command> and its required parameters to the MT.

| AT+CRSM Restricted (U)SIM Access |  |
| --- | --- |
| Test Command AT+CRSM=? | Response OK |
| Write Command AT+CRSM=<command>[,<fileld>[,<P1 >,<P2>,<P3>[,<data>][,<pathld>]]] | Response +CRSM: <sw1>,<sw2>[,<response>] OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<command>	(U)SIM command number

176	READ BINARY

178	READ RECORD

192	GET RESPONSE

214	UPDATE BINARY

220	UPDATE RECORD

242	STATUS

<fileId>	Integer type. Identifier for an elementary data file on (U)SIM, if used by

#### <command>.

<P1>, <P2>, <P3>	Integer type. Parameters transferred by the MT to the (U)SIM. These parameters are mandatory for every command, except GET RESPONSE and STATUS. The values are described in 3GPP TS 51.011.

<data>	Information which shall be written to the (U)SIM (hexadecimal character format; refer to AT+CSCS).

<pathId>	The directory path of an elementary file on a SIM/UICC in hexadecimal format.

<sw1>, <sw2>	Integer type. Information from the (U)SIM about the execution of the actual command. These parameters are delivered to the TE in both cases, on successful or failed execution of the command.

<response>	Response of a successful completion of the command previously issued (hexadecimal character format; refer to AT+CSCS). STATUS and GET RESPONSE return data, which gives information about the current elementary data field. The information includes the type of file and its size (refer  to  3GPP TS 51.011).  After  READ  BINARY,  READ  RECORD  or

RETRIEVE   DATA   command,   the   requested   data   will   be   returned.

<response> is not returned after a successful UPDATE BINARY, UPDATE RECORD or SET DATA command.

## AT+QCCID	Show ICCID

The command returns the ICCID (Integrated Circuit Card Identifier) number of the (U)SIM card.

| AT+QCCID Show ICCID |  |
| --- | --- |
| Test Command AT+QCCID=? | Response OK |
| Execution Command AT+QCCID | Response +QCCID: <iccid> OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<iccid>	ICCID (Integrated Circuit Card Identifier) number of the (U)SIM card

Example

AT+QCCID

+QCCID: 89860025128306012474

//Query ICCID of the (U)SIM card

OK

## AT+QPINC	Display PIN Remainder Counter

The command can query the number of attempts left to enter the password of (U)SIM PIN/PUK.

| AT+ QPINC Display PIN Remainder Counter |  |
| --- | --- |
| Test Command AT+QPINC=? | Response +QPINC: ("SC","P2") OK |
| Read Command AT+QPINC? | Response +QPINC: "SC", <pincounter>,<pukcounter> +QPINC: "P2", <pincounter>,<pukcounter> OK |
| Write Command AT+QPINC=<facility> | Response +QPINC: <facility>,<pincounter>,<pukcounter> OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<facility>	“SC”	(U)SIM PIN “P2”	(U)SIM PIN2

<pincounter>	Number of attempts left to enter the password of PIN

<pukcounter>	Number of attempts left to enter the password of PUK

## AT+QINISTAT	Query Initialization Status of (U)SIM Card

The command is used to query the initialization status of (U)SIM card.

| AT+QINISTAT Query Initialization Status of (U)SIM Card |  |
| --- | --- |
| Test Command AT+QINISTAT=? | Response +QINISTAT: (0-7) OK |
| Execution Command AT+QINISTAT | Response +QINISTAT: <status> OK |
| Maximum Response Time | 300ms |

Parameter

<status>	Initialization status of (U)SIM card. Actual value is the sum of several of the following four kinds (e.g. 7=1+2+4 means CPIN READY & SMS DONE & PB DONE).

0	Initial state

- CPIN READY. Operation like lock/unlock PIN is allowed

- SMS initialization completed

4	Phonebook initialization completed

## AT+QSIMDET	(U)SIM Card Detection

The command enables (U)SIM card hot-swap function. (U)SIM card is detected by GPIO interrupt. The level of (U)SIM card detection pin should also be set when the (U)SIM card is inserted.

| AT+ QSIMDET (U)SIM Card Detection |  |
| --- | --- |
| Test Command AT+QSIMDET=? | Response +QSIMDET: (0,1),(0,1) OK |
| Read Command AT+QSIMDET? | Response +QSIMDET: <enable>,<insertlevel> OK |
| Write Command | Response |

| AT+QSIMDET=<enable>,<insertlevel> | OK ERROR |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<enable>	Enable or disable (U)SIM card detection

0	Disable

1	Enable

<insertlevel>	The level of (U)SIM detection pin when a (U)SIM card is inserted

0	Low level

1	High level

- Hot-swap function is invalid if the configured value of <insertlevel> is inconsistent with hardware design.

- Hot-swap function takes effect after the module is restarted.

NOTES

Example

AT+QSIMDET=1,0

OK

//Set (U)SIM card detection pin level as low when (U)SIM card is inserted

<Remove (U)SIM card>

+CPIN: NOT READY

<Insert (U)SIM card>

+CPIN: READY

//If PIN1 of the (U)SIM card is unlocked

## AT+QSIMSTAT	(U)SIM Card Insertion Status Report

The command queries (U)SIM card insertion status or determines whether (U)SIM card insertion status report is enabled. The configuration of this command can be saved by AT&W.

| AT+ QSIMSTAT (U)SIM Card Insertion Status Report |  |
| --- | --- |
| Test Command AT+QSIMSTAT=? | Response +QSIMSTAT: (0,1) OK |

| Read Command AT+QSIMSTAT? | Response +QSIMSTAT: <enable>,<inserted_status> OK |
| --- | --- |
| Write Command AT+QSIMSTAT=<enable> | Response OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<enable>	Enable or disable (U)SIM card insertion status report. If it is enabled, when (U)SIM card is removed or inserted, the URC +QSIMSTAT: <enable>,<insertedstatus> will be reported.

0	Disable

1	Enable

<insertedstatus>	(U)SIM card is inserted or removed. This argument is not allowed to be set.

0	Removed

- Inserted

- Unknown, before (U)SIM initialization

Example

AT+QSIMSTAT?

+QSIMSTAT: 0,1

//Query (U)SIM card insertion status

OK

AT+QSIMDET=1,0 OK AT+QSIMSTAT=1 OK AT+QSIMSTAT?

+QSIMSTAT: 1,1

//Enable (U)SIM card insertion status report

OK

<Remove (U)SIM card>

+QSIMSTAT : 1,0

//Report of (U)SIM card insertion status, removed

+CPIN: NOT READY

AT+QSIMSTAT?

+QSIMSTAT: 1,0

OK

<Insert (U)SIM card>

+QSIMSTAT: 1,1

//Report of (U)SIM card insertion status, inserted

+CPIN: READY

# Network	Service	Commands

## AT+COPS	Operator Selection

The command returns the current operators and their status, and allows setting automatic or manual network selection.

| AT+COPS Operator Selection |  |
| --- | --- |
| Test Command AT+COPS=? | Response TA returns a set of five parameters, each representing an operator presenting in the network. Any of the formats may be unavailable and should then be an empty field. The list of operators shall be in the order of: home network, networks referenced in (U)SIM and other networks. +COPS: (list of supported <stat>,long alphanumeric <oper>, short alphanumeric <oper>,numeric <oper>s)[,<Act>])s] [,,(list of supported <mode>s),(list of supported <format>s)] OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Read Command AT+COPS? | Response TA returns the current mode and the currently selected operator. If no operator is selected, <format>, <oper> and <Act> are omitted. +COPS: <mode>[,<format>[,<oper>][,<Act>]] OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Write Command AT+COPS=<mode>[,<format>[,<oper >[,<Act>]]] | Response TA forces an attempt to select and register the GSM/UMTS network operator. If the selected operator is not available, no other operator shall be selected (except <mode>=4). The format of selected operator name shall apply to further Read |

|  | Command (AT+COPS?). OK If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 180s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<stat>	0	Unknown

- Operator available

- Current operator

- Operator forbidden

<oper>	Operator in format as per <mode>

<mode>	0	Automatic mode. <oper> field is ignored

- Manual operator selection. <oper> field shall be present and <Act> optionally

- Manually deregister from network

- Set only <format> (for AT+COPS? Read Command), and do not attempt registration/deregistration (<oper> and <Act> fields are ignored). This value is invalid in the response of Read Command.

- Manual/automatic selection. <oper> field shall be presented. If manual selection fails, automatic mode (<mode>=0) is entered

<format>	0	Long format alphanumeric <oper> which can be up to 16 characters long

- Short format alphanumeric <oper>

- Numeric <oper>. GSM location area identification number

<Act>	Access technology selected. Values 3, 4, 5 and 6 occur only in the response of Read Command while MS is in data service state and is not intended for the AT+COPS Write Command.

0	GSM

- UTRAN

- GSM W/EGPRS

- UTRAN W/HSDPA

- UTRAN W/HSUPA

- UTRAN W/HSDPA and HSUPA

- E-UTRAN

100	CDMA

Example

AT+COPS=?

+COPS:

//List all current network operators

(1,"CHN-UNICOM","UNICOM","46001",2),(1,"CHN-UNICOM","UNICOM","46001",0),(2,"CHN-UNICO

M","UNICOM","46001",7),(1,"46011","46011","46011",7),(3,"CHINA

MOBILE","CMCC","46000",0),,(0,1,2,3,4),(0,1,2)

OK

AT+COPS?

+COPS: 0,0,“CHN-UNICOM”,7

//Query the currently selected network operator

OK

## AT+CREG	Network Registration

The Read Command returns the network registration status. The write command sets whether or not to present URC.

| AT+CREG Network Registration Status |  |
| --- | --- |
| Test Command AT+CREG=? | Response +CREG: (list of supported <n>s) OK |
| Read Command AT+CREG? | Response TA returns the status of result code presentation and an integer <stat> which shows whether the network has currently indicated the registration of the ME. Location information elements <lac> and <ci> are returned only when <n>=2 and ME is registered on the network. +CREG: <n>,<stat>[,<lac>,<ci>[,<Act>]] OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Write Command AT+CREG[=<n>] | Response TA controls the presentation of an unsolicited result code +CREG: <stat> when <n>=1 and there is a change in the ME network registration status. OK |
| Maximum Response Time | 300ms |

Reference 3GPP TS 27.007

| Parameter |  |  |
| --- | --- | --- |
| <n> | 0 | Disable network registration unsolicited result code |
|  | 1 | Enable network registration unsolicited result code: +CREG: <stat> |
|  | 2 | Enable network registration unsolicited result code with location information: |
|  |  | +CREG: <stat>[,<lac>,<ci>[,<Act>]] |
| <stat> | 0 | Not registered. ME is not currently searching a new operator to register to |
|  | 1 | Registered, home network |
|  | 2 | Not registered, but ME is currently searching a new operator to register to |
|  | 3 | Registration denied |
|  | 4 | Unknown |
|  | 5 | Registered, roaming |

<lac>	String type. Two bytes location area code in hexadecimal format

<ci>	String type. 16-bit (GSM) or 28-bit (UMTS/LTE) cell ID in hexadecimal format

<Act>	Access technology selected

0	GSM

- UTRAN

- GSM W/EGPRS

- UTRAN W/HSDPA

- UTRAN W/HSUPA

- UTRAN W/HSDPA and HSUPA

- E-UTRAN

Example

AT+CREG=1

OK

+CREG: 1

AT+CREG=2 OK

//URC reports that ME has registered on network

//Activate extended URC mode

+CREG: 1,“D509”,”80D413D”,7

//URC reports that operator has found location area code

and cell ID

## AT+CSQ	Signal Quality Report

The command indicates the received signal strength <rssi> and the channel bit error rate <ber>.

| AT+CSQ Signal Quality Report |  |
| --- | --- |
| Test Command AT+CSQ=? | Response The Test Command returns values supported by the TA. +CSQ: (list of supported <rssi>s),(list of supported <ber>s) OK |
| Execution Command AT+CSQ | Response The Execution Command returns received signal strength indication <rssi> and channel bit error rate <ber> from the ME. +CSQ: <rssi>,<ber> OK +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

| Parameter |  |  |  |
| --- | --- | --- | --- |
| <rssi> | 0 | -113dBm or less |  |
|  | 1 | -111dBm |  |
|  | 2...30 | -109dBm... -53dBm |  |
|  | 31 | -51dBm or greater |  |
|  | 99 | Not known or not detectable |  |
|  | 100 | -116dBm or less |  |
|  | 101 | -115dBm |  |
|  | 102...190 | -114dBm...-26dBm |  |
|  | 191 | -25dBm or greater |  |
|  | 199 | Not known or not detectable |  |
|  | 100~199 | Extended to be used in TD-SCDMA indicating received signal | code |
|  |  | power (RSCP) |  |

<ber>	Channel bit error rate (in percent)

0...7	As RXQUAL values in the table in 3GPP TS 45.008 subclause 8.2.4

99	Not known or not detectable

Example

AT+CSQ=?

+CSQ: (0-31,99),(0-7,99)

OK

AT+CSQ

+CSQ: 28,99

//The current signal strength indication is 28 and channel bit error rate is 99

OK

After using network related commands such as AT+CCWA and AT+CCFC, users are advised to wait for 3s before entering AT+CSQ. This is recommended to ensure that any network access required for the preceding command has been finished.

NOTE

## AT+CPOL	Preferred Operator List

The command edits and queries the list of preferred operators.

| AT+CPOL Preferred Operator List |  |
| --- | --- |
| Test Command AT+CPOL=? | Response +CPOL: (list of supported <index>s),(list of supported <format>s) OK |
| Read Command AT+CPOL? | Response Query the list of preferred operators: +CPOL: <index>,<format>,<oper>[,<GSM>,<GSM_compact,<UTR AN>,<E-UTRAN>] <index>,<format>,<oper>[,<GSM>,<GSM_compact,<UTR AN>,<E-UTRAN>] […] OK |
| Write Command AT+CPOL=<index>[,<format>[,<oper> [<GSM>,<GSM_compact>,<UTRAN>, <E-UTRAN>]]] | Response Edit the list of preferred operators: OK ERROR |

|  | If the <index> is given but the <operator> is left out, the entry is deleted. |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<index>	Integer type. The order number of operator in the (U)SIM preferred operator list

<format>	0	Long format alphanumeric <oper>

- Short format alphanumeric <oper>

- Numeric <oper>

<oper>	String  type.  <format>  indicates  the  format  is  alphanumeric  or  numeric  (see

#### AT+COPS)

<GSM>	GSM access technology

0	Access technology is not selected

1	Access technology is selected

<GSM_compact>	GSM compact access technology

0	Access technology is not selected

1	Access technology is selected

<UTRAN>	UTRAN access technology

0	Access technology is not selected

1	Access technology is selected

<E-UTRAN>	E-UTRAN access technology

0	Access technology is not selected

1	Access technology is selected

The access technology selection parameters <GSM>, <GSM_compact>, <UTRAN> and <E-UTRAN>

are required for SIM cards or UICC‟s containing PLMN selector with access technology.

NOTE

## AT+COPN	Read Operator Names

The command returns the list of operator names from the ME. Each operator code <numericn> that has an alphanumeric equivalent <alphan> in the ME memory is returned.

| AT+COPN Read Operator Names |  |
| --- | --- |
| Test Command AT+COPN=? | Response OK |
| Execution Command AT+COPN | Response +COPN: <numeric1>,<alpha1> [+COPN: <numeric2>,<alpha2> […]] OK +CME ERROR: <err> |
| Maximum Response Time | Depends on the number of operator names. |
| Reference 3GPP TS 27.007 |  |

Parameter

<numericn>	String type. Operator in numeric format (see AT+COPS)

<alphan>	String type. Operator in long alphanumeric format (see AT+COPS)

## AT+CTZU	Automatic Time Zone Update

The Write Command enables and disables automatic time zone update via NITZ. The configuration is stored to NV automatically.

| AT+CTZU Automatic Time Zone Update |  |
| --- | --- |
| Test Command AT+CTZU=? | Response +CTZU: (0,1,3) OK |
| Write Command AT+CTZU=<onoff> | Response OK ERROR |
| Read Command AT+CTZU? | Response +CTZU: <onoff> |

|  | OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<onoff>	Integer type. The mode of automatic time zone update.

0	Disable automatic time zone update via NITZ.

1	Enable automatic time zone update via NITZ

3	Enable automatic time zone update via NITZ and update LOCAL time to RTC

Example

AT+CTZU?

+CTZU: 0

OK AT+CTZU=?

+CTZU: (0,1,3)

OK AT+CTZU=1 OK AT+CTZU?

+CTZU: 1

OK

## AT+CTZR   Time Zone Reporting

This command controls the time zone reporting of changed event. If reporting is enabled, the MT returns the unsolicited result code +CTZV: <tz> or +CTZE: <tz>,<dst>,<time> whenever the time zone is changed. The configuration is stored to NV automatically.

| AT+CTZR Time Zone Reporting |  |
| --- | --- |
| Test Command AT+CTZR=? | Response +CTZR: (0-2) OK |

| Write Command AT+CTZR=<reporting> | Response OK ERROR |
| --- | --- |
| Read Command AT+CTZR? | Response +CTZR: <reporting> OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<reporting>	Integer type. The mode of time zone reporting

0	Disable time zone reporting of changed event

- Enable time zone reporting of changed event by unsolicited result code:

#### +CTZV: <tz>

- Enable extended time zone reporting by unsolicited result code:

#### +CTZE: <tz>,<dst>,<time>

<tz>	String type. The sum of the local time zone (difference between the local time and GMT is expressed in quarters of an hour) plus daylight saving time. The format is “±zz”, expressed as a fixed width, two-digit integer with the range -48 ... +56. To maintain a fixed width, numbers in the range -9 ... +9 are expressed with a leading zero, e.g. “-09”, “+00” and “+09”.

<dst>	Integer type. Indicates whether <tz> includes daylight savings adjustment

0	<tz> includes no adjustment for daylight saving time

- <tz> includes +1 hour (equals 4 quarters in <tz>) adjustment for daylight saving time

- <tz> includes +2 hours (equals 8 quarters in <tz>) adjustment for daylight saving time

<time>	String type. The local time. The format is “YYYY/MM/DD,hh:mm:ss”, expressed as integers representing year (YYYY), month (MM), date (DD), hour (hh), minute (mm) and second (ss). This parameter can be provided by the network when delivering time zone information and will be presented in the unsolicited result code of extended time zone reporting if provided by the network.

Example

AT+CTZR=2 OK AT+CTZR?

+CTZR: 2

OK

+CTZE: “+32”,0,“2017/11/04,06:51:13”

//<reporting> is 2

## AT+QLTS	Obtain the Latest Time Synchronized Through Network

The command is used to obtain the latest time synchronized through network.

| AT+QLTS Obtain the Latest Time Synchronized Through Network |  |
| --- | --- |
| Test Command AT+QLTS=? | Response +QLTS: list of supported <mode>s OK |
| Execution Command AT+QLTS | Response Execution Command returns the latest time that has been synchronized through network: +QLTS: <time>,<dst> OK |
| Write Command AT+QLTS=<mode> | Response +QLTS: <time>,<dst> OK ERROR If error is related to ME functionality +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<mode>	Query network time mode

0	Query the latest time that has been synchronized through network

- Query  the  current  GMT  time  calculated  from  the  latest  time  that  has  been synchronized through network

- Query  the  current  LOCAL time  calculated  from  the  latest  time  that  has  been synchronized through network

<time>     String type value. Format is "yy/MM/dd,hh:mm:ss±zz", where characters indicate year (two last digits), month, day, hour, minutes, seconds and time zone (indicates the difference, expressed in quarters of an hour, between the local time and GMT; range -48...+48). E.g. 6th of May 2004, 22:10:00 GMT+2 hours equals to “04/05/06,22:10:00+08”

<dst>	Daylight saving time.

If the time has not been synchronized through network, the command will return a null time string: +QLTS: ""

NOTE

Example

AT+QLTS=?

+QLTS: (0-2)

//Query supported network time modes

OK

AT+QLTS

//Query the latest time synchronized through network

+QLTS: "2017/10/13,03:40:48+32,0"

OK

AT+QLTS=0

//Query the latest time synchronized through network. It offers the same

function as Execution Command AT+QLTS.

+QLTS: "2017/10/13,03:40:48+32,0"

OK

AT+QLTS=1

//Query the current GMT time calculated from the latest time that has been

synchronized through network

+QLTS: "2017/10/13,03:41:22+32,0"

OK

AT+QLTS=2

//Query the current LOCAL time calculated from the latest time that has been

synchronized through network

+QLTS: "2017/01/13,11:41:23+32,0"

OK

## AT+QNWINFO	Query Network Information

The command indicates network information such as access technology selected, the operator and the band selected.

| AT+QNWINFO Query Network Information |  |
| --- | --- |
| Test Command AT+QNWINFO=? | Response OK |
| Execution Command AT+QNWINFO | Response +QNWINFO: <Act>,<oper>,<band>,<channel> |

|  | OK |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<Act>	String type. The access technology selected “NONE”

“CDMA1X”

“CDMA1X AND HDR” “CDMA1X AND EHRPD” “HDR”

“HDR-EHRPD” “GSM”

“GPRS”

“EDGE” “WCDMA” “HSDPA” “HSUPA” “HSPA+” “TDSCDMA” “TDD LTE” “FDD LTE”

<oper>	String type. The operator in numeric format

<band>	String type. The band selected “CDMA BC0” – “CDMA BC19” “GSM 450”

“GSM 480”

“GSM 750”

“GSM 850”

“GSM 900”

“GSM 1800”

“GSM 1900”

“WCDMA 2100”

“WCDMA 1900”

“WCDMA 1800”

“WCDMA 1700 US”

“WCDMA 850”

“WCDMA 800”

“WCDMA 2600”

“WCDMA 900”

“WCDMA 1700 JAPAN”

“WCDMA 1500”

“WCDMA 850 JAPAN”

“LTE BAND 1” – “LTE BAND 43” “TDSCDMA BAND A” “TDSCDMA BAND B” “TDSCDMA BAND C” “TDSCDMA BAND D” “TDSCDMA BAND E” “TDSCDMA BAND F”

<channel>	Integer type. Channel ID

Example

AT+QNWINFO=? OK AT+QNWINFO

+QNWINFO: "FDD LTE",46001,"LTE BAND 3",1650

OK

## AT+QSPN	Display the Name of Registered Network

| AT+QSPN Display the Name of Registered Network |  |
| --- | --- |
| Test Command AT+QSPN=? | Response OK |
| Execution Command AT+QSPN | Response +QSPN: <FNN>,<SNN>,<SPN>,<alphabet>,<RPLMN> OK |
| Reference |  |

Parameter

<SPN>	Service provider name

<alphabet>	Alphabet of full network name and short network name

0	GSM 7-bit default alphabet

1	UCS2

<RPLMN>	Registered PLMN

<FNN>	Full network name

<SNN>	Short network name

- If <alphabet> is 0, <FNN> and <SNN> will be shown in GSM 7 bit default alphabet string.

- If <alphabet> is 1, <FNN> and <SNN> will be shown in UCS2 hexadecimal string.

NOTES

Example

AT+QSPN

//Query the EONS information of RPLMN

+QSPN: "CHN-UNICOM","UNICOM","",0,"46001"

OK

# Call	Related	Commands

## ATA	Answer an Incoming Call

The command connects the module to an incoming voice or data call indicated by a RING URC.

| ATA Answer an Incoming Call |  |
| --- | --- |
| Execution Command ATA | Response TA sends off-hook to the remote station. Response in case of data call, if successfully connected: CONNECT <text> And TA switches to data mode. Note: <text> outputs only when <value> is greater than 0 in ATX <value> parameter setting. When TA returns to command mode after call release: OK Response in case of voice call, if successfully connected: OK Response if no connection: NO CARRIER |
| Maximum Response Time | 90s, determined by network. |
| Reference V.25ter |  |

- Any additional commands on the same command line are ignored.

- This command may be aborted generally by receiving a character during execution. The aborting is not possible during some states of connection establishment such as handshaking.

- See also ATX.

NOTES

Example

RING

AT+CLCC

//A voice call is ringing

+CLCC: 1,0,0,1,0,“”,128

+CLCC: 2,1,4,0,0,“02154450290”,129

//PS call in LTE mode

//Incoming call

OK

ATA OK

//Accept the voice call with ATA

## ATD	Mobile Originated Call to Dial a Number

The command can be used to set up outgoing voice and data calls. Supplementary services can also be controlled with this command.

| ATD Mobile Originated Call to Dial a Number |  |
| --- | --- |
| Execution Command ATD<n>[<mgsm>][;] | Response This command can be used to set up outgoing voice, data or fax calls. It also serves to control supplementary services. If no dial tone and (parameter setting ATX2 or ATX4): NO DIALTONE If busy and (parameter setting ATX3 or ATX4): BUSY If a connection cannot be established: NO CARRIER If connection is successful and non-voice call. CONNECT <text> And TA switches to data mode. Note: <text> outputs only when <value> is greater than 0 in ATX<value> parameter setting. When TA returns to command mode after call release: OK If connection is successful and voice call: OK |
| Maximum Response Time | 5s, determined by network (AT+COLP=0). |
| Reference V.25ter |  |

Parameter

<n>	String of dialing digits and optionally V.25ter modifiers Dialing digits: 0-9, * , #, +, A, B, C

Following V.25ter modifiers are ignored: ,(comma), T, P, !, W, @

<mgsm>	String of GSM modifiers:

I	Actives CLIR (Disable presentation of own number to called party)

i	Deactivates CLIR (Enable presentation of own number to called party)

G	Activates closed user group invocation for this call only

g	Deactivates closed user group invocation for this call only

<;>	Only required to set up voice call, return to command mode

- This command may be aborted generally by receiving an ATH command or a character during execution. The aborting is not possible during some states of connection establishment such as handshaking.

- Parameter “I” and “i” can be omitted only when there is no “*” or “#” code within the dial string.

- See ATX command for setting result code and call monitoring parameters.

- Responses returned after dialing with ATD

For voice call, two different responses mode can be determined. TA returns “OK” immediately either after dialing was completed or after the call was established. The setting is controlled by AT+COLP. Factory default is AT+COLP=0, which causes the TA to return “OK” immediately after dialing was completed. Otherwise TA will return “OK”, “BUSY”, “NO DIAL TONE”, or “NO CARRIER”.

- Using ATD during an active voice call:

  - When a user originates a second voice call while there is already an active voice call, the first call will be automatically put on hold.

  - The current states of all calls can be easily checked at any time by using AT+CLCC command.

NOTES

Example

ATD10086;

OK

//Dialing out the party‟s number

## ATH	Disconnect Existing Connection

The command disconnects circuit switched data calls or voice calls. AT+CHUP is also used to disconnect the voice call.

| ATH Disconnect Existing Connection |  |
| --- | --- |
| Execution Command ATH[n] | Response Disconnect existing call by local TE from command line and terminate the call. OK |
| Maximum Response Time | 90s, determined by network. |
| Reference V.25ter |  |

Parameter

<n>	0	Disconnect existing call from command line and terminate the call

## AT+CVHU	Voice Hang up Control

The command controls whether ATH can be used to disconnect the voice call.

| AT+CVHU Voice Hang up Control |  |
| --- | --- |
| Test Command AT+CVHU=? | Response +CVHU: (list of supported <mode>s) OK |
| Read Command AT+CVHU? | Response +CVHU: <mode> OK |
| Write Command AT+CVHU=<mode> | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

| Parameter |  |  |
| --- | --- | --- |
| <mode> | 0 | ATH can be used to disconnect the voice call. |
|  | 1 | ATH is ignored but OK response is returned. |

## AT+CHUP	Hang up Call

The command cancels all voice calls in the state of Active, Waiting and Held. For data connections, use

ATH.

| AT+CHUP Hang up Call |  |
| --- | --- |
| Test Command AT+CHUP=? | Response OK |
| Execution Command AT+CHUP | Response OK ERROR |
| Maximum Response Time | 90s, determined by network. |
| Reference 3GPP 27.007 |  |

Example

RING

//Incoming call

AT+CHUP

OK

//Hang up the call

## 7.6. +++	Switch from Data Mode to Command Mode

The +++ character sequence causes the module to switch from data mode to command mode. It allows inputting AT commands while maintaining the data connection with the remote device.

| +++ Switch from Data Mode to Command Mode |  |
| --- | --- |
| Execution Command +++ | Response This command is only available when TA is in data mode. The “+++” character sequence causes the TA to cancel the data flow over the AT interface and switch to command mode. This allows entering AT command while maintaining the data connection with the remote server or, accordingly, the GPRS |

|  | connection. OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

- To prevent the +++ escape sequence from being misinterpreted as data, the following sequence should be followed:

  - Do not input any character within 1s before inputting +++.

  - Input +++ within 1s, and no other characters can be inputted during the time.

  - Do not input any character within 1s after +++ has been inputted.

  - Switch to command mode successfully; otherwise return to Step 1.

- To return back to data mode from command mode, please enter ATO.

- Another way to change to command mode is through DTR level change, and please refer to AT&D

command for details.

NOTES

## ATO	Switch from Command Mode to Data Mode

The command resumes the connection and switches back from command mode to data mode.

| ATO Switch from Command Mode to Data Mode |  |
| --- | --- |
| Execution Command ATO[n] | Response TA resumes the connection and switches back to data mode from command mode. If connection is not successfully resumed: NO CARRIER If connection is successfully resumed, TA returns to data mode from command mode CONNECT <text> |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0	Switch from command mode to data mode

When TA returns to data mode from command mode successfully, CONNECT <text> is returned. Please note that <text> outputs only when <value> is greater than 0 in ATX<value> parameter setting.

NOTE

## ATS0	Set Number of Rings before Automatically Answering Call

The command controls automatic answering mode for the incoming calls.

| ATS0 Set Number of Rings before Automatically Answering Call |  |
| --- | --- |
| Read Command ATS0? | Response <n> OK |
| Write Command ATS0=<n> | Response This parameter setting determines the number of rings before auto-answer. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0	Automatic answering is disabled

1-255	Enable automatic answering on the ring number specified

If <n> is set too high, the calling party may hang up before the call is answered automatically.

NOTE

Example

ATS0=3

OK

//Set three rings before automatically answering a call

RING

//A call is coming

RING

RING

//Automatically answering the call after three rings

## ATS6	Set Pause before Blind Dialing

The command is implemented for compatibility reasons only, and has no effect.

| ATS6 Set Pause before Blind Dialing |  |
| --- | --- |
| Read Command ATS6? | Response <n> OK |
| Write Command ATS6=<n> | Response OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	0-2-10	Number of seconds to wait before blind dialing

## ATS7   Set Time to Wait for Connection Completion

The command specifies the amount of time (unit: second) to wait for the connection completion in case of answering or originating a call. If no connection is established during the time, the module disconnects from the line.

| ATS7 Set Time to Wait for Connection Completion |  |
| --- | --- |
| Read Command ATS7? | Response <n> OK |

| Write Command ATS7=<n> | Response This parameter setting determines the amount of time (unit: second) to wait for the connection completion in case of answering or originating a call. OK |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

| <n> | 0 | Disabled |
| --- | --- | --- |
|  | 1-255 | Number of seconds to wait for connection completion. Unit: second. |

## ATS8	Set the Time to Wait for Comma Dial Modifier

The command is implemented for compatibility reasons only, and has no effect.

| ATS8 Set the Time to Wait for Comma Dial Modifier |  |
| --- | --- |
| Read Command ATS8? | Response <n> OK |
| Write Command ATS8=<n> | Response OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

| <n> | 0 | No pause when comma encountered in dial string |
| --- | --- | --- |
|  | 1-2-255 | Number of seconds to wait for comma dial modifier |

## ATS10	Set Disconnection Delay after Indicating the Absence of Data Carrier

The command determines the amount of time (unit: tenths of a second) during which the UE remains connected in absence of a data carrier.

| ATS10 Set Disconnection Delay after Indicating the Absence of Data Carrier |  |
| --- | --- |
| Read Command ATS10? | Response <n> OK |
| Write Command ATS10=<n> | Response This parameter setting determines the amount of time (unit: tenths of a second) during which the TA will remain connected in absence of a data carrier. If the data carrier is once more detected before disconnection, the TA remains connected. OK |
| Maximum Response Time | 300ms |
| Reference V.25ter |  |

Parameter

<n>	1-15-254	Number of tenths of seconds to wait before disconnecting after UE has indicated the absence of received line signal

## AT+CBST	Select Bearer Service Type

The Write Command selects the bearer service <name>, the data rate <speed> and the connection element <ce> to be used when data calls are originated.

| AT+CBST Select Bearer Service Type |  |
| --- | --- |
| Test Command AT+CBST=? | Response +CBST: (list of supported <speed>s),(list of supported <name>s),(list of supported <ce>s) OK |
| Read Command | Response |

| AT+CBST? | +CBST: <speed>,<name>,<ce> OK |
| --- | --- |
| Write Command AT+CBST=[<speed>[,<name>[,<ce>]]] | Response TA selects the bearer service <name> with data rate <speed>, and the connection element <ce> to be used when data calls are originated. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<speed>	0	Automatic speed selection 7	9600 bps (V.32)

12	9600 bps (V.34)

14	14400 bps (V.34)

16	28800 bps (V.34)

17	32000 bps (V.34)

39	9600 bps (V.120)

43	14400 bps (V.120)

48	28800 bps (V.120)

51	56000 bps (V.120)

71	9600 bps (V.110)

75	14400 bps (V.110)

- 28800 bps (V.110 or X.31 flag stuffing)

- 38400 bps (V.110 or X.31 flag stuffing)

- 56000 bps (V.110 or X.31 flag stuffing; this setting can be used in conjunction with asynchronous non-transparent UDI or RDI service in order to get FTM)

- 64000  bps  (X.31  flag  stuffing;  this  setting  can  be  used  in  conjunction  with asynchronous non-transparent UDI service in order to get FTM)

116	64000 bps (bit transparent)

134	64000 bps (multimedia)

<name>	0	Asynchronous Modem

1	Synchronous Modem

4	Asynchronous Modem (RDI)

<ce>	0	Transparent

1	Non-transparent

#### Table 4: Parameter Configurations Supported by AT+CBST

<speed>	GSM

WCDMA

SYNC.

ASYNC.

ASYNC.

(RDI)

NON-

TRANSP.

TRANSP.

0	Y	Y	N	Y	N	N	Y

7	Y	N	N	Y	N	N	Y

12	Y	N	N	Y	N	N	Y

14	Y	Y	N	Y	N	N	Y

- N	Y	N	Y	N	N	Y

- N	Y	N	Y	N	N	Y

39	Y	N	N	Y	N	N	Y

43	Y	Y	N	Y	N	N	Y

48	N	Y	N	Y	N	N	Y

51	N	Y	N	Y	N	N	Y

71	Y	N	N	Y	N	N	Y

75	Y	Y	N	Y	N	N	Y

- Y	Y	N	Y	N	N	Y

- Y	Y	N	Y	N	N	Y

- Y	Y	N	Y	Y	N	Y

- N	Y	N	Y	N	N	Y

116	N	Y	Y	N	N	Y	N

134	N	Y	Y	N	N	Y	N

3GPP TS 22.002 lists the allowed combinations of the sub-parameters.

NOTE

## AT+CSTA	Select Type of Address

The Write Command selects the type of number for further dialing commands ATD according to 3GPP Specifications. Test command returns values supported a compound value.

| AT+CSTA Select Type of Address |  |
| --- | --- |
| Test Command AT+CSTA=? | Response +CSTA: (list of supported <type>s) OK |
| Read Command AT+CSTA? | Response +CSTA: <type> OK |
| Write Command AT+CSTA=<type> | Response OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<type>	Current address type setting.

129	Unknown type

145	International type (contains the character “+”)

## AT+CLCC	List Current Calls of ME

The execution command returns the list of all current calls. If the command is executed successfully, but no calls existed, no information response but OK is sent to TE.

| AT+CLCC List Current Calls of ME |  |
| --- | --- |
| Test Command AT+CLCC=? | Response OK |
| Execution Command AT+CLCC | Response TA returns a list of current calls of ME. If the command is executed successfully, but no calls are existed, then no information but OK response is sent to TE. [+CLCC : |

|  | <id1>,<dir>,<stat>,<mode>,<mpty>[,<number>,<type>[,< alpha>]] [+CLCC: <id2>,<dir>,<stat>,<mode>,<mpty>[,<number>,<type>[,< alpha>]] [...] OK If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |

Parameter

<idx>	Integer type. Call identification number as described in  3GPP TS  22.030 subclause

4.5.5.1. This number can be used in AT+CHLD command operation

<dir>	0	Mobile originated (MO) call

1	Mobile terminated (MT) call

<stat>	State of the call

0	Active

- Held

- Dialing (MO call)

- Alerting (MO call)

- Incoming (MT call)

- Waiting (MT call)

<mode>	Bearer/tele service

|  | 0 | Voice |
| --- | --- | --- |
|  | 1 | Data |
|  | 2 | FAX |
| <mpty> | 0 | Call is not one of multiparty (conference) call parties |
|  | 1 | Call is one of multiparty (conference) call parties |

<number>	Phone number in string type in format specified by <type>

<type>	Type of address of octet in integer format (refer to 3GPP TS 24.008 subclause 10.5.4.7 for details). Usually, it has three kinds of values:

129	Unknown type

145	International type (contains the character “+”)

161	National type

<alpha>	Alphanumeric representation of <number> corresponding to the entry found in phonebook.

Example

ATD10086;

OK AT+CLCC

+CLCC: 1,0,0,1,0,"",128

+CLCC: 2,0,0,0,0,“10086”,129

//Establish a call

//PS call in LTE mode

//Establish a call, and the call has been answered

OK

## AT+CR   Service Reporting Control

The command controls the module whether or not to transmit an intermediate result code +CR: <serv> to the TE when a call is being set up.

If it is enabled, an intermediate result code is transmitted at the point during connect negotiation at which the TA has determined which speed and quality of service will be used, before any error control or data compression reports are transmitted, and before any final result code (e.g. CONNECT) is transmitted.

| AT+CR Service Reporting Control |  |
| --- | --- |
| Test Command AT+CR=? | Response +CR: (list of supported <mode>s) OK |
| Read Command AT+CR? | Response +CR: <mode> OK |
| Write Command AT+CR=[<mode>] | Response TA controls whether or not intermediate result code +CR: <serv> is returned from the TA to the TE when a call is being set up. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<mode>	0	Disable

1	Enable

| <serv> | ASYNC | Asynchronous transparent |
| --- | --- | --- |
|  | SYNC | Synchronous transparent |
|  | REL ASYNC | Asynchronous non-transparent |
|  | REL SYNC | Synchronous non-transparent |
|  | GPRS | GPRS |

## AT+CRC   Set Cellular Result Codes for Incoming Call Indication

The command controls whether or not to use the extended format of incoming call indication. When it is enabled, an incoming call is indicated to the TE with unsolicited result code +CRING: <type> instead of the normal RING.

| AT+CRC Set Cellular Result Codes for Incoming Call Indication |  |
| --- | --- |
| Test Command AT+CRC=? | Response +CRC: (list of supported <mode>s) OK |
| Read Command AT+CRC? | Response +CRC: <mode> OK |
| Write Command AT+CRC=[<mode>] | Response TA controls whether or not the extended format of incoming call indication is used. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<mode>	0	Disable extended format

1	Enable extended format

<type>	ASYNC	Asynchronous transparent SYNC	Synchronous transparent

REL ASYNC	Asynchronous non-transparent REL SYNC	Synchronous non-transparent FAX	Facsimile

VOICE	Voice

Example

AT+CRC=1	//Enable extended format

#### OK

+CRING: VOICE	//Indicate incoming call to the TE

#### ATH OK

AT+CRC=0	//Disable extended format

#### OK

RING	//Indicate incoming call to the TE

#### ATH OK

## AT+CRLP	Select Radio Link Protocol Parameter

The Write Command sets radio link protocol (RLP) parameters used when non-transparent data calls are originated.

| AT+CRLP Select Radio Link Protocol Parameter |  |
| --- | --- |
| Test Command AT+CRLP=? | Response TA returns values supported. RLP (Radio Link Protocol) versions 0 and 1 share the same parameter set. TA returns only one line for this set (where <ver> is not presented). +CRLP: (list of supported <iws>s),(list of supported <mws>s),(list of supported <T1>s),(list of supported <N2>s),<ver> +CRLP: (list of supported <iws>s),(list of supported <mws>s),(list of supported <T1>s),(list of supported <N2>s),<ver> +CRLP: (list of supported <iws>s),(list of supported <mws>s),(list of supported <T1>s),(list of supported <N2>s),<ver> OK |
| Read Command AT+CRLP? | Response TA returns current settings for RLP version. RLP versions 0 and 1 share the same parameter set. TA returns only one line for this set (where <ver> is not presented). +CRLP: <iws>,<mws>,<T1>,<N2>,<ver> +CRLP: <iws>,<mws>,<T1>,<N2>,<ver> |

|  | +CRLP: <iws>,<mws>,<T1>,<N2>,<ver> OK |
| --- | --- |
| Write Command AT+CRLP=[<iws>[,<mws>[,<T1>[,<N2 >[,<ver>]]]]] | Response TA sets radio link protocol (RLP) parameters used when non-transparent data calls are set up. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS27.007 |  |

Parameter

<iws>	0-61	Interworking window size (IWF to MS) 0-240-488	For <ver>=2

<mws>	0-61	Mobile window size (MS to IWF) 0-240-488	For <ver>=2

<T1>	38-48-255	Acknowledgment timer T1 in a unit of 10ms 42-52-255	For <ver>=2

<N2>	1-6-255	Retransmission attempts N2

<ver>	0-2	RLP version number in integer format

## AT+QECCNUM   Configure Emergency Call Numbers

The command can be used to query, add and delete ECC numbers (emergency call numbers). There are two kinds of ECC numbers: ECC numbers without (U)SIM and ECC numbers with (U)SIM. The default ECC numbers without SIM is 911, 112, 00, 08, 110, 999, 118 and 119. The default ECC number with (U)SIM is 911 and 112. 911 and 112 will always be supported as ECC numbers, and cannot be deleted. ECC numbers can be saved into NV automatically. If the (U)SIM card contains ECC file, the numbers in ECC file can also be regarded as ECC numbers.

The maximal supported ECC numbers of each type is 20.

| AT+QECCNUM Configure Emergency Call Numbers |  |
| --- | --- |
| Test Command AT+QECCNUM=? | Response +QECCNUM: (0-2) OK |
| Write Command AT+QECCNUM=<mode>,<type>[,<ecc | Response If <mode> is equal to 0, query the ECC numbers. In this |

| num1>[,<eccnum2>,…[,<eccnumN>]] ] | cases, <eccnumN> should be omitted, and the response is: +QECCNUM: <type>,<eccnum1>,<eccnum2>[…] OK If <mode> is not equal to 0: <mode>=1 is used to add the ECC number; <mode>=2 is used to delete the ECC number. In this case, at least one ECC number <eccnumN> should be inputted, and the response is: OK ERROR |
| --- | --- |
| Read Command AT+QECCNUM? | Response +QECCNUM: 0,<eccnum1>,<eccnum2>[…] +QECCNUM: 1,<eccnum1>,<eccnum2>[…] OK |
| Maximum Response Time | 300ms |

Parameter

<mode>	ECC number operation mode

0	Query ECC numbers

- Add ECC numbers

- Delete ECC numbers

<type>	ECC number type

0	ECC numbers without (U)SIM

1	ECC numbers with (U)SIM

<eccnum>	String type. ECC numbers ( e.g. 110, 119)

Example

AT+QECCNUM=?

+QECCNUM: (0-2)

//Query the supported ECC number operation mode

OK

AT+QECCNUM?

//Query the ECC numbers with or without (U)SIM

+QECCNUM: 0,"911","112","00","08","110","999","118","119"

+QECCNUM: 1,"911","112"

OK

AT+QECCNUM=0,1

+QECCNUM: 1,"911","112"

//Query the ECC numbers with (U)SIM

OK

AT+QECCNUM=1,1,"110", "234"	//Add "110" and "234" into the type of ECC numbers with (U)SIM

OK

AT+QECCNUM=0,1	//Query the ECC numbers with (U)SIM

+QECCNUM: 1, "911","112","110","234"

OK

AT+QECCNUM=2,1,"110" OK

AT+QECCNUM=0,1

+QECCNUM: 1, "911","112","234"

//Delete "110" from the type of ECC numbers with (U)SIM

//Query the ECC numbers with (U)SIM

OK

## AT+QHUP	Hang up Call with a Specific Release Cause

The command can terminate a call or calls (including both voice call and data call) with a specific 3GPP TS 24.008 release cause specified by the host.

| AT+QHUP Hang up Call with a Specific Release Cause |  |
| --- | --- |
| Test Command AT+QHUP=? | Response OK |
| Write Command AT+QHUP=<cause>[,<idx>] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 90s, determined by network. |

Parameter

<cause>	Release cause. 3GPP TS 24.008 release cause to be indicated to the network.

1	Release cause "unassigned (unallocated) number"

- Release cause "normal call clearing"

- Release cause "user busy"

- Release cause "no user responding"

21	Release cause "call rejected"

27	Release cause "destination out of order"

31	Release cause "normal, unspecified"

88	Release cause "incompatible destination"

<idx>		Call identification number is an optional index in the list of current calls indicated by AT+CLCC. AT+QHUP will terminate the call identified by the given call number. The default call number 0 is not assigned to any call, but signifies all calls.

0	Terminate all known calls. However, if circuit switches data calls and voice calls at the same time, this command only terminates the CSD calls.

1…7	Terminate the specific call with identification number.

Example

AT+QHUP=?

OK ATD10010; OK ATD10086; OK AT+CLCC

//Test Command

//Dial 10010

//Dial 10086

//Query the status of calls

+CLCC: 1,0,1,0,0,"10010",129

+CLCC: 2,0,0,0,0,"10086",129

OK

AT+QHUP=17,1 OK

AT+CLCC

//Terminate the call whose ID is 1. Disconnect cause is "user busy"

//Query the status of calls

+CLCC: 1,0,0,0,0,"10086",129

OK

AT+QHUP=16 OK

AT+CLCC

OK

//Terminate all existed calls. Disconnect cause is "normal call clearing"

# Phonebook	Commands

## AT+CNUM	Subscriber Number

The command can get the subscribers own number(s) from the (U)SIM.

| AT+CNUM Subscriber Number |  |
| --- | --- |
| Test Command AT+CNUM=? | Response OK |
| Execution Command AT+CNUM | Response [+CNUM: [<alpha>],<number>,<type>] [+CNUM: [<alpha>],<number>,<type>] OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP 27.007 |  |

Parameter

<alpha>	Optional alphanumeric string associated with  <number>. The used character set should be the one selected with AT+CSCS command

<number>	String type phone number of format specified by <type>

<type>	Type of address of octet in integer format (Refer to 3GPP TS 24.008 subclause

10.5.4.7 for details ). Usually, it has three kinds of values:

129	Unknown type

145	International type (contains the character “+”)

161	National type

## AT+CPBF   Find Phonebook Entries

The command can search the phonebook entries starting with the given <findtext> string from the current phonebook memory storage selected with AT+CPBS, and return all found entries sorted in alphanumeric order.

| AT+CPBF Find Phonebook Entries |  |
| --- | --- |
| Test Command AT+CPBF=? | Response +CPBF: <nlength>,<tlength> OK |
| Write Command AT+CPBF=<findtext> | Response [+CPBF: <index>,<number>,<type>,<text>] […] OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | Depends on the storage of phonebook entries. |
| Reference 3GPP 27.007 |  |

Parameter

<nlength>	Integer type. Indicates the maximum length of field <number>

<tlength>	Integer type. Indicates the maximum length of field <text>

<findtext>	String type field of maximum length <tlength> in current TE character set specified by

#### AT+CSCS.

<index>	Integer type values in the range of location numbers of phone book memory.

<type>	Type of address of octet in integer format (refer to 3GPP TS 24.008 subclause

10.5.4.7 for details). Usually, it has three kinds of values:

129	Unknown type

145	International type (contains the character “+”)

161	National type

<text>	String type field of maximum length <tlength> in current TE character set specified by

#### AT+CSCS.

## AT+CPBR	Read Phonebook Entries

The command can return phonebook entries in location number range <index1>... <index2> from the current phonebook memory storage selected with  AT+CPBS. If <index2> is left out, only location

<index1> is returned.

| AT+CPBR Read Phonebook Entries |  |
| --- | --- |
| Test Command AT+CPBR=? | Response +CPBR: (list of supported <index>s),<nlength>,<tlength> OK |
| Write Command AT+CPBR=<index1>[,<index2>] | Response +CPBR: <index1>,<number>,<type>,<text> [+CPBR: <index2>,<number>,<type>,<text> […]] OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | Depends on the storage of phonebook entries. |
| Reference 3GPP 27.007 |  |

Parameter

<index>	Integer type value in the range of location numbers of phone book memory.

<nlength>	Integer type. Indicates the maximum length of field <number>

<tlength>	Integer type. Indicates the maximum length of field <text>

<index1>	The first phone book record to read

<index2>	The last phonebook record to read

<type>	Type of address of octet in integer format (refer to 3GPP TS 24.008 subclause

10.5.4.7 for details ). Usually, it has three kinds of values:

129	Unknown type

145	International type(contains the character “+”)

161	National type

<text>	String type field of maximum length <tlength> in current TE character set specified by

#### AT+CSCS.

## AT+CPBS   Select Phonebook Memory Storage

The command selects phonebook memory storage, which is used by other phonebook commands. The Read Command returns currently selected memory, the number of used locations and the total number of locations in the memory when supported by manufacturer. The Test Command returns  supported storages as compound value.

| AT+CPBS Select Phonebook Memory Storage |  |
| --- | --- |
| Test Command AT+CPBS=? | Response +CPBS: (list of supported <storage>s) OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Read Command AT+CPBS? | Response +CPBS: <storage>,<used>,<total> OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Write Command AT+CPBS=<storage> | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP 27.007 |  |

Parameter

<storage>	“SM”	(U)SIM phonebook

“DC”	ME dialed calls list (AT+CPBW may not be applicable to this storage)

“FD”	(U)SIM fix dialing-phone book (AT+CPBW operation need the authority of PIN2) “LD”	(U)SIM last-dialing-phone book (AT+CPBW may not be applicable to this storage) “MC”	ME missed (unanswered) calls list (AT+CPBW may not be applicable to this

storage)

“ME”	Mobile equipment phonebook

“RC”	ME received calls list (AT+CPBW may not be applicable to this storage)

“EN”	(U)SIM (or ME) emergency number (AT+CPBW may not be applicable to this storage)

“ON”	(U)SIM own numbers (MSISDNs) list

<used>	Integer type. Indicates the total number of used locations in selected memory

<total>	Integer type. Indicates the total number of locations in selected memory

## AT+CPBW	Write Phonebook Entry

The command writes phonebook entry in location number <index> in the current phonebook memory storage selected with AT+CPBS. It can also delete a phonebook entry in location number <index>.

| AT+CPBW Write Phonebook Entry |  |
| --- | --- |
| Test Command AT+CPBW=? | Response +CPBW: (The range of supported <index>s), <nlength>, (list of supported <type>s), <tlength> OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Write Command AT+CPBW=[<index>][,<number>[,<ty pe>[,<text>]]] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP 27.007 |  |

Parameter

<index>	Integer type value in the range of location numbers of phone book memory. If <index> is not given, the first free entry will be used. If <index> is given as the only parameter, the phonebook entry specified by <location> is deleted.

<nlength>	Integer type. Indicates the maximum length of field <number>.

<tlength>	Integer type. Indicates the maximum length of field <text>.

<type>	Type of address of octet in integer format (refer to 3GPP TS 24.008 subclause

10.5.4.7 for details). Usually, it has three kinds of values:

129	Unknown type

145	International type (contains the character “+”)

161	National type

<text>	String type field of maximum length <tlength> in current TE character set specified by

#### AT+CSCS.

Example

AT+CSCS=“GSM” OK

AT+CPBW=10,“15021012496”,129,“QUECTEL”

OK	//Make a new phonebook entry at location 10

AT+CPBW=10	//Delete the entry at location 10

OK AT+CPBR=10 OK

# Short	Message	Service	Commands

## AT+CSMS	Select Message Service

The command selects messaging service <service> and returns the types of messages supported by the ME.

| AT+CSMS Select Message Service |  |
| --- | --- |
| Test Command AT+CSMS=? | Response +CSMS: (list of supported <service>s) OK |
| Read Command AT+CSMS? | Response +CSMS: <service>,<mt>,<mo>,<bm> OK |
| Write Command AT+CSMS=<service> | Response +CSMS: <mt>,<mo>,<bm> OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<service>	Type of message service

0	3GPP TS 23.040 and 3GPP TS 23.041 (the syntax of SMS AT commands is compatible with 3GPP TS 27.005 Phase 2 version 4.7.0; Phase 2+ features which do not require new command syntax may be supported, e.g. correct routing of messages with new Phase 2+ data coding schemes).

1	3GPP TS 23.040 and 3GPP TS 23.041 (the syntax of SMS AT commands is compatible with 3GPP TS 27.005 Phase 2+ version; the requirement of

<service>	setting	1	is	mentioned	under	corresponding	command descriptions).

<mt>	Mobile terminated messages

0	Type not supported

1	Type supported

<mo>	Mobile originated messages

0	Type not supported

1	Type supported

<bm>	Broadcast type messages

| 0 | Type not supported |  |
| --- | --- | --- |
| 1 | Type supported |  |
| Example |  |  |
| AT+CSMS=? +CSMS: (0,1) |  | //Test command |
| OK AT+CSMS=1 +CSMS: 1,1,1 |  | //Set type of message service as 1 |
| OK AT+CSMS? +CSMS: 1,1,1,1 |  | //Read command |
| OK |  |  |
| 9.2. AT+CMGF | Message Format |  |

The command specifies the input and output format of the short messages. <mode> indicates the format of messages used with Test, Read, Write and Execution Commands and unsolicited result codes resulting from received messages.

The format of messages can be either PDU mode (entire TP data units used) or text mode (headers and body of the messages given as separate parameters). Text mode uses the value of parameter <chset> specified by AT+CSCS command to inform the character set to be used in the message body in the TA-TE interface.

| AT+CMGF Message Format |  |
| --- | --- |
| Test Command AT+CMGF=? | Response +CMGF: (list of supported <mode>s) |

|  | OK |
| --- | --- |
| Read Command AT+CMGF? | Response +CMGF: <mode> OK |
| Write Command AT+CMGF[=<mode>] | Response TA sets parameter to denote which kind of I/O format of messages is used. OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<mode>	0	PDU mode

1	Text mode

## AT+CSCA   Service Center Address

The Write Command updates the SMSC address when mobile originated SMS are transmitted. In text mode, the setting is used by Write Command. In PDU mode, setting is used by the same command, but only when the length of the SMSC address is coded into the <pdu> parameter which equals to zero.

| AT+CSCA Service Center Address |  |
| --- | --- |
| Test Command AT+CSCA=? | Response OK |
| Read Command AT+CSCA? | Response +CSCA: <sca>,<tosca> OK |
| Write Command AT+CSCA=<sca>[,<tosca>] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<sca>	Service center address. 3GPP TS 24.011 RP SC address Address-Value field in string format; BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <tosca>.

<tosca>	Type of service center address. 3GPP TS 24.011 RP SC address Type-of-Address octet in integer format (default refer to <toda>).

Example

AT+CSCA=“+8613800210500”,145

OK AT+CSCA?

+CSCA: “+8613800210500”,145

//Set SMS service center address

//Query SMS service center address

OK

## AT+CPMS	Preferred Message Storage

The command selects the memory storages <mem1>, <mem2> and <mem3> to be used for reading, writing, etc.

| AT+CPMS Preferred Message Storage |  |
| --- | --- |
| Test Command AT+CPMS=? | Response +CPMS: (list of supported <mem1>s),(list of supported <mem2>s),(list of supported <mem3>s) OK |
| Read Command AT+CPMS? | Response +CPMS: <mem1>,<used1>,<total1>,<mem2>,<used2>,<total2>,<m em3>,<used3>,<total3> OK |
| Write Command AT+CPMS=<mem1>[,<mem2>[,<mem 3>]] | Response TA selects memory storages <mem1>, <mem2> and <mem3> to be used for reading, writing, etc. +CPMS: <used1>,<total1>,<used2>,<total2>,<used3>,<total3> OK |

|  | If there is any error related to ME functionality: +CMS ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<mem1>	Messages to be read and deleted from this memory storage “SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

<mem2>	Messages will be written and sent to this memory storage “SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

<mem3>	Received messages will be placed in this memory storage if routing to PC is not set (AT+CNMI)

“SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

<usedx>	Integer type. Number of current messages in <memx>

<totalx>	Integer type. Total number of messages which can be stored in <memx>

Example

AT+CPMS?

//Query the current SMS message storage

+CPMS: “ME”,0,255,“ME”,0,255,“ME”,0,255

OK

AT+CPMS=“SM”,“SM”,“SM”

+CPMS: 0,50,0,50,0,50

//Set SMS message storage as “SM”

OK

AT+CPMS?	//Query the current SMS message storage

+CPMS: “SM”,0,50,“SM”,0,50,“SM”,0,50

OK

## AT+CMGD	Delete Message

The command deletes short messages from the preferred message storage <mem1> location <index>. If

<delflag> is presented and not set to 0, then the ME shall ignore <index> and follow the rules of

<delflag> shown as below.

| AT+CMGD Delete Message |  |
| --- | --- |
| Test Command AT+CMGD=? | Response +CMGD: (list of supported <index>s),(list of supported <delflag>s) OK |
| Write Command AT+CMGD=<index>[,<delflag>] | Response TA deletes message from preferred message storage <mem1> location <index>. OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms. Note: Operation of <delflag> depends on the storage of deleted messages. |
| Reference 3GPP TS 27.005 |  |

Parameter

<index>	Integer type value in the range of location numbers supported by the associated memory.

<delflag>	0	Delete the message specified in <index>

- Delete all read messages from <mem1> storage

- Delete all read messages from <mem1> storage and sent mobile originated messages

- Delete  all  read  messages  from  <mem1>  storage,  sent  and  unsent  mobile originated messages

- Delete all messages from <mem1> storage

<mem1>	Messages to be read and deleted from this memory storage “SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

Example

AT+CMGD=1

OK AT+CMGD=1,4 OK

//Delete the message specified in <index>=1

//Delete all messages from <mem1> storage

## AT+CMGL   List Messages

The  Read  Command  returns  messages  with  status  value  <stat> from  preferred  message  storage

<mem1> to the TE. If the status of the message is “REC UNREAD”, the status in the storage changes to “REC READ”. When executing command AT+CMGL without status value <stat>, it will report the list of SMS with “REC UNREAD” status.

| AT+CMGL List Messages |  |
| --- | --- |
| Test Command AT+CMGL=? | Response +CMGL: (list of supported <stat>s) OK |
| Write Command AT+CMGL[=<stat>] | Response If in text mode (AT+CMGF=1) and the command is executed successfully: For SMS-SUBMITs and/or SMS-DELIVERs: +CMGL: <index>,<stat>,<oa/da>,[<alpha>],[<scts>][,<tooa/toda>,< length>]<CR><LF><data>[<CR><LF> +CMGL: <index>,<stat>,<da/oa>,[<alpha>],[<scts>][,<tooa/toda>,< length>]<CR><LF><data>[...]] For SMS-STATUS-REPORTs: +CMGL: <index>,<stat>,<fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>,<st >[<CR><LF> +CMGL: <index>,<stat>,<fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>,<st >[...]] For SMS-COMMANDs: +CMGL: <index>,<stat>,<fo>,<ct>[<CR><LF> +CMGL: <index>,<stat>,<fo>,<ct>[...]] |

|  | For CBM storage: +CMGL:<index>,<stat>,<sn>,<mid>,<page>,<pages><CR ><LF><data>[<CR><LF> +CMGL: <index>,<stat>,<sn>,<mid>,<page>,<pages><CR><LF><d ata>[...]] OK If in PDU mode (AT+CMGF=0) and the command is executed successfully: +CMGL:<index>,<stat>,[<alpha>],<length><CR><LF><pd u><CR><LF> +CMGL: <index>,<stat>,[alpha],<length><CR><LF><pdu>[...]] OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| --- | --- |
| Execution Command AT+CMGL | Response List all messages with “REC UNREAD” status from message storage <mem1>, and then the status in the storage changes to “REC READ”. |
| Maximum Response Time | 300ms. Note: Operation of <stat> depends on the storage of listed messages. |
| Reference 3GPP TS 27.005 |  |

| Parameter |  |  |
| --- | --- | --- |
| <stat> | In text mode: |  |
|  | “REC UNREAD” | Received unread messages |
|  | “REC READ” | Received read messages |
|  | “STO UNSENT” | Stored unsent messages |
|  | “STO SENT” | Stored sent messages |
|  | “ALL” | All messages |
|  | In PDU mode: |  |
|  | 0 | Received unread messages |
|  | 1 | Received read messages |
|  | 2 | Stored unsent messages |
|  | 3 | Stored sent messages |

4	All messages

<index>	Integer type value in the range of location numbers supported by the associated memory

<da>	Destination Address. 3GPP TS 23.040 TP-Destination-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <toda>.

<oa>	Originating address. 3GPP TS 23.040 TP-Originating-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <tooa>.

<alpha>	String type alphanumeric representation of <da> or <oa> corresponding to the entry found in MT phonebook. Implementation of this feature is manufacturer specified. The used character set should be the one selected with AT+CSCS command (see definition of this command in 3GPP TS 27.007).

<scts>	Service center time stamp. 3GPP TS 23.040 TP-Service-Centre-Time-Stamp in time-string format (refer to <dt>).

<toda>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format.

<tooa>		Type of originating address. 3GPP TS 24.011 TP-Originating-Address Type-of-Address  octet in integer format (default refer to <toda>).

<length>		Message length. Integer type. Indicating in the text mode (AT+CMGF=1) the length of the message body <data> (or <cdata>) in characters, or in PDU mode (AT+CMGF=0) the length of the actual TP data unit in octets (i.e. the RP layer SMSC address octets are not counted in the length).

<data>	In the case of SMS: 3GPP TS 23.040 TP-User-Data in text mode responses; format:

- If <dcs>, indicates that 3GPP TS 23.038 GSM 7 bit default alphabet is used and

<fo> indicates that 3GPP TS 23.040 TP-User-Data-Header-Indication is not set.

- If TE character set other than “HEX” (refer to AT+CSCS command in 3GPP TS 27.007): ME/TA converts GSM alphabet into current TE character set according to rules of Annex A in 3GPP TS 27.007.

- If TE character set is “HEX”: ME/TA converts each 7-bit character of GSM 7 bit default alphabet into two IRA character long hexadecimal number (e.g. character  (GSM 7 bit default alphabet 23) is presented as 17 (IRA 49 and 55)).

- If <dcs>, indicates that 8-bit or UCS2 data coding scheme is used, or <fo> indicates that 3GPP TS 23.040 TP-User-Data-Header-Indication is set: ME/TA converts each 8-bit octet into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65)).

In  the  case  of  CBS:  3GPP TS  23.041  CBM  Content  of  Message  in  text  mode responses; format:

- If <dcs>, indicates that 3GPP TS 23.038 GSM 7 bit default alphabet is used:

- If TE character set other than “HEX” (refer to AT+CSCS command in 3GPP TS27.007): ME/TA converts GSM alphabet into current TE character set according

to rules of Annex A in 3GPP TS 27.007.

- If TE character set is “HEX”: ME/TA converts each 7-bit character of the GSM 7 bit default alphabet into two IRA character long hexadecimal number.

- If <dcs>, indicates that 8-bit or UCS2 data coding scheme is used: ME/TA converts each 8-bit octet into two IRA character long hexadecimal number.

<pdu>	In the case of SMS: 3GPP TS 24.011 SC address followed by 3GPP TS 23.040 TPDU in hexadecimal format: ME/TA converts each octet of TP data unit into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65)) 3GPP TS 27.007.

<mem1>	Messages to be read and deleted from this memory storage “SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

Example

AT+CMGF=1

OK AT+CMGL=“ALL”

+CMGL: 1,“STO UNSENT”,“”,,

//Set SMS message format as text mode

//List all messages from message storage

<This is a test from Quectel>

+CMGL: 2,“STO UNSENT”,“”,,

<This is a test from Quectel> OK

## AT+CMGR	Read Message

The Read Command returns SMS message with location value <index> from message storage <mem1>

to the TE. If status of the message is “REC UNREAD”, status in the storage changes to “REC READ”.

| AT+CMGR Read Message |  |
| --- | --- |
| Test Command AT+CMGR=? | Response OK |
| Write Command AT+CMGR=<index> | Response TA returns SMS message with location value <index> from message storage <mem1> to the TE. If status of the message is “REC UNREAD”, status in the storage changes to “REC READ”. If in text mode (AT+CMGF=1) and the command is executed successfully: |

|  | For SMS-DELIVER: +CMGR: <stat>,<oa>,[<alpha>],<scts>[,<tooa>,<fo>,<pid>,<dcs>,< sca>,<tosca>,<length>]<CR><LF><data> OK For SMS-SUBMIT: +CMGR: <stat>,<da>,[<alpha>][,<toda>,<fo>,<pid>,<dcs>,[<vp>],< sca>,<tosca>,<length>]<CR><LF><data> OK For SMS-STATUS-REPORTs: +CMGR: <stat>,<fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>,<st> OK For SMS-COMMANDs: +CMGR: <stat>,<fo>,<ct>[,<pid>,[<mn>],[<da>],[<toda>],<length>< CR><LF><cdata>] OK For CBM storage: +CMGR: <stat>,<sn>,<mid>,<dcs>,<page>,<pages><CR><LF><dat a> OK If in PDU mode (AT+CMGF=0) and command is executed successfully: +CMGR: <stat>,[<alpha>],<length><CR><LF><pdu> OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| --- | --- |
| Maximum Response Time | Depends on the length of message content. |
| Reference 3GPP TS 27.005 |  |

Parameter

<index>		Integer type value in the range of location numbers supported by the associated memory

<stat>	In text mode

“REC UNREAD”	Received unread messages “REC READ”	Received read messages “STO UNSENT”	Stored unsent messages “STO SENT”	Stored sent messages

“ALL”	All messages In PDU mode

0	Received unread messages

- Received read messages

- Stored unsent messages

- Stored sent messages

- All messages

<alpha>	String type alphanumeric representation of <da> or <oa> corresponding to the entry found in MT phonebook. Implementation of this feature is manufacturer specified. The used character set should be the one selected with AT+CSCS command (see definition of this command in 3GPP TS 27.007).

<da>		Destination address. 3GPP TS 23.040 TP-Destination-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <toda>.

<oa>	Originating address. 3GPP TS 23.040 TP-Originating-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <tooa>.

<scts>	Service center time stamp. 3GPP TS 23.040 TP-Service-Centre-Time-Stamp in time-string format (refer to <dt>).

<fo>		First octet. Depending on the command or result code: First octet of 3GPP TS 23.040 SMS-DELIVER,     SMS-SUBMIT     (default     17),     SMS-STATUS-REPORT,     or

SMS-COMMAND  in  integer  format.  If  a  valid  value  has  been  entered  once,  the parameter can be omitted.

<pid>	Protocol identifier. 3GPP TS 23.040 TP-Protocol-Identifier in integer format (default 0).

<dcs>	Data  coding  scheme.  Depending  on  the  command  or  result  code:  3GPP TS  23.038 SMS Data Coding Scheme (default 0), or Cell Broadcast Data Coding Scheme in integer format.

<vp>	Validity period. Depending on SMS-SUBMIT <fo> setting: 3GPP TS 23.040 TP-Validity-Period either in integer format or in time-string format (refer to <dt>).

<mn>	Message number. 3GPP TS 23.040 TP-Message-Number in integer format.

<mr>	Message reference. 3GPP TS 23.040 TP-Message-Reference in integer format.

<ra>	Recipient  address.  3GPP  TS  23.040  TP-Recipient-Address  Address-Value  field  in

string format. BCD numbers (or GSM default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command). The type of address is given by <tora>.

<tora>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format (default refer <toda>).

<toda>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format.

<tooa>	Type of originating address.3GPP TS 24.011 TP-Originating-Address Type-of-Address octet in integer format (default refer to <toda>).

<sca>	Service center address. 3GPP TS 24.011 RP SC address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <tosca>.

<tosca>	Type of service center address. 3GPP TS 24.011 RP SC address Type-of-Address octet in integer format (default refer to <toda>).

<length>	Message length. Integer type. Indicating in the text mode (AT+CMGF=1) the length of the message body <data> (or <cdata>) in characters, or in PDU mode (AT+CMGF=0) the length of the actual TP data unit in octets (i.e. the RP layer SMSC address octets are not counted in the length).

<data>	The text of short message. Please refer Chapter 14.8 for details.

<pdu>	In the case of SMS: 3GPP TS 24.011 SC address followed by 3GPP TS 23.040 TPDU in hexadecimal format: ME/TA converts each octet of TP data unit into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65)).

<prt>	Priority

0     Normal

- Interactive

- Urgent

- Emergency

<fmt>	Format

0     GSM 7 bit

1     ASCII

6     UNICODE

<prv>	Privacy

0     Normal

- Restricted

- Confidential

- Secret

<lang>	Language

0     Unspecified

- English

- French

- Spanish

- Japanese

- Korean

- Chinese

- Hebrew

<type>	0	Normal

- CPT

- Voice Mail

- SMS Report

<mem1>	Messages to be read and deleted from this memory storage “SM”	(U)SIM message storage

“ME”	Mobile equipment message storage “MT”	Same as “ME” storage

Example

+CMTI: “SM”,3

//Indicates that new message has been received and saved

to <index>=3 of “SM”

AT+CSDH=1

OK

AT+CMGR=3	//Read message

+CMGR:	“REC	UNREAD”,“+8615021012496”,,“13/12/13,15:06:37+32”,145,4,0,0,“+861380021050 0”,145,27

<This is a test from Quectel>

OK

## AT+CMGS   Send Message

The Write Command sends a short message from TE to network (SMS-SUBMIT). After invoking the write command, wait for the prompt “>” and then start to write the message. After that, enter <CTRL-Z> to indicate the ending of PDU and begin to send the message. Sending can be cancelled by giving <ESC> character. Abortion is acknowledged with “OK”, though the message will not be sent. The message reference <mr> is returned to the TE on successful message delivery. The value can be used to identify message upon unsolicited delivery status report result code.

| AT+CMGS Send Message |  |
| --- | --- |
| Test Command AT+CMGS=? | Response OK |
| Write Command 1) If text mode (AT+CMGF=1): AT+CMGS=<da>[,<toda>]<CR> text is entered | Response TA sends message from TE to the network (SMS-SUBMIT). Message reference value <mr> is returned to the TE on successful message delivery. Optionally (when AT+CSMS |

| <Ctrl+Z/ESC> ESC quits without sending 2) If PDU mode (AT+CMGF=0): AT+CMGS=<length><CR> PDU is given <Ctrl+Z/ESC> | <service> value is 1 and network supports) <scts> is returned. Values can be used to identify message upon unsolicited delivery status report result code. If in text mode (AT+CMGF=1) and sent successfully: +CMGS: <mr> OK If in PDU mode (AT+CMGF=0) and sent successfully: +CMGS: <mr> OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| --- | --- |
| Maximum Response Time | 120s, determined by network. |
| Reference 3GPP TS 27.005 |  |

Parameter

<da>	Destination address. 3GPP TS 23.040 TP-Destination-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address is given by <toda>.

<toda>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format.

<length>	Message length. Integer type. Indicating in the text mode (AT+CMGF=1) the length of the message body <data> (or <cdata>) in characters, or in PDU mode (AT+CMGF=0), the length of the actual TP data unit in octets (i.e. the RP layer SMSC address octets are not counted in the length).

<mr>	Message reference. 3GPP TS 23.040 TP-Message-Reference in integer format.

Example

AT+CMGF=1	//Set SMS message format as text mode

#### OK

AT+CSCS=“GSM”	//Set character set as GSM which is used by the TE

#### OK AT+CMGS=“15021012496”

> <This is a test from Quectel>	//Enter in text. Use <CTRL+Z> to send message, or

<ESC> quits without sending

#### +CMGS: 247

OK

## AT+CMMS	More Messages to Send

The command controls the continuity of the SMS relay protocol link. If the feature is enabled (and supported by the currently used network) multiple messages can be sent faster as the link is kept open.

| AT+CMMS More Messages to Send |  |
| --- | --- |
| Test Command AT+CMMS=? | Response +CMMS: (list of supported <n>s) OK |
| Read Command AT+CMMS? | Response +CMMS: <n> OK |
| Write Command AT+CMMS=<n> | Response OK ERROR If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 120s, determined by network. |
| Reference 3GPP TS 27.005 |  |

Parameter

| <n> | 0 | Feature disabled |
| --- | --- | --- |
|  | 1 | Keep enabled until the time between the response of the latest message send command |
|  |  | (AT+CMGS, AT+CMSS, etc.) and the next send command exceeds 1-5 seconds (the |
|  |  | exact value is up to ME implementation), and then ME shall close the link and TA |
|  |  | switches <n> back to 0 automatically |
|  | 2 | Feature enabled (if the time between the response of the latest message send command |
|  |  | and the next send command exceeds 1-5 seconds (the exact value is up to ME |
|  |  | implementation), ME shall close the link but TA will not switch <n> back to 0 |
|  |  | automatically) |

After the use of Read Command, a delay of 5-10 seconds is required before issuing the Write Command. Otherwise the +CMS ERROR: 500 may appear.

NOTE

## AT+CMGW   Write Message to Memory

The Write and Execution Commands store short messages from TE to memory storage <mem2>, and then the memory location <index> of the stored message is returned. Message status will be set to “stored unsent” by default, but parameter <stat> also allows other status values to be given.

The syntax of input text is the same as the one specified in AT+CMGS Write Command.

| AT+CMGW Write Message to Memory |  |
| --- | --- |
| Test Command AT+CMGW=? | Response OK |
| Write Command - If text mode (AT+CMGF=1): AT+CMGW=<oa/da>[,<tooa/toda>[,<st at>]]<CR> text is entered <Ctrl+Z/ESC> <ESC> quits without sending - If PDU mode (AT+CMGF=0): AT+CMGW=<length>[,<stat>]<CR> PDU is given <Ctrl+Z/ESC> | Response TA transmits SMS message (either SMS-DELIVER or SMS-SUBMIT) from TE to memory storage <mem2>, and then the memory location <index> of the stored message is returned. By default the message status will be set to „stored unsent‟, but parameter <stat> also allows other status values to be given. If writing is successful: +CMGW: <index> OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<da>	Destination address. 3GPP TS 23.040 TP-Destination-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in

3GPP TS 27.007). The type of address is given by <toda>.

<oa>	Originating address. 3GPP TS 23.040 TP-Originating-Address Address-Value field in string format. BCD numbers (or GSM 7 bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to AT+CSCS command in 3GPP TS 27.007). The type of address given by <tooa>.

<tooa>	Type of originating address. 3GPP TS 24.011 TP-Originating-Address Type-of-Address octet in integer format (default refer to <toda>).

<stat>	PDU mode	Text mode	Explanation

0	“REC UNREAD”	Received unread messages

- “REC READ”	Received read messages

- “STO UNSENT”	Stored unsent messages

- “STO SENT”	Stored sent messages

- “ALL”	All messages

<toda>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format.

<length>	Message length. Integer type, indicating in the text mode (AT+CMGF=1) the length of the message body <data> (or <cdata>) in characters, or in PDU mode (AT+CMGF=0), the length of the actual TP data unit in octets (i.e. the RP layer SMSC address octets are not counted in the length).

<pdu>	In the case of SMS: 3GPP TS 24.011 SC address followed by 3GPP TS 23.04TPDU in hexadecimal format: ME/TA converts each octet of TP data unit into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65)).

<index>	Index of message in selected storage <mem2>.

Example

AT+CMGF=1	//Set SMS message format as text mode

#### OK

AT+CSCS=“GSM”	//Set character set as GSM which is used by the TE

#### OK AT+CMGW=“15021012496”

> <This is a test from Quectel>	//Enter  in  text.  Use  <CTRL+Z>  to  write  message  or

<ESC> to quit without sending.

#### +CMGW: 4

#### OK

AT+CMGF=0	//Set SMS message format as PDU mode

#### OK AT+CMGW=18

#### > 0051FF00000008000A0500030002016D4B8BD5

#### +CMGW: 5

#### OK

## AT+CMSS   Send Message from Storage

The Write Command sends messages with location value <index> from message storage <mem2> to the network. If a new recipient address <da> is given for SMS-SUBMIT, it shall be used instead of the one stored with the message.

| AT+CMSS Send Message from Storage |  |
| --- | --- |
| Test Command AT+CMSS=? | Response OK |
| Write Command AT+CMSS=<index>[,<da>[,<toda>]] | Response TA sends message with location value <index> from message storage <mem2> to the network (SMS-SUBMIT). If new recipient address <da> is given, it shall be used instead of the one stored with the message. Reference value <mr> is returned to the TE on successful message delivery. Values can be used to identify message upon unsolicited delivery status report result code. If in text mode (AT+CMGF=1) and sent successfully: +CMSS: <mr>[,<scts>] OK If in PDU mode (AT+CMGF=0) and sent successfully: +CMSS: <mr> [,<ackpdu>] OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 120s, determined by network. |
| Reference 3GPP TS 27.005 |  |

Parameter

<index>	Integer type value in the range of location numbers supported by the associated memory.

<da>	Destination Address. 3GPP TS 23.040 TP-Destination-Address Address-Value field in string  format.  BCD  numbers  (or  GSM  7  bit  default  alphabet  characters)  are

converted  to  characters  of  the  currently  selected  TE  character  set  (refer  to

AT+CSCS command in 3GPP TS 27.007). The type of address is given by <toda>.

<toda>	Type of recipient address. 3GPP TS 24.011 TP-Recipient-Address Type-of-Address octet in integer format.

<mr>	Message reference. 3GPP TS 23.040 TP-Message-Reference in integer format.

<scts>	Service center time stamp. 3GPP TS 23.040 TP-Service-Centre-Time-Stamp in time-string format (refer to <dt>).

<ackpdu>	The format is the same as <pdu> in case of SMS, but without 3GPP TS 24.011 SC address field and the parameter shall be bounded by double quote characters like a normal string type parameter.

Example

AT+CMGF=1

OK AT+CSCS=“GSM” OK

AT+CMGW=“15021012496”

> Hello

//Set SMS message format as text mode

//Set character set as GSM which is used by the TE

//Enter  in  text.  Use  <CTRL+Z>  to  send  message  or

<ESC> to quit without sending.

+CMGW: 4

OK

AT+CMSS=4

+CMSS: 54

//Send the message of index 4 from memory storage.

OK

## AT+CNMA   New Message Acknowledgement to UE/TE

The Write and Execution Commands confirm successful receipt of a new message (SMS-DELIVER or SMS-STATUS-REPORT) routed directly to the TE. If the UE does not receive acknowledgement within required time (network timeout), it sends an RP-ERROR message to the network. The UE will automatically disable routing to the TE by setting both <mt> and <ds> values of AT+CNMI to 0.

| AT+CNMA New Message Acknowledgement to UE/TE |  |
| --- | --- |
| Test Command AT+CNMA=? | Response +CNMA: (list of supported <n>s) OK |
| Execution Command AT+CNMA | Response OK |

|  | ERROR If there is any error related to ME functionality: +CMS ERROR: <err> |
| --- | --- |
| Write Command AT+CNMA=<n> | Response OK ERROR If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<n>	Parameter required only for PDU mode

0	Command operates similarly as in text mode

- Send positive (RP-ACK) acknowledgement to the network. Accepted only in PDU mode

- Send negative (RP-ERROR) acknowledgement to the network. Accepted only in PDU mode

The Execution and Write Commands shall only be used when AT+CSMS parameter <service> equals 1 (phase 2+) and an appropriate URC has been issued by the module, i.e.:

+CMT for <mt>=2 incoming message classes 0,1,3 and none;

+CMT for <mt>=3 incoming message classes 0 and 3;

+CDS for <ds>=1.

NOTE

Example

AT+CSMS=1

OK AT+CNMI=1,2,0,0,0 OK

+CMT: “+8615021012496”,,“13/03/18,17:07:21+32”,145,4,0,0,“+8613800551500”,145,28

This is a test from Quectel.

AT+CNMA OK AT+CNMA

+CMS ERROR：340

//Short message is outputted directly when an SMS is incoming.

//Send ACK to the network

//The second time to return error. It needs ACK only once

## AT+CNMI   SMS Event Reporting Configuration

The Write Command selects the procedure on how the received new messages from the network are indicated to the TE when TE is active, e.g. DTR is at low level (ON). If TE is inactive (e.g. DTR is at high level (OFF)), message receiving should be done as specified in 3GPP TS 23.038.

| AT+CNMI SMS Event Reporting Configuration |  |
| --- | --- |
| Test Command AT+CNMI=? | Response +CNMI: (list of supported <mode>s),(list of supported <mt>s),(list of supported <bm>s),(list of supported <ds>s),(list of supported <bfr>s) OK |
| Read Command AT+CNMI? | Response +CNMI: <mode>,<mt>,<bm>,<ds>,<bfr> OK |
| Write Command AT+CNMI[=<mode>[,<mt>[,<bm>[,<ds >[,<bfr>]]]]] | Response TA selects the procedure on how the received new messages from the network are indicated to the TE when TE is active, e.g. DTR is at low level (ON). If TE is inactive (e.g. DTR is at high level (OFF)), message receiving should be done as specified in 3GPP TS 23.038. OK ERROR If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<mode>        0        Buffer unsolicited result codes in the TA. If TA result code buffer is full, indications can be buffered in some other place or the oldest indications may be discarded and replaced with the new received indications.

- Discard indication and reject new received message unsolicited result codes when TA-TE link is reserved (e.g. in on-line data mode). Otherwise forward them directly to the TE.

- Buffer unsolicited result codes in the TA when TA-TE link is reserved (e.g. in data

mode) and flush them to the TE after reservation. Otherwise forward them directly to the TE.

<mt>		The rules for storing received SMS depend on its data coding scheme (refer to 3GPPTS 23.038) and preferred memory storage (AT+CPMS) setting, and the value is:

0	No SMS-DELIVER indications are routed to the TE.

- If SMS-DELIVER is stored into ME/TA, indication of the memory location is routed to the TE by using unsolicited result code: +CMTI: <mem>,<index>

- SMS-DELIVERs (except class 2) are routed directly to the TE using unsolicited result code: +CMT: [<alpha>],<length><CR><LF><pdu> (PDU mode enabled) or   +CMT:   <oa>,[<alpha>],<scts>[,<tooa>,<fo>,<pid>,<dcs>,<sca>,<tosca>,

<length>]<CR><LF><data> (text mode enabled; about the parameters in italics, please         refer         to         AT+CSDH         command)         or         ^HCMT:

<oa>,<scts>,<lang>,<fmt>,<length>,<prt>,<prv>,<type>,<stat><CR><LF><da ta> (text mode for CDMA SMS). Class 2 messages result in indication as defined in <mt>=1.

- Class 3 SMS-DELIVERs are routed directly to TE by using unsolicited result codes defined in <mt>=2. Messages of other classes result in indication as defined in <mt>=1.

|  | 0 | No CBM indications are routed to the TE. |
| --- | --- | --- |
|  | 2 | New CBMs are routed directly to the TE using unsolicited result code: |
|  |  | +CBM: <length><CR><LF><pdu> (PDU mode); or |
|  |  | +CBM: <sn>,<mid>,<dcs>,<page>,<pages><CR><LF><data> (text mode) |
| <ds> | 0 | No SMS-STATUS-REPORTs are routed to the TE. |
|  | 1 | SMS-STATUS-REPORTs are routed to the TE using unsolicited result code: |
|  |  | +CDS: <length><CR><LF><pdu> (PDU mode) |
|  |  | +CDS: <fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>,<st> (text mode) |
|  | 2 | If SMS-STATUS-REPORT is stored into ME/TA, indication of the memory location |
|  |  | is routed to the TE using unsolicited result code: |
|  |  | +CDSI: <mem>,<index> |
| <bfr> | 0 | TA buffer of unsolicited result codes defined within this command is flushed to the |
|  |  | TE when <mode> 1...2 is entered (OK response shall be given before flushing the |
|  |  | codes). |
|  | 1 | TA buffer of unsolicited result codes defined within this command is cleared when |
|  |  | <mode> 1...2 is entered. |

<bm>		The rules for storing received CBMs depend on its data coding scheme (refer to 3GPP TS 23.038) and the setting of Select CBM Types (AT+CSCB), and the value is:

Unsolicited result code:

+CMTI: <mem>,<index>	Indicates that new message has been received

+CMT: [<alpha>],<length><CR><LF><pdu>	Short message is outputted directly

+CBM: <length><CR><LF><pdu>	Cell broadcast message is outputted directly

NOTE

Example

AT+CMGF=1

OK AT+CSCS=“GSM” OK AT+CNMI=1,2,0,1,0 OK

//Set SMS message format as text mode

//Set character set as GSM which is used by the TE

//Set SMS-DELIVERs are routed directly to the TE

+CMT: “+8615021012496”,,“13/03/18,17:07:21+32”,145,4,0,0,“+8613800551500”,145,28

This is a test from Quectel.	//Short message is outputted directly when an SMS is incoming.

## AT+CSCB	Select Cell Broadcast Message Types

The Write Command selects which types of CBMs are to be received by the ME. The command writes the parameters in NON-VOLATILE memory.

| AT+CSCB Select Cell Broadcast Message Types |  |
| --- | --- |
| Test Command AT+CSCB=? | Response It returns supported modes as a compound value. +CSCB: (list of supported <mode>s) OK |
| Read Command AT+CSCB? | Response +CSCB : <mode>,<mids>,<dcss> OK |
| Write Command AT+CSCB=<mode>[,mids>[,<dcss>]] | Response TA selects which types of CBMs are to be received by the ME. OK If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<mode>	0	Message types specified in <mids> and <dcss> are accepted

1	Message types specified in <mids> and <dcss> are not accepted

<mids>	String type. All different possible combinations of CBM message identifiers (refer to <mid>) (default is empty string), e.g. “0,1,5,320-478,922”

<dcss>	String type. All different possible combinations of CBM data coding schemes (refer to

<dcs>) (default is empty string), e.g. “0-3,5”

## AT+CSDH	Show SMS Text Mode Parameters

The Write Command controls whether detailed header information is shown in text mode result codes.

| AT+CSDH Show SMS Text Mode Parameters |  |
| --- | --- |
| Test Command AT+CSDH=? | Response +CSDH: (list of supported <show>s) OK |
| Read Command AT+CSDH? | Response +CSDH: <show> OK |
| Write Command AT+CSDH[=<show>] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<show>	0	Do not show header values defined in commands +CSCA, +CSMP (<sca>,

<tosca>, <fo>, <vp>, <pid>, <dcs>) and <length>, <toda> or <tooa> in +CMT,

+CMGL, +CMGR result codes for SMS-DELIVERs and SMS-SUBMITs in text mode

1	Show the values in result codes

Example

AT+CSDH=0 OK AT+CMGR=2

+CMGR: “STO UNSENT” ,“”,

<This is a test from Quectel>

OK AT+CSDH=1 OK AT+CMGR=2

+CMGR: “STO UNSENT”,“”,,128,17,0,0,143,“+8613800551500”,145,18

<This is a test from Quectel> OK

## AT+CSMP	Set SMS Text Mode Parameters

The command is used to set values for additional parameters needed when a short message is sent to the network or placed in a storage in text mode.

| AT+CSMP Set SMS Text Mode Parameters |  |
| --- | --- |
| Test Command AT+CSMP=? | Response OK |
| Read Command AT+CSMP? | Response +CSMP: <fo>,<vp>,<pid>,<dcs> OK |
| Write Command AT+CSMP=<fo>[,<vp>[,<pid>[,<dcs>]] ] | Response TA selects values for additional parameters needed when SM is sent to the network or placed in a storage when text mode is selected (AT+CMGF=1). It is possible to set the validity period starting from when the SM is received by the SMSC (<vp> ranges from 0 to 255) or define the absolute time of the validity period termination (<vp> is a string). OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.005 |  |

Parameter

<fo>	First octet. Depending on the command or result code: First octet of 3GPP TS 23.040 SMS-DELIVER, SMS-SUBMIT (default 17), SMS-STATUS-REPORT, SMS-COMMAND

in integer format. If a valid value has been entered once, parameter can be omitted.

<vp>	Validity   period.   Depending   on   SMS-SUBMIT   <fo>   setting:   3GPP  TS   23.040 TP-Validity-Period either in integer format or in time-string format (refer to <dt>).

<pid>	Protocol identifier. 3GPP TS 23.040 TP-Protocol-Identifier in integer format (default 0).

<dcs>	Data coding scheme. Depending on the command or result code: 3GPP TS 23.038 SMS Data Coding Scheme (default 0), or Cell Broadcast Data Coding Scheme in integer format.

## AT+QCMGS   Send Concatenated Messages

The command is used to send concatenated massages. Different from AT+CMGS, when sending a concatenated message via this command, each segment of the concatenated message must be identified by the additional parameters: <uid>, <msg_seg> and <msg_total>. When sending all segments of the message one by one, AT+QCMGS must be executed multiple times (equal to <msg_total>) for each segment. This command is only used in text mode (AT+CMGF=1).

| AT+QCMGS Send Concatenated Messages |  |
| --- | --- |
| Test Command AT+QCMGS=? | Response OK |
| Write Command If text mode (+CMGF=1): AT+QCMGS=<da>[,<toda>][,<uid>,<m sg_seg>,<msg_total>]<CR> text is entered <Ctrl+Z/ESC> | Response If in text mode (AT+CMGF=1) and sent successfully: +QCMGS: <mr> OK ERROR If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | 120s, determined by network. |

Parameter

<uid>		Message identification in the user data header (UDH). Range: 0-255. This parameter is defined and inputted by the user. All segments of a same concatenated message must have the same <uid>. Different concatenated messages should have different <uid>.

<msg_seg>	Sequence number of a concatenated message. Range: 0-7.

<msg_seg>=0 means: ignore the value and regard it as a non-concatenated message.

<msg_total>	The total number of the segments of one concatenated message. Range: 0-7.

<msg_total>=0	or	1	means:	ignore	the	value	and	regard	it	as	a non-concatenated message.

<da>	Please refer to AT+CMGS.

<toda>	Please refer to AT+CMGS.

<mr>	Please refer to AT+CMGS.

- For concatenated messages, the maximum length will be reduced by the length of the user data header (UDH). 3GPP TS 23.040 defines two kinds of UDH length: 6 bytes and 7 bytes, so the two kinds of <uid> are 8-bit (6 bytes) and 16-bit (7 bytes). AT+QCMGS uses 8-bit <uid>.

  - In the case of GSM 7 bit default alphabet data coding scheme, the maximum length of each segment of a concatenated message is (140 octets - 6 )*8/7=153 characters.

  - In the case of 16 bit UCS2 data coding scheme, the maximum length of each segment is (140-6)/2=67 characters.

  - In the case of 8-bit data coding scheme, the maximum length of each segment is 140-6=134 characters.

- <mr> Message-Reference field gives an integer representation of a reference number of the SMS-SUBMIT or SMS-COMMAND submitted to the SC by the MS, and it is used to confirm whether the SMS-DELIVER has been received from SC duplicate or not.

<uid>	The field of UDH. It is message identification of the concatenated SMS, which is different from <mr>. Each segment in a concatenated message should have the same <uid>, but

<mr> must be incremented for each segment of a concatenated message.

- AT+QCMGS does not support to send message in PDU mode (AT+CMGF=0).

NOTES

Example

AT+CMGF=1

OK AT+CSCS="GSM" OK

AT+QCMGS="15056913384",120,1,2 <CR>

//Set SMS message format as text mode

//Set character set as GSM which is used by the TE

//Input 120 for <uid>, and send the first segment of the concatenated SMS

>ABCD<Ctrl-Z>

+QCMGS: 190

OK

AT+QCMGS="15056913384",120,2,2 <CR>

>EFGH<Ctrl-Z>

+QCMGS: 191

//Send the second segment of the concatenated SMS.

OK

## AT+QCMGR   Read Concatenated Messages

The function of this command is similar to AT+CMGR, except that the message to be read is a segment of concatenated messages, and parameters <uid>, <msg_seg> and <msg_total> would be shown in the result. Several segments should be concatenated to a whole concatenated message according to these three parameters. Similar to AT+QCMGS, AT+QCMGR is only used in text mode (AT+CMGF=1).

| AT+QCMGR Read Concatenated Messages |  |
| --- | --- |
| Test Command AT+QCMGR=? | Response OK |
| Write Command AT+QCMGR=<index> | Response If in text mode (AT+CMGF=1) and command is executed successfully: For SMS-DELIVER: +QCMGR: <stat>,<oa>,[<alpha>],<scts>[,<tooa>,<fo>,<pid>,<dcs>,< sca>,<tosca>,<length>][,<uid>,<msg_seg>,<msg_total>] <CR><LF><data> OK For SMS-SUBMIT: +QCMGR: <stat>,<da>,[<alpha>][,<toda>,<fo>,<pid>,<dcs>,[<vp>],< sca>,<tosca>,<length>][,<uid>,<msg_seg>,<msg_total>] <CR><LF><data> OK For SMS-STATUS-REPORTs: +QCMGR: <stat>,<fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>,<st> OK For SMS-COMMANDs: +QCMGR: <stat>,<fo>,<ct>[,<pid>,[<mn>],[<da>],[<toda>],<length>< CR><LF><cdata>] OK Else, If there is any error related to ME functionality: +CMS ERROR: <err> |
| Maximum Response Time | Depends on the length of message content. |

Parameter

<uid>	Message identification in the user data header (UDH). Range: 0-65535 (see NOTES). All segments of a same concatenated message have same <uid>. Different concatenated messages should have different <uid>.

<msg_seg>	Sequence number of a concatenated message. Range: 1-7.

<msg_total>	The total number of the segments of one concatenated message. Range: 2-7.

Other parameters please refer to AT+CMGR

- The <uid> in AT+QCMGR is different from the <uid> in AT+QCMGS. It is possible that UE receives concatenated messages with 8-bit or 16-bit <uid>. So its maximal value is 255 with 8-bit and 65535 with 16-bit.

- If the message to be read is not a concatenated message, <uid>, <msg_seg> and <msg_total>

would not be shown in the result.

NOTES

Example

+CMTI: "SM",3

//The first message of a concatenated message comes

+CMTI: "SM",4

//The second message of a concatenated message comes

AT+QCMGR= 3

//Read the first segment of the concatenated message

+QCMGR: "REC UNREAD","+8615056913384",,"13/07/30,14:44:37+32",120,1,2

ABCD

OK

AT+QCMGR= 4

//Read the second segment of the concatenated message

+QCMGR: "REC UNREAD","+8615056913384",,"13/07/30,14:44:37+32",120,2,2

EFGH

OK

# Packet Domain Commands

## AT+CGATT   Attachment or Detachment of PS

The Write Command is used to attach the MT to, or detach the MT from the Packet Domain service. After the command has been completed, the MT remains in V.25ter command state. If the MT is already in the requested state, the command is ignored and the OK response will be returned. If the requested state cannot be achieved, an ERROR or +CME ERROR response is returned.

| AT+CGATT Attachment or Detachment of PS |  |
| --- | --- |
| Test Command AT+CGATT=? | Response +CGATT: (list of supported <state>s) OK |
| Read Command AT+CGATT? | Response +CGATT: <state> OK |
| Write Command AT+CGATT=<state> | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 140s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<state>	Indicates the state of PS attachment

0	Detached

1	Attached

Other values are reserved and will result in an ERROR response to the Write Command

Example

AT+CGATT=1

OK AT+CGATT=0 OK AT+CGATT?

+CGATT: 0

//Attach to PS service

//Detach from PS service

//Query the current PS service state

OK

## AT+CGDCONT   Define PDP Context

The command specifies PDP context parameters for a specific context <cid>. A special form of the Write Command (AT+CGDCONT=<cid>) causes the values for context <cid> to become undefined. It is not allowed to change the definition of an already activated context.

The Read Command returns the current settings for each defined PDP context.

| AT+CGDCONT Define PDP Context |  |
| --- | --- |
| Test Command AT+CGDCONT=? | Response +CGDCONT: (range of supported <cid>s),<PDP_type>, <APN>,<PDP_addr>,(list of supported <data_comp>s),(list of supported <head_comp>s) OK |
| Read Command AT+CGDCONT? | Response +CGDCONT: <cid>,<PDP_type>,<APN>,<PDP_addr>,<data_comp>,<h ead_comp>[…] OK |
| Write Command AT+CGDCONT=<cid>[,<PDP_type>[,< APN>[,<PDP_addr>[,<data_comp>[,< head_comp>]]]]] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	PDP context identifier. A numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of permitted  values  (minimum value=1) is returned by the test form of the command.

<PDP_type> Packet data protocol type, a string parameter which specifies the type of packet data protocol.

“IP”	IPV4 “PPP”

“IPV6” “IPV4V6”

<APN>	Access point name, a string parameter that is a logical name used to select the GGSN or the external packet data network. If the value is null or omitted, then the subscription value will be requested.

<PDP_addr> A string parameter identifies the MT in the address space applicable to the PDP. If the value is null or omitted, then a value may be provided by the TE during the PDP startup procedure or, failing that, a dynamic address will be requested. The allocated address may be read using the AT+CGPADDR command.

<data_comp> A numeric parameter that controls PDP data compression (applicable for SNDCP only) (refer to 3GPP TS 44.065).

0	Off (Default if value is omitted)

- On (Manufacturer preferred compression)

- V.42bis

- V.44 (Not supported currently)

<head_comp>	A numeric parameter that controls PDP header compression (refer to 3GPP TS 44.065

and 3GPP TS 25.323).

0	Off

1	On

2	RFC1144

3	RFC2507

4	RFC3095

## AT+CGQREQ   Quality of Service Profile (Requested)

The command allows the TE to specify the quality of service profile that is used when the MT activates a PDP context.

The Write Command specifies a profile for the context <cid>. A special form of the Write Command, AT+CGQREQ=<cid> causes the requested profile for context number <cid> to become undefined. The Read Command returns the current settings for each defined context. Details can be found in 3GPP TS

23.107 and all parameters are saved in NV automatically.

| AT+CGQREQ Quality of Service Profile (Requested) |  |
| --- | --- |
| Test Command AT+CGQREQ=? | Response +CGQREQ: <PDP_type>, (list of supported <precedence>s), (list of supported <delay>s), (list of supported <reliability>s), (list of supported <peak>s), (list of supported <mean>s) OK |
| Read Command | Response |
| AT+CGQREQ? | [+CGQREQ: |
|  | <cid>,<precedence>,<delay>,>reliability>,<peak>,<mean |
|  | >] |
|  | [+CGQREQ: |
|  | <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean |
|  | >] |
|  | […] |
|  | OK |
| Write Command AT+CGQREQ=<cid>[,<precedence>[, <delay>[,<reliability>[,<peak>[,<mean >]]]]] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	A  numeric  parameter  which  specifies  a  particular  PDP  context  definition  (see

AT+CGDCONT command)

<PDP_type>	Packet Data Protocol type

“IP”	Internet Protocol (IETF STD 5) “PPP”

“IPV6” “IPV4V6”

<precedence>	A numeric parameter which specifies the precedence class

0	Network subscribed value

- High Priority. Service commitments shall be maintained ahead of precedence classes 2 and 3

- Normal  priority.  Service  commitments  shall   be  maintained   ahead   of precedence class 3

- Low priority. Service commitments shall be maintained

<delay>	A numeric parameter which specifies the delay class. This parameter defines the end-to-end transfer delay incurred in the transmission of SDUs through the network. For the details, please refer to Table 5.

0	Network subscribed value

<reliability>	A numeric parameter which specifies the reliability class

0	Network subscribed value

- Non real-time traffic, error-sensitive application that cannot cope with data loss

- Non real-time traffic, error-sensitive application that can cope with infrequent data loss

- Non real-time traffic, error-sensitive application that can cope with data loss, GMM/SM, and SMS

- Real-time traffic, error-sensitive application that can cope with data loss

- Real-time traffic, error non-sensitive application that can cope with data loss

<peak>	A numeric parameter which specifies the peak throughput class, in octets per second.

0	Network subscribed value

1	Up to 1 000 (8 kbit/s) 2	Up to 2 000 (16 kbit/s) 3	Up to 4 000 (32 kbit/s) 4	Up to 8 000 (64 kbit/s)

5	Up to 16 000 (128 kbit/s)

6	Up to 32 000 (256 kbit/s)

7	Up to 64 000 (512 kbit/s)

8	Up to 128 000 (1024 kbit/s)

9	Up to 256 000 (2048 kbit/s)

<mean>	A numeric parameter which specifies the mean throughput class, in octets per hour.

0	Network subscribed value 1	100 (~0.22 bit/s)

2	200 (~0.44 bit/s)

3	500 (~1.11 bit/s)

4	1 000 (~2.2 bit/s)

5	2 000 (~4.4 bit/s)

6	5 000 (~11.1 bit/s)

7	10 000 (~22 bit/s)

8	20 000 (~44 bit/s)

9	50 000 (~111 bit/s)

10	100 000 (~0.22 kbit/s)

11	200 000 (~0.44 kbit/s)

12	500 000(~1.11 kbit/s)

13	1000 000 (~2.2 kbit/s)

14	2 000 000 (~4.4 kbit/s)

15	5 000 000 (~11.1 kbit/s)

16	10 000 000 (~22 kbit/s)

17	20 000 000 (~44 kbit/s)

18	50 000 000 (~111 kbit/s)

31	Best effort

#### Table 5: Delay Class

SDU Size	Delay Class	Mean Transfer Delay	95 Percentile

1 (Predictive)	<0.5	<1.5

128 octets

- (Predictive)	<5	<25

- (Predictive)	<50	<250

- (Best Effort)	Unspecified	-

1 (Predictive)	<0.5	<1.5

1024 octets

- (Predictive)	<5	<25

- (Predictive)	<50	<250

- (Best Effort)	Unspecified	-

## AT+CGQMIN   Quality of Service Profile (Minimum Acceptable)

The command allows the TE to specify a minimum acceptable profile which is checked by the MT against the negotiated profile when the PDP context is activated. The write command specifies a profile for the context identified by the context identification parameter <cid>.

A special form of the write command, AT+CGQMIN=<cid> causes the minimum acceptable profile for context number <cid> to become undefined. In this case no check is made against the negotiated profile. The read command returns the current settings for each defined context. Details can be found in 3GPP TS 23.107 and all parameters are saved in NV automatically.

| AT+CGQMIN Quality of Service Profile (Minimum Acceptable) |  |
| --- | --- |
| Test Command AT+CGQMIN=? | Response +CGQMIN: <PDP_type>, (list of supported <precedence>s), (list of supported <delay>s), (list of supported <reliability>s), |

|  | (list of supported <peak>s), (list of supported <mean>s) OK |
| --- | --- |
| Read Command | Response |
| AT+CGQMIN? | [+CGQMIN: |
|  | <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean |
|  | >] |
|  | [+CGQMIN: |
|  | <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean |
|  | >] |
|  | […] |
|  | OK |
| Write Command AT+CGQMIN=<cid>[,<precedence>[,< delay>[,<reliability>[,<peak>[,<mean> ]]]]] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	A  numeric  parameter  which  specifies  a  particular  PDP  context  definition  (see

AT+CGDCONT command)

<PDP_type>	Packet Data Protocol type

“IP”	IPV4. Internet Protocol (IETF STD 5) “PPP”

“IPV6” “IPV4V6”

<precedence>	A numeric parameter which specifies the precedence class

0	Network subscribed value

- High Priority. Service commitments shall be maintained ahead of precedence classes 2 and 3

- Normal  priority.  Service  commitments  shall   be  maintained  ahead   of precedence class 3

- Low priority. Service commitments shall be maintained

<delay>		A numeric parameter which specifies the delay class. This parameter defines the end-to-end transfer delay incurred in the transmission of SDUs through the network. For the detail please refer to Table 5

0	Network subscribed value

<reliability>	A numeric parameter which specifies the reliability class.

0	Network subscribed value

- Non real-time traffic, error-sensitive application that cannot cope with data loss

- Non real-time traffic, error-sensitive application that can cope with infrequent data loss

- Non real-time traffic, error-sensitive application that can cope with data loss, GMM/SM, and SMS

- Real-time traffic, error-sensitive application that can cope with data loss

- Real-time traffic, error non-sensitive application that can cope with data loss

<peak>	A numeric parameter which specifies the peak throughput class, in octets per second.

0	Network subscribed value

- Up to 1 000 (8 kbit/s)

- Up to 2 000 (16 kbit/s)

- Up to 4 000 (32 kbit/s)

- Up to 8 000 (64 kbit/s)

5	Up to 16 000 (128 kbit/s)

6	Up to 32 000 (256 kbit/s)

7	Up to 64 000 (512 kbit/s)

8	Up to 128 000 (1024 kbit/s)

9	Up to 256 000 (2048 kbit/s)

<mean>	A numeric parameter which specifies the mean throughput class, in octets per hour.

0	Network subscribed value

1	100 (~0.22 bit/s)

2	200 (~0.44 bit/s)

3	500 (~1.11 bit/s)

4	1 000 (~2.2 bit/s)

5	2 000 (~4.4 bit/s)

6	5 000 (~11.1 bit/s)

7	10 000 (~22 bit/s)

8	20 000 (~44 bit/s)

9	50 000 (~111 bit/s)

10	100 000 (~0.22 kbit/s)

11	200 000 (~0.44 kbit/s)

12	500 000(~1.11 kbit/s)

13	1000 000 (~2.2 kbit/s)

14	2 000 000 (~4.4 kbit/s)

15	5 000 000 (~11.1 kbit/s)

16	10 000 000 (~22 kbit/s)

17	20 000 000 (~44 kbit/s)

18	50 000 000 (~111 kbit/s)

31	Best effort

## AT+CGEQREQ   3G Quality of Service Profile (Requested)

The command allows the TE to specify a UMTS Quality of Service Profile that is used when the MT activates a PDP context. Details can be found in 3GPP TS 23.107 and all parameters are saved in NV automatically.

| AT+CGEQREQ 3G Quality of Service Profile (Requested) |  |
| --- | --- |
| Test Command AT+CGEQREQ=? | Response +CGEQREQ: <PDP_type>, (list of supported <Traffic class>s), (list of supported <Maximum bitrate UL>s), (list of supported <Maximum bitrate DL>s), (list of supported <Guaranteed bitrate UL>s), (list of supported <Guaranteed bitrate DL>s), (list of supported <Delivery order>s), (list of supported <Maximum SDU size>s), (list of supported <SDU error ratio>s), (list of supported <Residual bit error ratio>s), (list of supported <Delivery of erroneous SDUs>s), (list of supported <Transfer delay>s), (list of supported <Traffic handling priority>s), (list of supported <Source statistics descriptor>s), (list of supported <Signalling indication>s) OK |
| Read Command AT+CGEQREQ? | Response [+CGEQREQ: <cid>, <Traffic class>, <Maximum bitrate UL>, <Maximum bitrate DL>, <Guaranteed bitrate UL>, <Guaranteed bitrate DL>, <Delivery order>, <Maximum SDU size>, <SDU error ratio>, <Residual bit error ratio>, <Delivery of erroneous SDUs>, <Transfer delay>, <Traffic handling priority>, <Source statistics descriptor>, <Signalling indication>] [...] |

|  | OK |
| --- | --- |
| Write Command AT+CGEQREQ=[<cid>[,<Traffic class>[,<Maximum bitrate UL> [,<Maximum bitrate DL> [,<Guaranteed bitrate UL> [,<Guaranteed bitrate DL> [,<Delivery order> [,<Maximum SDU size> [,<SDU error ratio> [,<Residual bit error ratio> [,<Delivery of erroneous SDUs> [,<Transfer delay> [,<Traffic handling priority> [,<Source statistics descriptor> [,<Signalling indication>]]]]]]]]]]]]]]] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	PDP context identifier, a numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of permitted values (minimum value=1) is returned by the test form of the command

<PDP_type>	Packet data protocol type, a string parameter which specifies the type

of packet data protocol

“IP”	IPV4. Internet protocol (IETF STD 5) “PPP”

“IPV6” “IPV4V6”

The following parameters are defined in 3GPP TS 23.107

<Traffic class>	Integer type. Indicates the type of application for which the UMTS

bearer service is optimized (refer to 3GPP TS 24.008 subclause 10.5.6.5). If the parameter is specified as conversational or streaming, then the Guaranteed and Maximum bitrate parameters should also be provided.

0	Conversational

- Streaming

- Interactive

- Background

- Subscribed value

<Maximum bitrate UL>	Integer type. Indicates the maximum number of kbits/s delivered to

UMTS (up-link traffic) at a SAP. As an example a bit rate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~5760

<Maximum bitrate DL>	Integer type. Indicates the maximum number of kbits/s delivered by

UMTS (down-link traffic) at a SAP. As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~42200

<Guaranteed bitrate UL>	Integer type. Indicates the guaranteed number of kbits/s delivered to

UMTS (up-link traffic) at a SAP (provided that there is data to deliver). As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~5760

<Guaranteed bitrate DL>	Integer type. Indicates the guaranteed number of kbits/s delivered by

UMTS (down-link traffic) at a SAP (provided that there is data to deliver). As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~42200

<Delivery order>	Integer  type.  Indicates  whether  the  UMTS  bearer  shall  provide

in-sequence SDU delivery or not (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	No

- Yes

- Subscribed value

<Maximum SDU size>	Integer type. (1,2,3,…) indicates the maximum allowed SDU size in

octets. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed value

10...1520	(Value needs to be divisible by 10 without remainder) 1520

<SDU error ratio>	String type. Indicates the target value for the fraction of SDUs lost or

detected as erroneous. SDU error ratio is defined only for conforming traffic. The value is specified as „mEe‟. As an example a target SDU error ratio of 5*10-3 would be specified as “5E3” (e.g. AT+CGEQREQ=…,"5E3",…).

“0E0”	Subscribed value “1E1”

“1E2”

“7E3”

“1E3”

“1E4”

“1E5”

“1E6”

<Residual bit error ratio>	String type. Indicates the target value for the undetected bit error ratio

in the delivered SDUs. If no error detection is requested, Residual bit error ratio indicates the bit error ratio in the delivered SDUs. The value is specified as “mEe”. As an example a target residual bit error ratio of 5•10-3 would be specified as “5E3” (e.g. AT+CGEQREQ=…,“5E3”,…).

“0E0”	Subscribed value “5E2”

“1E2”

“5E3”

“4E3”

“1E3”

“1E4”

“1E5”

“1E6”

“6E8”

<Delivery of erroneous SDUs> Integer type. Indicates whether SDUs detected as erroneous shall be

delivered or not (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	No

- Yes

- No detect

- Subscribed value

<Transfer delay>	Integer type. (0,1,2,…) indicates the targeted time between request to

transfer an SDU at one SAP to its delivery at the other SAP, in milliseconds. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed value

100~150	(value needs to be divisible by 10 without remainder) 200~950	(value needs to be divisible by 50 without remainder) 1000~4000	(value needs to be divisible by 100 without remainder)

<Traffic handling priority>	Integer type. (1,2,3,…) specifies the relative importance for handling of

all SDUs belonging to the UMTS bearer compared to the SDUs of other bearers. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed 1

2

3

<Source statistics descriptor> Integer type. Specifies characteristics of the source of the submitted

SDUs for a PDP context.

0	Characteristics of SDUs is unknown

1	Characteristics of SDUs correspond to a speech source

<Signalling indication>	Integer type. Indicates signaling content of submitted SDUs for a PDP

context.

0	PDP context is not optimized for signaling

1	PDP context is optimized for signaling

## AT+CGEQMIN   3G Quality of Service Profile (Minimum Acceptable)

The command allows the TE to specify a minimum acceptable profile, which is checked by the MT against the negotiated profile returned in the PDP context establishment and PDP context modification procedures. Details can be found in 3GPP TS 23.107 and all parameters are saved in NV automatically.

| AT+CGEQMIN 3G Quality of Service Profile (Minimum Acceptable) |  |
| --- | --- |
| Test Command AT+CGEQMIN=? | Response +CGEQMIN: <PDP_type>, (list of supported <Traffic class>s), (list of supported <Maximum bitrate UL>s), (list of supported <Maximum bitrate DL>s), (list of supported <Guaranteed bitrate UL>s), (list of supported <Guaranteed bitrate DL>s), (list of supported <Delivery order>s), (list of supported <Maximum SDU size>s), (list of supported <SDU error ratio>s), (list of supported <Residual bit error ratio>s), (list of supported <Delivery of erroneous SDUs>s), (list of supported <Transfer delay>s), (list of supported <Traffic handling priority>s), (list of supported <Source statistics descriptor>s), (list of supported <Signalling indication>s) OK |
| Read Command AT+CGEQMIN? | Response [+CGEQMIN: <cid>, <Traffic class>, <Maximum bitrate UL>, <Maximum bitrate DL>, <Guaranteed bitrate UL>, <Guaranteed bitrate DL>, <Delivery order>, <Maximum SDU size>, <SDU error ratio>, |

|  | <Residual bit error ratio>, <Delivery of erroneous SDUs>, <Transfer delay>, <Traffic handling priority>, <Source statistics descriptor>, <Signalling indication>] […] OK |
| --- | --- |
| Write Command AT+CGEQMIN=[<cid>[,<Traffic class> [,<Maximum bitrate UL> [,<Maximum bitrate DL> [,<Guaranteed bitrate UL> [,<Guaranteed bitrate DL> [,<Delivery order> [,<Maximum SDU size> [,<SDU error ratio> [,<Residual bit error ratio> [,<Delivery of erroneous SDUs> [,<Transfer delay> [,<Traffic handling priority> [,<Source statistics descriptor> [,<Signalling indication>]]]]]]]]]]]]]]] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	PDP context identifier. A numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of permitted values (minimum value=1) is returned by the test form of the command

<PDP_type>	Packet data protocol type. A string parameter which specifies the type

of packet data protocol. “IP”	IPV4

“PPP”

“IPV6” “IPV4V6”

The following parameters are defined in 3GPP TS 23.107

<Traffic class>	Integer type. Indicates the type of application for which the UMTS

bearer service is optimized (refer to 3GPP TS 24.008 subclause 10.5.6.5). If the parameter is specified as conversational or streaming, then the Guaranteed and Maximum bitrate parameters should also be provided.

0	Conversational

- Streaming

- Interactive

- Background

- Subscribed value

<Maximum bitrate UL>	Integer type. Indicates the maximum number of kbits/s delivered to

UMTS (up-link traffic) at a SAP. As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~5760

<Maximum bitrate DL>	Integer type. Indicates the maximum number of kbits/s delivered by

UMTS (down-link traffic) at a SAP. As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~42200

<Guaranteed bitrate UL>	Integer type. Indicates the guaranteed number of kbits/s delivered to

UMTS (up-link traffic) at a SAP (provided that there is data to deliver). As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~5760

<Guaranteed bitrate DL>	Integer type. Indicates the guaranteed number of kbits/s delivered by

UMTS (down-link traffic) at a SAP (provided that there is data to deliver). As an example a bitrate of 32kbit/s would be specified as „32‟ (e.g. AT+CGEQREQ=…,32, …).

0	Subscribed value 1~42200

<Delivery order>	Integer  type.  Indicates  whether  the  UMTS  bearer  shall  provide

in-sequence SDU delivery or not (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	No

- Yes

- Subscribed value

<Maximum SDU size>	Integer type. (1,2,3,…) indicates the maximum allowed SDU size in

octets. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed value

10...1520	(value needs to be divisible by 10 without remainder) 1502

<SDU error ratio>	String type. Indicates the target value for the fraction of SDUs lost or

detected as erroneous. SDU error ratio is defined only for conforming traffic. The value is specified as „mEe‟. As an example a target SDU error ratio of 5*10-3 would be specified as “5E3” (e.g. AT+CGEQREQ=…,”5E3”,…).

“0E0”	Subscribed value “1E2”

“7E3”

“1E3”

“1E4”

“1E5”

“1E6”

“1E1”

<Residual bit error ratio>	String type. Indicates the target value for the undetected bit error ratio

in the delivered SDUs. If no error detection is requested, Residual bit error ratio indicates the bit error ratio in the delivered SDUs. The value is specified as “mEe”. As an example a target residual bit error ratio of 5•10-3 would be specified as “5E3” (e.g. AT+CGEQREQ=…,”5E3”,…).

“0E0”	Subscribed value “5E2”

“1E2”

“5E3”

“4E3”

“1E3”

“1E4”

“1E5”

“1E6”

“6E8”

<Delivery of erroneous SDUs> Integer type. Indicates whether SDUs detected as erroneous shall be

delivered or not (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	No

- Yes

- No detect

- Subscribed value

<Transfer delay>	Integer type. (0,1,2,…) indicates the targeted time between request to

transfer an SDU at one SAP to its delivery at the other SAP, in milliseconds. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed value

100~150	(value needs to be divisible by 10 without remainder) 200~950	(value needs to be divisible by 50 without remainder) 1000~4000	(value needs to be divisible by 100 without remainder)

<Traffic handling priority>	Integer type. (1,2,3,…) specifies the relative importance for handling of

all SDUs belonging to the UMTS bearer compared to the SDUs of other bearers. If the parameter is set to „0‟ the subscribed value will be requested (refer to 3GPP TS 24.008 subclause 10.5.6.5).

0	Subscribed 1

2

3

<Source statistics descriptor> Integer type. Specifies characteristics of the source of the submitted

SDUs for a PDP context.

0	Characteristics of SDUs are unknown

1	Characteristics of SDUs corresponds to a speech source

<Signalling indication>	Integer type. Indicates signaling content of submitted SDUs for a PDP

context.

0	PDP context is not optimized for signaling

1	PDP context is optimized for signaling

## AT+CGACT   Activate or Deactivate PDP Context

The Write Command is used to activate or deactivate the specified PDP context(s). After the command has been completed, the MT remains in V.250 command state. If any PDP context is already in the requested state, the state for that context remains unchanged. If the MT is not PS attached when the activation form of the command is executed, the MT first performs a PS attach and then attempts to activate the specified contexts. If no <cid>s specify the activation/deactivation form of the command, it will activate or deactivate all defined contexts.

| AT+CGACT Activate or Deactivate PDP Context |  |
| --- | --- |
| Test Command AT+CGACT=? | Response +CGACT: (list of supported <state>s) OK |
| Read Command AT+CGACT? | Response +CGACT: <cid>,<state>[<CR><LF>+CGACT: <cid>,<state >…] OK |
| Write Command AT+CGACT=<state>,<cid> | Response OK NO CARRIER If there is any error related to ME functionality: +CME ERROR: <err> |

| Maximum Response Time | 150s, determined by network. |
| --- | --- |
| Reference 3GPP TS 27.007 |  |

Parameter

<state>	Indicates the state of PDP context activation

0	Deactivated

1	Activated

Other values are reserved and will result in an ERROR response to the Write Command

<cid>	A   numeric   parameter   which   specifies   a   particular   PDP   context   definition   (see

AT+CGDCONT command)

Example

AT+CGDCONT=1,“IP”,“UNINET”

OK AT+CGACT=1,1 OK AT+CGACT=0,1

OK

//Define PDP context

//Activated PDP

//Deactivated PDP

## AT+CGDATA   Enter Data State

The Write Command  causes the MT to perform whatever  actions that  are  necessary to  establish communication between the TE and the network using one or more packet domain PDP types. This may include per-forming a PS attach and one or more PDP context activations. Commands following the AT+CGDATA command in the AT command line will not be processed by the MT.

If the <L2P> parameter value is unacceptable to the MT, the MT shall return an ERROR or +CME ERROR response. Otherwise, the MT issues the intermediate result code CONNECT and enters V.250 online data state. After data transfer is completed, and the layer 2 protocol termination procedure has been completed successfully, the command state is reentered and the MT returns the final result code OK.

| AT+CGDATA Enter Data State |  |
| --- | --- |
| Test Command AT+CGDATA=? | Response +CGDATA: (list of supported <L2P>s) OK |
| Write Command AT+CGDATA=<L2P>[,<cid>[,<cid>[,… | Response CONNECT |

| ]]] | ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<L2P>		A string parameter that indicates the layer 2 protocol to be used between the TE and MT: PPP (Point to Point protocol) for a PDP such as IP

Other values are not supported and will result in an ERROR response to the execution command

<cid>	A   numeric   parameter   which   specifies   a   particular   PDP   context   definition   (see

AT+CGDCONT command)

## AT+CGPADDR	Show PDP Address

The Write Command returns a list of PDP addresses for the specified context identifiers. If no <cid> is specified, the addresses for all defined contexts are returned.

| AT+CGPADDR Show PDP Address |  |
| --- | --- |
| Test Command AT+CGPADDR=? | Response +CGPADDR: (list of defined <cid>s) OK |
| Write Command AT+CGPADDR[=<cid>[,<cid>[,…]]] | Response +CGPADDR: <cid>,<PDP_addr> [+CGPADDR: <cid>,<PDP_addr>[…]] OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<cid>	A  numeric  parameter  which  specifies  a  particular  PDP  context  definition  (see

AT+CGDCONT command)

<PDP_addr> A string that identifies the MT in the address space applicable to the PDP. The address may be static or dynamic. For a static address, it will be the one set by AT+CGDCONT command when the context was defined. For a dynamic address it will be the one assigned during the last PDP context activation that used the context definition referred to by <cid>. <PDP_address> is omitted if none is available.

Example

AT+CGDCONT=1,“IP”,“UNINET”

OK AT+CGACT=1,1 OK AT+CGPADDR=1

+CGPADDR: 1,“10.76.51.180”

//Define PDP context

//Activated PDP

//Show PDP address

OK

## AT+CGCLASS	GPRS Mobile Station Class

The command is used to set the MT to operate according to the specified mode of operation. See 3GPP TS 23.060.

| AT+CGCLASS GPRS Mobile Station Class |  |
| --- | --- |
| Test Command AT+CGCLASS=? | Response +CGCLASS: (list of supported <class>s) OK |
| Read Command AT+CGCLASS? | Response +CGCLASS: <class> OK |
| Write Command AT+CGCLASS=<class> | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |

| Maximum Response Time | 300ms |
| --- | --- |
| Reference 3GPP TS 27.007 |  |

Parameter

<class>	A string  parameter  which  indicates  the  GPRS  mobile  class  (Functionality  in descending order)

“A”	Class A

## AT+CGREG   Network Registration Status

The command queries the network registration status and controls the presentation of an unsolicited result code +CGREG: <stat> when <n>=1 and there is a change in the MT‟s GPRS network registration status in GERAN/UTRAN, or unsolicited result code +CGREG: <stat>[,[<lac>],[<ci>],[<Act>],[<rac>]] when <n>=2 and there is a change of the network cell in GERAN/UTRAN.

| AT+CGREG Network Registration Status |  |
| --- | --- |
| Test Command AT+CGREG=? | Response +CGREG: (list of supported <n>s) OK |
| Read Command AT+CGREG? | Response +CGREG: <n>,<stat>[,<lac>,<ci>[,<Act>]] OK |
| Write Command AT+CGREG[=<n>] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Disable network registration unsolicited result code

- Enable network registration unsolicited result code +CGREG: <stat>

- Enable network registration and location information unsolicited result code

#### +CGREG: <stat>[,<lac>,<ci>[,<Act>]]

| <stat> | 0 | Not registered. MT is not currently searching an operator to register to. The UE is |
| --- | --- | --- |
|  |  | in GMM state GMM-NULL or GMM-DEREGISTERED-INITIATED. The GPRS |
|  |  | service is disabled, but the UE is allowed to attach for GPRS if requested by the |
|  |  | user. |
|  | 1 | Registered, home network. The UE is in GMM state GMM-REGISTERED or |
|  |  | GMM-ROUTING-AREA-UPDATING-INITIATED INITIATED on the home PLMN. |
|  | 2 | Not registered, but MT is currently trying to attach or searching an operator to |

register to. The UE is in GMM state GMM-DEREGISTERED or GMM-REGISTERED-INITIATED. The GPRS service is enabled, but an allowable PLMN is currently not available. The UE will start a GPRS attach as soon as an allowable PLMN is available.

- Registration denied. The UE is in GMM state GMM-NULL. The GPRS service is disabled, and the UE is not allowed to attach for GPRS if requested by the user.

- Unknown

- Registered, roaming

<lac>	String type. Two-byte location area code in hexadecimal format (e.g. “00C3” equals 195 in decimal)

<ci>	String type. 16-bit (GSM) or 28-bit (UMTS/LTE) cell ID in hexadecimal format

<Act>	Access technology selected

0	GSM

- UTRAN

- GSM W/EGPRS

- UTRAN W/HSDPA

- UTRAN W/HSUPA

- UTRAN W/HSDPA and HSUPA

- E-UTRAN

Example

AT+CGREG=2 OK AT+CGATT=0 OK

+CGREG: 2 AT+CGATT=1 OK

+CGREG: 1,“D504”,“80428B5”,7

## AT+CGEREP   Packet Domain Event Reporting

The Write Command enables or disables sending of unsolicited result codes +CGEV: XXX from MT to TE in the case of certain events occurring in the Packet Domain MT or the network. <mode> controls the processing of unsolicited result codes specified within this command. <bfr> controls the effect on buffered codes when <mode> 1 or 2 is entered.

| AT+CGEREP Packet Domain Event Reporting |  |
| --- | --- |
| Test Command AT+CGEREP=? | Response +CGEREP: (list of supported <mode>s),(list of supported <bfr>s) OK |
| Read Command AT+CGEREP? | Response +CGEREP: <mode>,<bfr> OK |
| Write Command AT+CGEREP=mode[,<bfr>] | Response OK ERROR |
| Execution Command AT+CGEREP | Response OK |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<mode>	0	Buffer unsolicited result codes in the MT; if MT result code buffer is full, the oldest ones can be discarded. No codes are forwarded to the TE.

- Discard unsolicited result codes when MT-TE link is reserved (e.g. in on-line data mode), otherwise forward them directly to the TE.

- Buffer unsolicited result codes in the MT when MT-TE link is reserved (e.g. in on-line data mode) and flush them to the TE when MT-TE link becomes available. Otherwise forward them directly to the TE.

<bfr>	0	MT buffer of unsolicited result codes defined within this command is cleared when

<mode> 1 or 2 is entered.

1	MT buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> 1 or 2 is entered (OK response shall be given before flushing the codes).

#### NOTE

The unsolicited result codes and the corresponding events are defined as follows:

1. +CGEV: REJECT <PDP_type>, <PDP_addr>: A network request for PDP context  activation occurred when the MT was unable to report it to the TE with a +CRING unsolicited result code and was automatically rejected.

Note: This event is not applicable for EPS.

2. +CGEV: NW REACT <PDP_type>, <PDP_addr>, [<cid>]: The network has requested a context reactivation. The <cid> used to reactivate the context is provided if known to the MT.

Note: This event is not applicable for EPS.

3. +CGEV: NW DEACT <PDP_type>, <PDP_addr>, [<cid>]: The network has forced  a context deactivation. The <cid> used to activate the context is provided if known to the MT.

4. +CGEV: ME DEACT <PDP_type>, <PDP_addr>, [<cid>]: The mobile equipment has forced a context deactivation. The <cid> used to activate the context is provided if known to the MT.

5. +CGEV: NW DETACH: The network has forced a Packet Domain detach. This implies that all active contexts have been deactivated. These are not reported separately.

6.   +CGEV: ME DETACH: The mobile equipment has forced a Packet Domain detach. This implies that all active contexts have been deactivated. These are not reported separately.

7. +CGEV: NW CLASS <class>: The network has forced a change of MS class. The highest available class is reported (see AT+CGCLASS).

8. +CGEV: ME CLASS <class>: The mobile equipment has forced a change of MS class. The highest available class is reported (see AT+CGCLASS).

9. +CGEV: PDN ACT <cid>: Activated a context. The context represents a PDN connection in LTE or a Primary PDP context in GSM/UMTS.

10. +CGEV: PDN DEACT <cid>: Deactivated a context. The context represents a PDN connection in LTE or a Primary PDP context in GSM/UMTS.

Example

AT+CGEREP=?

+CGEREP: (0-2),(0,1)

OK AT+CGEREP?

+CGEREP: 0,0

OK

## AT+CGSMS	Select Service for MO SMS Messages

The command specifies the service or service preference that the MT will use to send MO (mobile originated) SMS messages.

| AT+CGSMS Select Service for MO SMS Messages |  |
| --- | --- |
| Test Command AT+CGSMS=? | Response +CGSMS: (list of currently available <service>s) OK |
| Read Command AT+CGSMS? | Response +CGSMS: <service> OK |
| Write Command AT+CGSMS=[<service>] | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<service>	A numeric parameter which indicates the service or service preference to be used

0	GPRS

- Circuit switch

- GPRS preferred (use circuit switched if GPRS not available)

- Circuit switch preferred (use GPRS if circuit switched not available)

The circuit switched service route is the default method.

NOTE

## AT+CEREG   EPS Network Registration Status

The command queries the network registration status and controls the presentation of an unsolicited result code +CEREG: <stat> when <n>=1 and there is a change in the MT‟s EPS network registration status in E-UTRAN, or unsolicited result code +CEREG: <stat>[,[<tac>],[<ci>],[<Act>]] when <n>=2 and there is a change of the network cell in E-UTRAN.

| AT+CEREG EPS Network Registration Status |  |
| --- | --- |
| Test Command AT+CEREG=? | Response +CEREG: (list of supported <n>s) OK |
| Read Command AT+CEREG? | Response +CEREG: <n>,<stat>[,<tac>,<ci>[,<Act>]] OK |
| Write Command AT+CEREG[=<n>] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Disable network registration unsolicited result code

- Enable network registration unsolicited result code +CEREG: <stat>

- Enable network registration and location information unsolicited result code

#### +CEREG: <stat>[,<tac>,<ci>[,<Act>]]

<stat>	0	Not registered. MT is not currently searching an operator to register to

- Registered, home network

- Not registered, but MT is currently trying to attach or searching an operator to register to

- Registration denied

- Unknown

- Registered, roaming

<tac>	String type. Two-byte tracking area code in hexadecimal format

<ci>	String type. 28-bit E-UTRAN cell ID in hexadecimal format

<Act>	Access technology selected

0	GSM

- UTRAN

- GSM W/EGPRS

- UTRAN W/HSDPA

- UTRAN W/HSUPA

- UTRAN W/HSDPA and HSUPA

- E-UTRAN

## AT+QGDCNT	Packet Data Counter

The command allows the application to check how much bytes are sent to or received by the module.

| AT+QGDCNT Packet Data Counter |  |
| --- | --- |
| Test Command AT+QGDCNT=? | Response +QGDCNT: (0,1) OK |
| Read Command AT+QGDCNT? | Response +QGDCNT: <bytes_sent>,<bytes_recv> OK |
| Write Command AT+QGDCNT=<op> | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

<op>	A numeric parameter. The operation about data counter

0	Reset the data counter

1	Save the results of data counter to NV

If results need to be automatically saved, please refer to AT+QAUGDCNT command.

<bytes_sent>	A numeric parameter. The amount of sent bytes.

<bytes_recv>	A numeric parameter. The amount of received bytes.

When module is powered on, <bytes_sent> and <bytes_recv> will be loaded from results of data counter in NV. The default result in NV is 0.

NOTE

Example

AT+QGDCNT=?

+QGDCNT: (0,1)

//Test command

OK

AT+QGDCNT?

//Query the current bytes sent and received

+QGDCNT: 3832,4618

OK

AT+QGDCNT=1

OK

//Save the results to NV

AT+QGDCNT =0

OK

//Reset counter

## AT+QAUGDCNT	Auto Save Packet Data Counter

The command allows AT+QGDCNT command to save results to NV automatically.

| AT+QAUGDCNT Auto Save Packet Data Counter |  |
| --- | --- |
| Test Command AT+QAUGDCNT=? | Response +QAUGDCNT: (0,30-65535) OK |
| Read Command AT+QAUGDCNT? | Response +QAUGDCNT: <value> OK |
| Write Command AT+QAUGDCNT=<value> | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |

Parameter

#### <value>

A numeric parameter. Default value is 0. The parameter is the time-interval for command AT+QGDCNT to save results to NV automatically. If it is set to 0, auto-save feature would be disabled. Unit is second.

The configuration would not be saved into NV.

NOTE

Example

AT+QAUGDCNT=?

//Test command

+QAUGDCNT: (0,30-65535)

OK

AT+QGDCNT=35

OK

//Set <value>

AT+QAUGDCNT?

+QAUGDCNT: 35

//Query the interval of auto-save

OK

# Supplementary Service Commands

## AT+CCFC	Call Forwarding Number and Conditions Control

The command allows control of the call forwarding supplementary service according to 3GPP TS 22.082. Registration, erasure, activation, deactivation and status query are supported.

| AT+CCFC Call Forwarding Number and Conditions Control |  |
| --- | --- |
| Test Command AT+CCFC=? | Response +CCFC: (list of supported <reads>s) OK |
| Write Command AT+CCFC=<reads>,<mode>[,<numbe r>[,<type>[,<class>[,<subaddr>[,<sat ype>[,time]]]]]] | Response TA controls the call forwarding supplementary service. Registration, erasure, activation, deactivation, and status query are supported. Only <reads> and <mode> should be entered with mode (0-2,4) If <mode> is not equal to 2 and the command is executed successfully: OK If <mode>=2 and the command is executed successfully (only in connection with <reads> 0-3): For registered call forwarding numbers: +CCFC: <status>,<class1>[,<number>,<type>[,<subaddr>,<satyp e>[,<time>]]] [<CR><LF>+CCFC: ....] OK If no call forwarding numbers are registered (and therefore all classes are inactive): +CCFC: <status>,<class> OK |

|  | where <status>=0 and <class>=15 If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<reads>	0	Unconditional

- Mobile busy

- No reply

- Not reachable

- All call forwarding (0-3)

- All conditional call forwarding (1-3)

<mode>	0	Disable

- Enable

- Query status

- Registration

- Erasure

<number>	Phone number in string type of forwarding address in format specified by <type>

<type>	Type of  address  in integer  format; default value is  145  when  dialing  string  includes international access code character “+”; otherwise 129

<subaddr>   String type sub-address of format specified by <satype>

<satype>	Type of sub-address in integer

| <class> | 1 | Voice |
| --- | --- | --- |
|  | 2 | Data |
|  | 4 | FAX |
|  | 7 | All telephony except SMS |
|  | 8 | Short message service |
|  | 16 | Data circuit synchronization |
|  | 32 | Data circuit asynchronization |
| <time> | 1…30 | When “no reply” (<reads>=no reply) is enabled or queried, this gives the time in |
|  |  | seconds to wait before call is forwarded; default value is 20 |
| <status> | 0 | Not active |
|  | 1 | Active |

Example

AT+CCFC=0,3,“15021012496”

//Register the destination number for unconditional call

forwarding (CFU)

OK

AT+CCFC=0,2

+CCFC: 1,1,“+8615021012496”,145,,,

//Query the status of CFU without specifying <class>

OK

AT+CCFC=0,4 OK AT+CCFC=0,2

+CCFC: 0,255

//Erase the registered CFU destination number

//Query the status, no destination number

OK

## AT+CCWA	Call Waiting Control

The command allows control of the call waiting supplementary service according to 3GPP TS 22.083. Activation, deactivation and status query are supported.

| AT+CCWA Call Waiting Control |  |
| --- | --- |
| Test Command AT+CCWA=? | Response +CCWA: (list of supported <n>s) OK |
| Read Command AT+CCWA? | Response +CCWA: <n> OK |
| Write Command AT+CCWA[=<n>][,<mode>[,<class>]] | Response TA controls the call waiting supplementary service. Activation, deactivation and status query are supported. If <mode> is not equal to 2 and the command is executed successfully: OK If <mode>=2 and the command is executed successfully: +CCWA: <status>,<class1>[<CR><LF>+CCWA:<status>,<class2>[. ..]] OK If there is any error related to ME functionality: +CME ERROR: <err> |

| Maximum Response Time | 300ms |
| --- | --- |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Disable presentation of an unsolicited result code

1	Enable presentation of an unsolicited result code

<mode>	When <mode> parameter is not given, network is not interrogated

0	Disable

- Enable

- Query status

<class>	A sum of integers, each integer represents a class of information

- Voice (telephony)

- Data (bearer service)

4	FAX (facsimile)

16	Data circuit synchronization

32	Data circuit asynchronization

<status>	0	Disable

1	Enable

<number>	Phone number in string type of calling address in format specified by <type>

<type>	Type of address octet in integer format

129	Unknown type (IDSN format number)

145	International number type (ISDN format )

<alpha>	Optional string type alphanumeric representation of <number> corresponding to the entry found in phone book

- <status>=0 should be returned only if service is not active for any <class> i.e. +CCWA: 0, 7 will be returned in this case.

- When <mode>=2, all active call waiting classes will be reported. In this mode the command is aborted by pressing any key.

- Unsolicited result code:

When the presentation call waiting at the TA is enabled (and call waiting is enabled) and a terminating call set up during an established call, an unsolicited result code is returned:

+CCWA: <number>,<type>,<class>[,<alpha>]

NOTES

Example

AT+CCWA=1,1

OK ATD10086;

//Enable presentation of an unsolicited result code

//Establish a call

OK

+CCWA: “02154450293”,129,1

//Indication of a call that has been waiting

## AT+CHLD   Call Related Supplementary Services

The command allows the control of the following call related services:

- A call can be temporarily disconnected from the MT but the connection is retained by the network;

- Multiparty conversation (conference calls);

- The served subscriber who has two calls (one held and the other either active or alerting) can connect the other parties and release the served subscriber‟s own connection.

Calls can be put on hold, recovered, released and added to a conversation, and transferred similarly as defined in 3GPP TS 22.030.

This is based on the GSM/UMTS supplementary services HOLD (Call Hold; refer to 3GPP TS 22.083 clause 2), MPTY (MultiParty; refer to 3GPP TS 22.084) and ECT (Explicit Call Transfer; refer to 3GPP TS 22.091). The interaction of this command with other commands based on other GSM/UMTS supplementary services is described in the GSM/UMTS standards. Call Hold, MultiParty and Explicit Call Transfer are only applicable to teleservice 11.

| AT+CHLD Call Related Supplementary Services |  |
| --- | --- |
| Test Command AT+CHLD=? | Response +CHLD: (list of supported <n>s) OK |
| Write Command AT+CHLD[=<n>] | Response TA controls the supplementary services call hold, multiparty and explicit call transfer. Calls can be put on hold, recovered, released, added to conversation and transferred. OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Terminate all held calls or UDUB (User Determined User Busy) for a waiting call. If a call is waiting, terminate the waiting call. Otherwise, terminate all held calls (if any)

- Terminate all active calls (if any) and accept the other call (waiting call or held call).

1X	Terminate the specific call number X (X=1-7)

- Place all active calls on hold (if any) and accept the other call (waiting call or held call) as the active call

2X	Place all active calls except call X (X=1-7) on hold

- Add the held call to the active calls

- Connects the two calls and disconnects the subscriber from both calls (ECT)

Example

ATD10086;	//Establish a call

#### OK

+CCWA: “02154450293”,129,1	//Indication of a call that has been waiting

AT+CHLD=2	//Place the active call on hold and accept the waiting call as the active call

#### OK AT+CLCC

+CLCC: 1,0,1,0,0,“10086”,129	//The first call on hold

+CLCC: 2,1,0,0,0,“02154450293”,129	//The second call be active

#### OK

AT+CHLD=21	//Place the active call except call X=1 on hold

#### OK AT+CLCC

+CLCC: 1,0,0,0,0,“10086”,129	//The first call be active

+CLCC: 2,1,1,0,1,“02154450293”,129	//The second call on hold

#### OK

AT+CHLD=3	//Add a held call to the active calls in order to set up a conference (multiparty) call

#### OK AT+CLCC

#### +CLCC: 1,0,0,0,1,“10086”,129

#### +CLCC: 2,1,0,0,1,“02154450293”,129

OK

## AT+CLIP   Calling Line Identification Presentation

The command refers to the GSM/UMTS supplementary service CLIP (Calling Line Identification Presentation) that enables a called subscriber to get the calling line identity (CLI) of the calling party when receiving a mobile terminated call.

| AT+CLIP Calling Line Identification Presentation |  |
| --- | --- |
| Test Command AT+CLIP=? | Response +CLIP: (list of supported <n>s) OK |
| Read Command AT+CLIP? | Response +CLIP: <n>,<m> OK |
| Write Command AT+CLIP=<n> | Response TA enables or disables the presentation of the calling line identity (CLI) at the TE. It has no effect on the execution of the supplementary service CLIP in the network. OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 15s, determined by network. |
| Reference 3GPP TS 27.007 |  |

| Parameter |  |  |
| --- | --- | --- |
| <n> | 0 | Suppress unsolicited result codes |
|  | 1 | Display unsolicited result codes |
| <m> | 0 | CLIP not provisioned |
|  | 1 | CLIP provisioned |
|  | 2 | Unknown |

<number>	Phone number in string type of calling address in format specified by <type>

<subaddr>	String type subaddress of format specified by <satype>

<satype>	Type of subaddress octet in integer format (refer to 3GPP TS 24.008 [8] subclause 10.5.4.8)

<type>	Type of address octet in integer format

|  | 129 | Unknown type (IDSN format) |
| --- | --- | --- |
|  | 145 | International number type (ISDN format) |
|  | 161 | National number |
| <alpha> | String | type alphanumeric representation of <number> corresponding to the entry |
| found in phone book |  |  |
| <CLI_validity> | 0 | CLI valid |
|  | 1 | CLI has been withheld by the originator |
|  | 2 | CLI is not available due to interworking problems or limitations of originating |
|  |  | network |

Unsolicited result code:

When the presentation of the CLIP at the TE is enabled (and calling subscriber allows), an unsolicited result code is returned after every RING (or +CRING: <type>) at a mobile terminating call:

+CLIP: <number>,<type>,[subaddr],[satype],[<alpha>],<CLI_validity>

NOTE

Example

AT+CPBW=1,“02151082965”,129,“QUECTEL” OK

AT+CLIP=1 OK

RING

+CLIP: “02151082965”,129,,,“QUECTEL”,0

## AT+CLIR   Calling Line Identification Restriction

AT+CLIR refers to the CLIR supplementary service (Calling Line Identification Restriction) according to 3GPP TS 22.081 and the OIR supplementary service (Originating Identification Restriction) according to 3GPP TS 24.607 that allows a calling subscriber to enable or disable the presentation of the calling line identity (CLI) to the called party when originating a call.

| AT+CLIR Calling Line Identification Restriction |  |
| --- | --- |
| Test Command AT+CLIR=? | Response +CLIR: (list of supported <n>s) OK |

| Read Command AT+CLIR? | Response +CLIR: <n>,<m> OK |
| --- | --- |
| Write Command AT+CLIR[=<n>] | Response TA restricts or enables the presentation of the calling line identity (CLI) to the called party when originating a call. The command overrides the CLIR subscription (default is restricted or allowed) when temporary mode is provisioned as a default adjustment for all following outgoing calls. This adjustment can be revoked by using the opposite command. OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 15s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	Parameter sets the adjustment for outgoing calls

0	Presentation indicator is used according to the subscription of the CLIR service

- CLIR invocation

- CLIR suppression

<m>	Parameter shows the subscriber CLIR service status in the network

0	CLIR not provisioned

- CLIR provisioned in permanent mode

- Unknown (e.g. no network, etc.)

- CLIR temporary mode presentation restricted

- CLIR temporary mode presentation allowed

## AT+COLP   Connected Line Identification Presentation

The command refers to the GSM/UMTS supplementary service COLP (Connected Line Identification Presentation) that enables a calling subscriber to get the connected line identity (COL) of the called party after setting up a mobile originated call. The command enables or disables the presentation of the COL at the TE. It has no effect on the execution of the supplementary service COLR in the network.

| AT+COLP Connected Line Identification Presentation |  |
| --- | --- |
| Test Command AT+COLP=? | Response +COLP: (list of supported <n>s) OK |
| Read Command AT+COLP? | Response +COLP: <n>,<m> OK |
| Write Command AT+COLP[=<n>] | Response TA enables or disables the presentation of the COL (Connected Line) at the TE for a mobile originating a call. It has no effect on the execution of the supplementary service COLR in the network. Intermediate result code is returned from TA to TE before any +CR or V.25ter responses. OK |
| Maximum Response Time | 15s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	Parameter sets/shows the result code presentation status in the TA

0	Disable

1	Enable

<m>	Parameter shows the subscriber COLP service status in the network

0	COLP not provisioned

- COLP provisioned

- Unknown (e.g. no network, etc.)

<number>	Phone number in string type, format specified by <type>

<type>	Type of address octet in integer format

129	Unknown type (IDSN format number)

145	International number type (ISDN format )

<subaddr>	String type sub-address of format specified by <satype>

<satype>	Type of sub-address octet in integer format (refer to 3GPP TS 24.008 subclause 10.5.4.8)

<alpha>	Optional string type alphanumeric representation of <number> corresponding to the entry found in phone book

Intermediate result code:

When enabled (and called subscriber allows), an intermediate result code is returned before any +CR or V.25ter responses:

+COLP: <number>,<type>,[<subaddr>],[<satype>],[<alpha>]

NOTE

Example

AT+CPBW=1,“02151082965”,129,“QUECTEL” OK

AT+COLP=1 OK

ATD02151082965;

+COLP: “02151082965”,129,,,“QUECTEL”

OK

## AT+CSSN	Supplementary Service Notifications

The  command  refers  to  supplementary  service  related  network  initiated  notifications.  The  Write Command enables/disables the presentation of notification result codes from TA to TE.

| AT+CSSN Supplementary Service Notifications |  |
| --- | --- |
| Test Command AT+CSSN=? | Response +CSSN: (list of supported <n>s),(list of supported <m>s) OK |
| Read Command AT+CSSN? | Response +CSSN: <n>,<m> OK |
| Write Command AT+CSSN=<n>[,<m>] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	Integer type. Sets/shows the +CSSI intermediate result code presentation status to the TE

0	Disable

1	Enable

<m>	Integer type. Sets/shows the +CSSU unsolicited result code presentation status to the TE

0	Disable

1	Enable

<code1>	Integer type. It is manufacturer specific and supports the following codes:

0	Unconditional call forwarding is active

- Some of the conditional call forwardings are active

- Call has been forwarded

- Waiting call is pending

5	Outgoing call is barred

<code2>	Integer type. It is manufacturer specific and supports the following codes:

0	The incoming call is a forwarded call

- Call has been put on hold (during a voice call)

- Call has been retrieved (during a voice call)

5	Held call was terminated by another party

10	Additional incoming call forwarded

- When <n>=1 and a supplementary service notification is received after a mobile originated call setup, the +CSSI intermediate result code is sent to TE before any other MO call setup result codes:

+CSSI: <code1>

- When <m>=1 and a supplementary service notification is received during a mobile terminated call setup or during a call, the +CSSU unsolicited result code is sent to TE:

+CSSU: <code2>

NOTES

## AT+CUSD	Unstructured Supplementary Service Data

The command allows control of the Unstructured Supplementary Service Data (USSD) according to

3GPP TS 22.090. Both network and mobile initiated operations are supported.

Parameter <mode> is used to disable/enable the presentation of an unsolicited result code. The value

<mode>=2 is used to cancel an ongoing USSD session. For an USSD response from the network, or a network initiated operation, the format is: +CUSD: <status>[,<rspstr>,[<dcs>]].

When <reqstr> is given, a mobile initiated USSD string or a response USSD string to a network initiated operation is sent to the network. The response USSD string from the network is returned in a subsequent

+CUSD URC.

| AT+CUSD Unstructured Supplementary Service Data |  |
| --- | --- |
| Test Command AT+CUSD=? | Response +CUSD: (list of supported <mode>s) OK |
| Read Command AT+CUSD? | Response +CUSD: <mode> OK |
| Write Command AT+CUSD[=<mode>[,<reqstr>[,<dcs> ]]] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 120s, determined by network. |
| Reference 3GPP TS 27.007 |  |

Parameter

<mode>	Integer type. Sets/shows the result code presentation status to the TE

0	Disable the result code presentation to the TE

- Enable the result code presentation to the TE

- Cancel session (not applicable to Read Command response)

<reqstr>	Unstructured Supplementary Service  Data  (USSD)  to  be  sent to  the network. If  this parameter is not given, network is not interrogated.

<rspstr>	Unstructured Supplementary Service Data (USSD) received from the network

<dcs>	Integer type. 3GPP TS 23.038 Cell Broadcast Data Coding Scheme (default 15)

<status>	USSD response from the network or the network initiated operation

0		No  further  user  action  required  (network  initiated  USSD  Notify,  or  no  further information needed after mobile initiated operation)

- Further  user  action  required  (network  initiated  USSD  Request,  or  further information needed after mobile initiated operation)

- USSD terminated by network

- Another local client has responded

- Operation not supported

- Network time out

# Audio	Commands

## AT+CLVL	Loudspeaker Volume Level Selection

The command is used to select the volume of the internal loudspeaker of the MT.

| AT+CLVL Loudspeaker Volume Level Selection |  |
| --- | --- |
| Test Command AT+CLVL=? | Response +CLVL: (list of supported <level>s) OK |
| Read Command AT+CLVL? | Response +CLVL: <level> OK |
| Write Command AT+CLVL=<level> | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<level>	Integer type value (0-3-5) with manufacturer specific range (Smallest value represents the lowest sound level)

This parameter will not be saved.

NOTE

## AT+CMUT	Mute Control

The command is used to enable/disable the uplink voice muting during a voice call.

| AT+CMUT Mute Control |  |
| --- | --- |
| Test Command AT+CMUT=? | Response +CMUT: (list of supported <n>s) OK |
| Read Command AT+CMUT? | Response +CMUT: <n> OK |
| Write Command AT+CMUT=<n> | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<n>	0	Mute OFF

1	Mute ON

This parameter will not be saved and must be set during the call.

NOTE

## AT+QAUDLOOP	Enable/Disable Audio Loop Test

The command is used to enable/disable audio loop test.

| AT+QAUDLOOP Enable/Disable Audio Loop Test |  |
| --- | --- |
| Test Command AT+QAUDLOOP=? | Response +QAUDLOOP: (0,1) |

|  | OK |
| --- | --- |
| Read Command AT+QAUDLOOP? | Response +QAUDLOOP: <enable> OK |
| Write Command AT+QAUDLOOP=<enable> | Response OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<enable>	Numeric type. To enable or disable audio loop test

0	Disable audio loop test

1	Enable audio loop test

This parameter will not be saved.

NOTE

## AT+VTS	DTMF and Tone Generation

The command is used to send ASCII characters which cause MSC to transmit DTMF tones to a remote subscriber. It can only be operated in voice call.

| AT+VTS DTMF and Tone Generation |  |
| --- | --- |
| Test Command AT+VTS=? | Response +VTS: (0-9,A-D,*,#),(0-255) OK |
| Write Command AT+VTS=<dtmfstring>[,<duration>] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | Depends on the length of <dtmfstring> and <duration>. |

Reference 3GPP TS 27.007

Parameter

<dtmfstring>		ASCII characters in the set 0...9, #, *, A, B, C, D. The string should be enclosed in quotation marks ("...").

When  sending  multiple  tones  at  a  time,  the  time  interval  of  two  tones

<interval> can be specified by AT+VTD. The maximal length of the string is 31.

<duration>	The duration of each tone in 1/10 seconds with tolerance.

The value ranges from 0 to 255.

If the duration is less than the minimum time specified by the network, the actual duration will be the network specified time.

If this parameter is omitted, <duration> is specified by AT+VTD.

Example

ATD12345678900;

OK

//Dial

<Call connect>

AT+VTS="1" OK

AT+VTS="1234567890A"

OK

//The remote caller can hear the DTMF tone

//Send multiple tones at a time

## AT+VTD	Set Tone Duration

The command sets the duration of DTMF tones. It can also set time interval of two tones when sending multiple tones at a time.

| AT+VTD Set Tone Duration |  |
| --- | --- |
| Test Command AT+VTD=? | Response +VTD: (0-255),(0-255) OK |
| Read Command AT+VTD? | Response +VTD: <duration>,<interval> OK |

| Write Command AT+VTD=<duration>[,<interval>] | Response OK ERROR If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<duration>	The duration tone in 1/10 seconds with tolerance. The value ranges from 0 to 255, and the default is 3. If the duration is less than the minimum time specified by the network, the actual duration will be network specified time.

<interval>	The time interval of two tones when sending multiple tones at a time by

AT+VTS. The value ranges from 0 to 255, and the default is 0.

These parameters will not be saved.

NOTE

## AT+QAUDMOD	Set Audio Mode

The command sets the audio mode required for the connected device. It will take effect at next sound activity.

| AT+QAUDMOD Set Audio Mode |  |
| --- | --- |
| Test Command AT+QAUDMOD=? | Response +QAUDMOD: (list of supported <mode>s) OK |
| Read command AT+QAUDMOD? | Response +QAUDMOD: <mode> OK |
| Write Command AT+QAUDMOD=<mode> | Response OK |

|  | If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference Quectel |  |

Parameter

<mode>	Numeric type. Indicates the current configured audio mode

0	Echo canceller, noise suppressor, digital gain and calibration parameter for Handset

- Echo canceller, noise suppressor, digital gain and calibration parameter for Headset

- Echo canceller, noise suppressor, digital gain and calibration parameter for Speaker

## AT+QDAI	Digital Audio Interface Configuration

The command is used to configure the digital audio interface.

- When <io>=1, customers can define the PCM formats by themselves. In the following conditions, the module can be used directly with default settings (master mode, short-synchronization, 2048K clock frequency, 16-bit liner data format, 8K sampling rate).

- When <io>=2, and the external codec chip linked with PCM interface is the NAU8814 model and configurable through the I2C

- when <io>=3, and the external codec chip linked with PCM interface is the ALC5616 model and configurable through the I2C

- when <io>=5, and the external codec chip linked with PCM interface is the TLV320AIC3104 model and configurable through the I2C

| AT+QDAI Digital Audio Interface Configuration |  |
| --- | --- |
| Test Command AT+QDAI=? | Response +QDAI: (list of supported <io>s),(list of supported <mode>s),(list of supported <fsync>s),(list of supported <clock>s),(list of supported <format>s),(list of supported <sample>s),(list of supported <num_slots>s),(list of supported <slot_mapping>s) OK |
| Read Command AT+QDAI? | Response +QDAI: <io>[,<mode>,<fsync>,<clock>,<format>,<sample>,<num _slots>,<slot_mapping>] |

|  | OK |
| --- | --- |
| Write Command AT+QDAI=<io>[,<mode>,<fsync>,<clo ck>[,<format>[,<sample>[,<num_slot s>,<slot_mapping>]]]] | Response OK ERROR |
| Maximum Response Time | 300ms |
| Reference Quectel |  |

| Parameter |  |  |
| --- | --- | --- |
| <io> | 1 | Digital PCM output (customer defined) |
|  | 2 | Analog output (for audio codec NAU8814) |
|  | 3 | Analog output (for our default audio codec ALC5616) |
|  | 5 | Analog output (for audio codec TLV320AIC3104) |
| <mode> | 0 | Master mode |
|  | 1 | Slave mode |
| <fsync> | 0 | Primary mode (short-synchronization) |
|  | 1 | Auxiliary mode (long-synchronization) |

<clock>	Clock frequency 0	128K

1	256K

2	512K

3	1024K

4	2048K

5	4096K

<format>	Data format

|  | 0 | 16-bit linear |
| --- | --- | --- |
|  | 1 | 8-bit a-law |
|  | 2 | 8-bit u-law |
| <sample> | 0 | 8K |
|  | 1 | 16K |
| <num_slots> | 1 | Number of slot |

<slot_mapping> Slot mapping value. The range is 1-16.

- The parameter settings will be saved to NV immediately by default and will take effect after the module is reset.

- 4096K clock frequency is only applicable for 16K sampling rate.

- 128K clock frequency is not supported.

NOTES

- 8-bit a-law and 8-bit u-law data formats are not supported.

- Bit per frame=<clock>/<sample>. For example, if <clock> is 2048K and <sample> is 8K, then bit per frame is 256. Bit per frame should be greater than 16.

- When slave mode is selected, master and synchronization clock should be provided for the module.

- When a recommended codec is selected and 16K sampling rate is desired, please input <sample>. Currently only ALC5616 supports 16K (AT+QDAI=3,0,0,5,0,1,1,1).

Example

AT+QDAI=?

//Query the range.

+QDAI: (1-3,5),(0,1),(0,1),(0-5),(0-2),(0,1),(1),(1-16)

OK

AT+QDAI?

+QDAI: 3,0,0,4,0,0,1,1

//Query the current interface configuration.

OK

AT+QDAI=1,1,0,4,0,0,1,1

//Set AUX PCM interface to slave, short-synchronization, 8K sample and

2048K BCLK.

OK

## AT+QEEC	Set Echo Cancellation Parameters

The command is used to set echo cancellation parameters.

| AT+QEEC Set Echo Cancellation Parameters |  |
| --- | --- |
| Test Command AT+QEEC=? | Response +QEEC: (0-50),(0-65535) OK |
| Read Command AT+QEEC? | Response +QEEC: <index>,<value> ...... +QEEC: <index>,<value> OK |
| Write Command AT+QEEC=<index>,<value> | Response OK ERROR |

Parameter

<index>	Numeric type. Indicates the parameter‟s index.

Range: 0-50

<value>	Numeric type. Indicates the parameter‟s value.

Range: 0-65535

These parameters will not be saved.

NOTE

Example

AT+QEEC=?

//Query the range.

+QEEC: (0-50), (0-65535)

OK

AT+QEEC=6,1234	//Set the value of index 6 to 1234.

OK

## AT+QSIDET	Set the Side Tone Gain in Current Mode

The command is used to set the side tone gain value in current mode. It will take effect at next sound activity.

| AT+QSIDET Set the Side Tone Gain in Current Mode |  |
| --- | --- |
| Test Command AT+QSIDET=? | Response +QSIDET: (list of supported <st_gain>s) OK |
| Read Command AT+QSIDET? | Response +QSIDET: <st_gain> OK |
| Write Command AT+QSIDET=<st_gain> | Response OK ERROR |
| Maximum Response Time | 300ms |

Reference Quectel

Parameter

<st_gain>	Numeric type. Indicates the configured side tone gain in current mode Range: 0-65535. Default value is 0.

This parameter will not be saved.

NOTE

## AT+QMIC	Set Uplink Gains of MIC

The command is used to set the uplink gains of microphone.

| AT+QMIC Set Uplink Gains of MIC |  |
| --- | --- |
| Test Command AT+QMIC=? | Response +QMIC: (0-65535),(0-65535) OK |
| Read Command AT+QMIC? | Response +QMIC: <txgain>,<txdgain> OK |
| Write Command AT+QMIC=<txgain>[,<txdgain>] | Response OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<txgain>	Numeric type. Indicates uplink codec gain and the range is 0-65535. The default value might be different in different audio modes.

<txdgain>	Numeric type. Indicates uplink digital gain and the range is 0-65535. The default value might be different in different audio modes.

These parameters will not be saved.

NOTE

## AT+QRXGAIN	Set Downlink Gains of RX

The command is used to set RX digital gains to change the downlink volume.

| AT+QRXAGIN Set Downlink Gains of RX |  |
| --- | --- |
| Test Command AT+QRXGAIN=? | Response +QRXGAIN: (0-65535) OK |
| Read Command AT+QRXGAIN? | Response +QRXGAIN: <rxgain> OK |
| Write Command AT+QRXGAIN=<rxgain> | Response OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<rxgain>	Numeric type. Indicates downlink digital gains. The range is 0-65535. The default value might be different in different audio modes.

Example

AT+QRXGAIN=?

+QRXGAIN: (0-65535)

//Test command.

OK

AT+QRXGAIN?

//Query the current value. The default value might be different in

different audio modes.

+QRXGAIN: 20577

OK

AT+QRXGAIN=8192

OK AT+QRXGAIN?

+QRXGAIN: 8192

//Set digital gain to 8192.

//Query the current configuration.

OK

## AT+QIIC	IIC Read & Write

The command is used to configure the codec via IIC interface.

| AT+QIIC IIC Read & Write |  |
| --- | --- |
| Test Command AT+QIIC=? | Response +QIIC: (0,1),(0~0xff),(0~0xff),(1,2),(0~0xffff) OK |
| Write Command AT+QIIC=<rw>,<device>,<addr>,<byt es>[,<value>] | If all configuration parameters are entered: Response OK If all configuration parameters are omitted: Response +QIIC: <value> OK |
| Maximum Response Time | 300ms |

Parameter

| <rw> | 0 | Write command |
| --- | --- | --- |
|  | 1 | Read command |
| <device> | 0-0xFF | 7-bit device address |
| <addr> | 0-0xFF | Register address |
| <bytes> | 1-2 | Read/write bytes |
| <value> | 0-0xFFFF | Data value |

These parameters will not be saved.

NOTE

Example

AT+QIIC=1,0x1B,0x00,2

//Read register value, slave address: 0x1B, register address:

0x00, read two bytes.

+QIIC: 0x0021

OK

AT+QIIC=0,0x1B,0x00,2,0x0000

//Write register value, slave address: 0x1B, register address:

0x00, write two bytes.

OK

## AT+QTONEDET	Enable/Disable DTMF Detection

The command is used to enable or disable DTMF detection. When this function is enabled, DTMF tones sent by other side will be detected, and it will be reported on the assigned serial port.

| AT+ QTONEDET Enable/Disable DTMF Detection |  |
| --- | --- |
| Test Command AT+QTONEDET=? | Response +QTONEDET: (0,1) OK |
| Read Command AT+QTONEDET? | Response +QTONEDET: <enable> OK |
| Write Command AT+QTONEDET=<enable> | Response OK ERROR |
| Maximum Response Time | 300ms |

Parameter

<enable>	Enable or disable DTMF detection

0	Disable

1	Enable

1.	This setting will take effect immediately. And it will revert to the default values after resetting the module.

NOTES

|  | 2. DTMF characters - ASCII: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DTMF | ASCII | DTMF | ASCII |  |  |
|  | 0 | 48 8 56 |  |  |  |  |
|  | 1 | 49 9 57 |  |  |  |  |
|  | 2 | 50 A 65 |  |  |  |  |
|  | 3 | 51 B 66 |  |  |  |  |
|  | 4 | 52 C 67 |  |  |  |  |
|  | 5 | 53 D 68 |  |  |  |  |
|  | 6 | 54 * 42 |  |  |  |  |
|  | 7 | 55 # 35 |  |  |  |  |

## AT+QLDTMF	Play Local DTMF

The command is used to play a local DTMF string, and the maximum length is 20 characters. It can be used to stop playing the DTMF string.

| AT+QLDTMF Play Local DTMF |  |
| --- | --- |
| Test Command AT+QLDTMF=? | Response +QLDTMF: (1-1000),(0-9,*,#,A-D ) OK |
| Write Command AT+QLDTMF=<n>,<DTMF_string>[,<y >] | Response OK +CME ERROR: <err> After the DTMF string is completely played: +QLDTMF: 5 |
| Execute Command AT+QLDTMF | Response OK |
| Maximum Response Time | 300ms |

Parameter

<n>	Numeric type. Indicates every DTMF‟s play time and mute time. The range is 1

-1000, and the unit is 1/100 second when <y> is set to 1, or 1/10 second when <y> is not set.

<DTMF_string>	String type. Maximum 20 DTMF strings, separated by comma. DTMF format: 0-9,

*, #, A-D.

These parameters will not be saved.

NOTE

Example

AT+QLDTMF=?	//Query the range.

+QLDTMF: (1-1000),(0-9,*,#,A-D)

OK

AT+QLDTMF=2,“A,B,1,2,#”

//Play A,B,1,2,#, on time & mute time is 200ms.

OK

AT+QLDTMF OK

//Stop playing.

## AT+QLTONE	Play a Local Customized Tone

The command is used to play a local customized tone, use <period_on> to indicate play time and

<period_off> to indicate mute time, and <duration> to indicate total time.

| AT+QLTONE Play a Local Customized Tone |  |
| --- | --- |
| Test Command AT+QLTONE=? | Response +QLTONE: (0,1),(100-4000),(0-1000),(0-1000),(0-15300000) OK |
| Write Command AT+QLTONE=<mode>[,<frequency>, <period_on>,<period_off>,<duration> ] | Response OK +CME ERROR: <err> After the tone is completely played: +QLTONE: 0 |
| Maximum Response Time | 300ms |

Parameter

<mode>	0	Stop playing

1	Start to play

<frequency>	Tone‟s frequency. The range is 100-4000, the unit is Hz.

<period_on>	Tone‟s play time. The range is 0-1000, the unit is ms.

<period_off>	Tone‟s mute time. The range is 0-1000, the unit is ms.

<duration>	Tone‟s total time. The ranges is 0-15300000, the unit is ms.

These parameters will not be saved.

NOTE

Example

AT+QLTONE=?

//Query the range.

+QLTONE: (0,1),(100-4000),(0-1000),(0-1000),(0-15300000)

OK

AT+QLTONE=1,1000,200,300,3000

//Play a 1000Hz tone, play time is 200ms and mute time is

300ms. Total time is 3000ms.

OK

+QLTONE: 0

AT+QLTONE=0 OK

//Stop playing.

# Hardware Related Commands

## AT+QPOWD   Power off

The command is used to shut down the module. The UE will return OK immediately when the command is executed. Then the UE deactivates the network. After it is completed, the UE outputs POWERED DOWN and enters into the shutdown state. The maximum time for unregistering network is 60 seconds. The UE is not allowed to turn off the power before the module‟s STATUS pin is set low or the URC POWERED DOWN is outputted to avoid data loss.

| AT+QPOWD Power off |  |
| --- | --- |
| Test Command AT+QPOWD=? | Response +QPOWD: (0,1) OK |
| Execution Command AT+QPOWD[=<n>] | Response OK POWERED DOWN |
| Maximum Response Time | 300ms |

Parameter

<n>	0	Immediately power down

1	Normal power down

## AT+CCLK	Clock

The command sets and queries the real time clock (RTC) of the module. The current setting is retained until the module is totally disconnected from power.

| AT+CCLK Clock |  |
| --- | --- |
| Test Command AT+CCLK=? | Response OK |

| Read Command AT+CCLK? | Response +CCLK: <time> OK |
| --- | --- |
| Write Command AT+CCLK=<time> | Response OK If there is any error related to ME functionality: +CME ERROR: <err> |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<time>  String type value. The format is “yy/MM/dd,hh:mm:ss±zz”, indicating year (two last digits), month, day, hour, minutes, seconds and time zone (indicates the difference, expressed in quarters of an hour, between the local time and GMT; range -48...+56). E.g. May 6th, 1994, 22:10:00 GMT+2 hours equals to “94/05/06,22:10:00+08”.

Example

AT+CCLK?

+CCLK: “08/01/04,00:19:43+00”

//Query the local time

OK

## AT+CBC	Battery Charge

The command returns battery charge status <bcs> and battery charge level <bcl> of the MT.

| AT+CBC Battery Charge |  |  |  |
| --- | --- | --- | --- |
| Test Command AT+CBC=? | Response +CBC: (list of supported <bcs>s),(list <bcl>s),<voltage> OK | of | supported |
| Execution Command AT+CBC | Response +CBC: <bcs>,<bcl>,<voltage> |  |  |

|  | OK If there is any error related to ME functionality: +CME ERROR: <err> |
| --- | --- |
| Maximum Response Time | 300ms |
| Reference 3GPP TS 27.007 |  |

Parameter

<bcs>	Battery charge status

0	ME is not charging

- ME is charging

- Charging has been finished

<bcl>	Battery charge level

0-100	Battery has 0-100 percent of capacity remaining vent

<voltage>	Battery voltage (Mv)

## AT+QADC	Read ADC Value

The command is used to read the voltage value of ADC channel.

| AT+QADC Read ADC Value |  |
| --- | --- |
| Test Command AT+QADC=? | Response +QADC: (0,1) OK |
| Read Command AT+QADC=<port> | Response +QADC: <status>,<value> OK |
| Maximum Response Time | 300ms |

Parameter

<port>	Channel number of the ADC

0	ADC Channel 0

1	ADC Channel 1

<status>	Indicate whether the ADC value is read successfully

0	Fail

1	Success

<value>	The voltage of specified ADC channel. Unit is mV.

## AT+QSCLK   Enable/Disable Entering into Sleep Mode

The command is used to control whether the module enters into sleep mode. When entering into sleep mode is enabled, DTR is pulled up and WAKEUP_IN is pulled up, the module can directly enter into sleep mode. If entering into sleep mode is enabled, DTR is pulled down and WAKEUP_IN is pulled down, there is a need to pull the DTR pin and the WAKEUP_IN pin up first, and then the module can enter into sleep mode.

| AT+QSCLK Enable/Disable Entering into Sleep Mode |  |
| --- | --- |
| Test Command AT+QSCLK=? | Response +QSCLK: (list of supported <n>s) OK |
| Read Command AT+QSCLK? | Response +QSCLK: <n> OK |
| Write Command AT+QSCLK=<n> | Response OK |
| Maximum Response Time | 300ms |
| Reference Quectel |  |

Parameter

<n>	0	Disable slow clock

1	Enable slow clock. It is controlled by DTR

# DFOTA	Related	AT	Command

Please refer to Quectel_EC2x&EG9x&EM05_DFOTA_User_Guide for details of DFOTA function.

#### Table 6: DFOTA Related AT Command

Command	Description

AT+QFOTADL	Upgrade firmware via DFOTA

# FTP(S)	Related	AT	Commands

Please  refer  to  Quectel_EC2x&EG9x&EM05_FTP(S)_AT_Commands_Manual  for  details  of  FTP(S) function.

#### Table 7: FTP(S) Related AT Commands

Command	Description

AT+QFTPCFG	Configure parameters for FTP(S) server

AT+QFTPOPEN	Login to FTP(S) server

AT+QFTPCWD	Configure the current directory on FTP(S) server

AT+QFTPPWD	Get the current directory on FTP(S) server

AT+QFTPPUT	Upload a file to FTP(S) server

AT+QFTPGET	Download a file from FTP(S) server

AT+QFTPSIZE	Get the file size on FTP(S) server

AT+QFTPDEL	Delete a file on FTP(S) server

AT+QFTPMKDIR	Create a folder on FTP(S) server

AT+QFTPRMDIR	Delete a folder on FTP(S) server

AT+QFTPLIST	List content of a directory on FTP(S) server

AT+QFTPNLIST	List file names of a directory on FTP(S) server

AT+QFTPMLSD	List standardized file and directory information

AT+QFTPMDTM	Get the file modification time on FTP(S) server

AT+QFTPRENAME	Rename a file or folder on FTP(S) server

AT+QFTPLEN	Get the length of transferred data

AT+QFTPSTAT	Get the status of FTP(S) server

AT+QFTPCLOSE	Log out from FTP(S) server

# HTTP(S)	Related	AT	Commands

Please refer to Quectel_EC2x&EG9x&EM05_HTTP(S)_AT_Commands_Manual for details of HTTP(S) function.

#### Table 8: HTTP(S) Related AT Commands

Command	Description

AT+QHTTPCFG	Configure parameters for HTTP(S) server

AT+QHTTPURL	Set URL of HTTP(S) server

AT+QHTTPGET	Send GET request to HTTP(S) server

AT+QHTTPGETEX	Send range GET request to HTTP(S) server

AT+QHTTPPOST	Send POST request to HTTP(S) server via UART/USB

AT+QHTTPPOSTFILE	Send POST request to HTTP(S) server via file

AT+QHTTPREAD	Read response from HTTP(S) server via UART/USB

AT+QHTTPREADFILE	Read response from HTTP(S) server via file

AT+QHTTPSTOP	Cancel HTTP(S) request

# MMS	Related	AT	Commands

Please refer to Quectel_EC2x&EG9x&EM05_MMS_AT_Commands_Manual for details of MMS function.

#### Table 9: MMS Related Commands

Command	Description

AT+QMMSCFG	Configure parameters of MMS

AT+QMMSEDIT	Edit MMS messages

AT+QMMSEND	Send MMS messages

# SMTP	Related	AT	Commands

Please  refer  to  Quectel_EC2x&EG9x&EM05_SMTP_AT_Commands_Manual  for  details  of  SMTP function.

#### Table 10: SMTP Related AT Commands

Command	Description

AT+QSMTPCFG	Configure parameters for SMTP server

AT+QSMTPDST	Add or delete recipients

AT+QSMTPSUB	Edit the subject of an Email

AT+QSMTPBODY	Edit the body of an Email

AT+QSMTPATT	Add or delete attachments for an Email

AT+QSMTPCLR	Clear the content of an Email

AT+QSMTPPUT	Send an Email

# TCP(IP)	Related	AT	Commands

Please refer to Quectel_EC2x&EG9x&EM05_TCP(IP)_AT_Commands_Manual for details of TCP(IP) function.

#### Table 11: TCP(IP) Related AT Commands

Command	Description

AT+QICSGP	Configure parameters of a TCP/IP context

AT+QIACT	Activate a PDP context

AT+QIDEACT	Deactivate a PDP context

AT+QIOPEN	Open a socket service

AT+QICLOSE	Close a socket service

AT+QISTATE	Query socket service status

AT+QISEND	Send data

AT+QIRD	Retrieve the received TCP/IP data

AT+QISENDEX	Send hex string

AT+QISWTMD	Switch data access modes

AT+QPING	Ping a remote server

AT+QNTP	Synchronize local time with NTP server

AT+QIDNSCFG	Configure address of DNS server

AT+QIDNSGIP	Get IP address by domain name

AT+QICFG	Configure optional parameters

AT+QISDE	Control whether to echo the data for AT+QISEND

AT+QIGETERROR	Query the last error code

# GNSS	Related	AT	Commands

Please refer to Quectel_EC25&EC21_GNSS_AT_Commands_Manual for details of GNSS function.

#### Table 12: GNSS Related AT Commands

Command	Description

AT+QGPSCFG	GNSS configurations

AT+QGPSDEL	Delete assistance data

AT+QGPS	Turn on GNSS

AT+QGPSEND	Turn off GNSS

AT+QGPSLOC	Acquire positioning information

AT+QGPSGNMEA	Acquire NMEA sentences

AT+QGPSXTRA	Enable gpsOneXTRA assistance function

AT+QGPSXTRATIME	Inject gpsOneXTRA time

AT+QGPSXTRADATA	Inject gpsOneXTRA data file

# Appendix

## References

#### Table 13: Related Documents

SN	Document Name	Remark

[1]	V.25ter	Serial asynchronous automatic dialing and control

[2]	3GPP TS 27.007

[3]	3GPP TS 27.005

Quectel_EC2x&EG9x&EM05_

[4]

DFOTA_User_Guide

Quectel_EC2x&EG9x&EM05_

Digital cellular telecommunications (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; AT command set for User Equipment (UE)

Digital cellular telecommunications (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Use of Data Terminal Equipment – Data Circuit terminating Equipment (DTE-DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)

EC2x&EG9x&EM05 DFOTA User Guide

[5]

[6]

[7]

[8]

[9]

FTP(S)_AT_Commands_Manu al Quectel_EC2x&EG9x&EM05_ HTTP(S)_AT_Commands_Ma

nual Quectel_EC2x&EG9x&EM05_ MMS_AT_Commands_Manual Quectel_EC2x&EG9x&EM05_ SMTP_AT_Commands_Manu al Quectel_EC2x&EG9x&EM05_ TCP(IP)_AT_Commands_Man ual

EC2x&EG9x&EM05 FTP(S) AT Commands Manual

EC2x&EG9x&EM05 HHTP(S) AT Commands Manual

EC2x&EG9x&EM05 MMS AT Commands Manual

EC2x&EG9x&EM05 SMTP AT Commands Manual

EC2x&EG9x&EM05 TCP(IP) AT Commands Manual

[10]	Quectel_EC25&EC21_GNSS_	EC2x&EG9x&EM05 GNSS AT Commands Manual

AT_Commands_Manual

#### Table 14: Terms and Abbreviations

Abbreviation	Description

AMR	Adaptive Multi-Rate

CSD	Circuit Switch Data

DCD	Dynamic Content Delivery

DCE	Data Communication Equipment

DTE	Data Terminal Equipment

DTR	Data Terminal Ready

ECT	Explicit Call Transfer supplementary service

GPRS	General Packet Radio Service

ME	Mobile Equipment

MS	Mobile Station

PDP	Packet Data Protocol

PSC	Primary Synchronization Code

RTS/CTS	Request To Send/Clear To Send

TA	Terminal Adapter

TCP	Transmission Control Protocol

TE	Terminal Equipment

UDP	User Datagram Protocol

UE	User Equipment

NV	Non-Volatile Random Access Memory

## Factory Default Settings Restorable with AT&F

#### Table 15: Factory Default Settings Restorable with AT&F

| AT Command | Parameters | Factory Defaults |
| --- | --- | --- |
| ATE | <value> | 1 |
| ATQ | <n> | 0 |
| ATS0 | <n> | 0 |
| ATS3 | <n> | 13 |
| ATS4 | <n> | 10 |
| ATS5 | <n> | 8 |
| ATS6 | <n> | 2 |
| ATS7 | <n> | 0 |
| ATS8 | <n> | 2 |
| ATS10 | <n> | 15 |
| ATV | <value> | 1 |
| ATX | <value> | 4 |
| AT&C | <value> | 1 |
| AT&D | <value> | 2 |
| AT+CREG | <n> | 0 |
| AT+CGREG | <n> | 0 |
| AT+CBST | <speed>,<name>,<ce> | 0,0,1 |
| AT+CMEE | <n> | 1 |
| AT+CSCS | <chset> | “GSM” |
| AT+CSTA | <type> | 129 |
| AT+CR | <mode> | 0 |

| AT+CRC | <mode> | 0 |
| --- | --- | --- |
| AT+CSMS | <service> | 0 |
| AT+CMGF | <mode> | 0 |
| AT+CSMP | <fo>,<vp>,<pid>,<dcs> | 17,167,0,0 |
| AT+CSDH | <show> | 0 |
| AT+CSCB | <mode> | 0 |
| AT+CPMS | <mem1>,<mem2>,<mem3> | “ME”,“ME”,“ME” |
| AT+CNMI | <mode>,<mt>,<bm>,<ds>,<bfr> | 2,1,0,0,0 |
| AT+CMMS | <n> | 0 |
| AT+CVHU | <mode> | 0 |
| AT+CLIP | <n> | 0 |
| AT+COLP | <n> | 0 |
| AT+CLIR | <n> | 0 |
| AT+CSSN | <n> | 0 |
| AT+CTZR | <reporting> | 0 |
| AT+CPBS | <storage> | ME |
| AT+CGEREP | <mode>,<brf> | 0,0 |
| AT+CEREG | <n> | 0 |
| AT+CCWA | <n> | 0 |
| AT+CUSD | <mode> | 0 |
| AT+CLVL | <level> | 3 |
| AT+QAUDMOD | <mode> | 0 |
| AT+QAUDLOOP | <enable> | 0 |

## AT Command Settings Storable with AT&W

#### Table 16: AT Command Settings Storable with AT&W

| AT Command | Parameters | Display with AT&V |
| --- | --- | --- |
| ATE | <value> | Yes |
| ATQ | <n> | Yes |
| ATS0 | <n> | Yes |
| ATS7 | <n> | Yes |
| ATS10 | <n> | Yes |
| ATV | <value> | Yes |
| ATX | <value> | Yes |
| AT&C | <value> | Yes |
| AT&D | <value> | Yes |
| AT+IPR | <rate> | No |
| AT+CREG | <n> | No |
| AT+CGREG | <n> | No |
| AT+CEREG | <n> | No |

## AT Command Settings Storable with ATZ

#### Table 17: AT Command Settings Storable with ATZ

| AT Command | Parameters | Factory Defaults |
| --- | --- | --- |
| ATE | <value> | 1 |
| ATQ | <n> | 0 |

| ATS0 | <n> | 0 |
| --- | --- | --- |
| ATS7 | <n> | 0 |
| ATS10 | <n> | 15 |
| ATV | <value> | 1 |
| ATX | <value> | 4 |
| AT&C | <value> | 1 |
| AT&D | <value> | 2 |
| AT+CREG | <n> | 0 |
| AT+CGREG | <n> | 0 |
| AT+CEREG | <n> | 0 |

## Summary of CME ERROR Codes

Final result code +CME ERROR: <err> indicates an error related to mobile equipment or network. The operation is similar to ERROR result code. None of the following commands in the same command line is executed. Neither ERROR nor OK result code shall be returned.

<err> values are mostly used by common message commands. The following table lists most of general and GRPS related ERROR codes. For some GSM protocol failure cause described in GSM specifications, the corresponding ERROR codes are not included.

#### Table 18: Different Coding Schemes of +CME ERROR: <err>

Code of <err>	Meaning

0	Phone failure

- No connection to phone

- Phone-adaptor link reserved

- Operation not allowed

- Operation not supported

- PH-SIM PIN required

- PH-FSIM PIN required

- PH-FSIM PUK required

- SIM not inserted

- SIM PIN required

- SIM PUK required

- SIM failure

- SIM busy

- SIM wrong

- Incorrect password

- SIM PIN2 required

- SIM PUK2 required

- Memory full

- Invalid index

- Not found

- Memory failure

- Text string too long

- Invalid characters in text string

- Dial string too long

- Invalid characters in dial string

- No network service

- Network timeout

- Network not allowed - emergency calls only

- Network personalization PIN required

- Network personalization PUK required

- Network subset personalization PIN required

- Network subset personalization PUK required

- Service provider personalization PIN required

- Service provider personalization PUK required

- Corporate personalization PIN required

- Corporate personalization PUK required

901	Audio unknown error

902	Audio invalid parameters

903	Audio operation not supported

904	Audio device busy

## Summary of CMS ERROR Codes

Final result code +CMS ERROR: <err> indicates an error related to mobile equipment or network. The operation is similar to ERROR result code. None of the following commands in the same command line is executed. Neither ERROR nor OK result code shall be returned.

<err> values are mostly used by common message commands:

#### Table 19: Different Coding Schemes of +CMS ERROR: <err>

Code of <err>	Meaning

- ME failure

- SMS ME reserved

- Operation not allowed

- Operation not supported

- Invalid PDU mode

- Invalid text mode

- SIM not inserted

- SIM pin necessary

- PH SIM pin necessary

- SIM failure

- SIM busy

- SIM wrong

- SIM PUK required

- SIM PIN2 required

- SIM PUK2 required

- Memory failure

- Invalid memory index

- Memory full

- SMSC address unknown

- No network

- Network timeout

500	Unknown

512	SIM not ready

513	Message length exceeds

514	Invalid request parameters

515	ME storage failure

517	Invalid service mode

528	More message to send state error

529	MO SMS is not allow

530	GPRS is suspended

531	ME storage full

## Summary of URC

#### Table 20: Summary of URC

Index	URC Display	Meaning	Condition

1	+CREG: <stat>	Indicate registration status of the ME	AT+CREG=1

+CREG:

2

<stat>[,<lac>,<ci>[,<Act>]]

3	+CGREG: <stat>

+CGREG:

4

<stat>[,<lac>,<ci>[,<Act>]]

After cell neighborhood changing shows whether the network has currently indicated the registration of the ME, with location area code

Indicate network registration status of the ME

Indicate network registration and location information of the ME

AT+CREG=2

AT+CGREG=1 AT+CGREG=2

5	+CTZV: <tz>	Time zone reporting	AT+CTZR=1

6	+CTZE: <tz>,<dst>,<time>	Extended time zone reporting	AT+CTZR=2

7	+CMTI: <mem>,<index>

+CMT:

8	[<alpha>],<length><CR><LF>

<pdu>

+CMT:

<oa>,[<alpha>],<scts>[,<tooa>

9

,<fo>,<pid>,<dcs>,<sca>,<tosc

a>,<length>]<CR><LF><data>

^HCMT:

<oa>,<scts>,<lang>,

10

<fmt>,<length>,<prt>,<prv>,<ty

pe>,<stat><CR><LF><data>

+CBM:

11

<length><CR><LF><pdu>

New message is received, and saved to memory

New short message is received and output directly to TE (PDU mode)

New short message is received and output directly to TE (Text mode)

New short message is received and output directly to TE

New  CBM  is  received  and  output directly (PDU mode)

See AT+CNMI

See AT+CNMI

See AT+CNMI

See AT+CNMI

See AT+CNMI

12	+CBM:	New  CBM  is  received  and  output	See AT+CNMI

<sn>,<mid>,<dcs>,<page>,<p ages><CR><LF><data>

+CDS:

13

<length><CR><LF><pdu>

+CDS:

14	<fo>,<mr>,[<ra>],[<tora>],<sct s>,<dt>,<st>

15	+CDSI: <mem>,<index>

^HCDS:

<oa>,<scts>,<lang>,

16

<fmt>,<length>,<prt>,<prv>,<ty

pe>,<stat><CR><LF><data>

+COLP:

- <number>,<type>,[<subaddr>], [<satype>],[<alpha>]

+CLIP:

directly to TE (Text mode)

New CDS is received and output directly (PDU mode)

New CDS is received and output directly to TE (Text mode)

New message status report is received, and saved to memory

New CDS is received and output directly to TE

The presentation of the COL (connected line) at the TE for a mobile originated call

See AT+CNMI

See AT+CNMI

See AT+CNMI

See AT+CNMI

AT+COLP=1

- <number>,<type>,[subaddr],[s atype],[<alpha>],<CLI validity>

Mobile terminating call indication	AT+CLIP=1

An incoming call is indicated to the

19	+CRING: <type>

+CCWA:

TE	with	unsolicited	result	code instead of the normal RING

AT+CRC=1

20	<number>,<type>,<class>[,<al pha>]

Call waiting indication	AT+CCWA=1,1

Shows the +CSSI intermediate result

21	+CSSI: <code1>

22	+CSSU: <code2>

+CUSD:

23

<status>[,<rspstr>,[<dcs>]]

code presentation status to the TE Shows the +CSSU unsolicited result code presentation status to the TE USSD response from the network, or a network initiated operation

AT+CSSN=1

AT+CSSN=<n>,1 AT+CUSD=1

24	RDY	ME initialization is successful	N/A

25	+CFUN: 1	All function of the ME is available	N/A

26	+CPIN: <state>	SIM card pin state	N/A

27	+QIND: SMS DONE	SMS initialization finished	N/A

28	+QIND: PB DONE	Phonebook initialization finished	N/A

29	POWERED DOWN	Module power down	AT+QPOWD

+CGEV: REJECT

30

<PDP_type>, <PDP_addr>

+CGEV: NW REACT

- <PDP_type>, <PDP_addr>, [<cid>]

+CGEV: NW DEACT

- <PDP_type>, <PDP_addr>, [<cid>]

+CGEV: ME DEACT

- <PDP_type>, <PDP_addr>, [<cid>]

34	+CGEV: NW DETACH

35	+CGEV: ME DETACH

36	+CGEV: NW CLASS <class>

37	+CGEV: ME CLASS <class>

A network request for PDP activation, and was automatically rejected.

The network request PDP reactivation

The  network  has  forced  a  context deactivation

The ME has forced a context deactivation.

The  network  has  forced  a  Packet Domain detach.

The mobile equipment has forced a Packet Domain detach.

The network has forced a change of MS class.

The mobile equipment has forced a change of MS class.

AT+CGEREP=2,1

AT+CGEREP=2,1

AT+CGEREP=2,1

AT+CGEREP=2,1

AT+CGEREP=2,1 AT+CGEREP=2,1 AT+CGEREP=2,1 AT+CGEREP=2,1

## SMS Character Sets Conversions

In 3GPP TS 23.038 DCS (Data Coding Scheme) defined three kinds of alphabets in SMS, GSM 7 bit default alphabet, 8 bit data and UCS2(16bit). AT+CSMP can set the DCS in text mode (AT+CMGF=1). In text mode, DCS (Data Coding Scheme) and AT+CSCS determine the way of SMS text input or output.

#### Table 21: The Way of SMS Text Input or Output

DCS	AT+CSCS	The Way of SMS Text Input or Output

GSM 7 bit	GSM	Input or output GSM character sets.

GSM 7 bit	IRA

GSM 7 bit	UCS2

Input or output IRA character sets.

Input: UE will convert IRA characters to GSM characters. Output: UE will convert GSM characters to IRA characters.

Input or output a hex string similar to PDU mode. So only support characters „0‟-„9‟ and „A‟-„F‟.

Input: UE will convert the UCS2 hex string to GSM characters. Output: UE will convert the GSM characters to UCS2 hex string.

UCS2	-	Ignore the value of AT+CSCS, input or output a hex string similar to

PDU mode. So only support characters „0‟-„9‟ and „A‟-„F‟.

8 bit	-

Ignore the value of AT+CSCS, input or output a hex string similar to PDU mode. So only support characters „0‟-„9‟ and „A‟-„F‟.

When DCS=GSM 7 bit, the input or output needs conversion. The detailed conversion tables are shown as below.

#### Table 22: The Input Conversions Table (DCS=GSM 7 bit and AT+CSCS=“GSM”)

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 00 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 1 | 01 | 11 | 21 | 31 | 41 | 51 | 61 | 71 |
| 2 | 02 | 12 | 22 | 32 | 42 | 52 | 62 | 72 |
| 3 | 03 | 13 | 23 | 33 | 43 | 53 | 63 | 73 |
| 4 | 04 | 14 | 24 | 34 | 44 | 54 | 64 | 74 |
| 5 | 05 | 15 | 25 | 35 | 45 | 55 | 65 | 75 |
| 6 | 06 | 16 | 26 | 36 | 46 | 56 | 66 | 76 |
| 7 | 07 | 17 | 27 | 37 | 47 | 57 | 67 | 77 |
| 8 | 08 | 18 | 28 | 38 | 48 | 58 | 68 | 78 |
| 9 | 09 | 19 | 29 | 39 | 49 | 59 | 69 | 79 |
| A | 0A | Submit | 2A | 3A | 4A | 5A | 6A | 7A |
| B | 0B | Cancel | 2B | 3B | 4B | 5B | 6B | 7B |
| C | 0C | 1C | 2C | 3C | 4C | 5C | 6C | 7C |
| D | 0D | 1A | 2D | 3D | 4D | 5D | 6D | 7D |
| E | 0E | 1E | 2E | 3E | 4E | 5E | 6E | 7E |
| F | 0F | 1F | 2F | 3F | 4F | 5F | 6F | 7F |

#### Table 23: The Output Conversions Table (DCS=GSM 7 bit and AT+CSCS=“GSM”)

0

1

2

3

4

5

6

7

0	00	10	20	30	40	50	60	70

1	01	11	21	31	41	51	61	71

2	02	12	22	32	42	52	62	72

3	03	13	23	33	43	53	63	73

4	04	14	24	34	44	54	64	74

5	05	15	25	35	45	55	65	75

6	06	16	26	36	46	56	66	76

7	07	17	27	37	47	57	67	77

8	08	18	28	38	48	58	68	78

9	09	19	29	39	49	59	69	79

- 0D0A	2A	3A	4A	5A	6A	7A

- 0B	2B	3B	4B	5B	6B	7B

- 0C	1C	2C	3C	4C	5C	6C	7C

- 0D	1A	2D	3D	4D	5D	6D	7D

- 0E	1E	2E	3E	4E	5E	6E	7E

- 0F	1F	2F	3F	4F	5F	6F	7F

#### Table 24: GSM Extended Characters

0	1	2	3	4	5	6	7

0	1B40

1

2

3

4	1B14

| 5 |  |  |
| --- | --- | --- |
| 6 |  |  |
| 7 |  |  |
| 8 | 1B28 |  |
| 9 | 1B29 |  |
| A |  |  |
| B |  |  |
| C |  | 1B3C |
| D |  | 1B3D |
| E |  | 1B3E |
| F | 1B2F |  |

#### Table 25: The Input Conversions Table (DCS=GSM 7 bit and AT+CSCS=“IRA”)

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 20 | 20 | 30 | 00 | 50 | 20 | 70 |
| 1 | 20 | 20 | 21 | 31 | 41 | 51 | 61 | 71 |
| 2 | 20 | 20 | 22 | 32 | 42 | 52 | 62 | 72 |
| 3 | 20 | 20 | 23 | 33 | 43 | 53 | 63 | 73 |
| 4 | 20 | 20 | 02 | 34 | 44 | 54 | 64 | 74 |
| 5 | 20 | 20 | 25 | 35 | 45 | 55 | 65 | 75 |
| 6 | 20 | 20 | 26 | 36 | 46 | 56 | 66 | 76 |
| 7 | 20 | 20 | 27 | 37 | 47 | 57 | 67 | 77 |
| 8 | backspace | 20 | 28 | 38 | 48 | 58 | 68 | 78 |
| 9 | 20 | 20 | 29 | 39 | 49 | 59 | 69 | 79 |
| A | 0A | Submit | 2A | 3A | 4A | 5A | 6A | 7A |
| B | 20 | Cancel | 2B | 3B | 4B | 1B3C | 6B | 1B28 |

| C | 20 | 20 | 2C | 3C | 4C | 1B2F | 6C | 1B40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D | 0D | 20 | 2D | 3D | 4D | 1B3E | 6D | 1B29 |
| E | 20 | 20 | 2E | 3E | 4E | 1B14 | 6E | 1B3D |
| F | 20 | 20 | 2F | 3F | 4F | 11 | 6F | 20 |

#### Table 26: IRA Extended Characters

| A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| 0 20 20 20 20 |  |  |  | 7F | 20 |

1	40	20	20	5D	20	7D

2	20	20	20	20	20	08

3	01	20	20	20	20	20

4	24	20	5B	20	7B	20

5	03	20	0E	20	0F	20

6	20	20	1C	5C	1D	7C

7	5F	20	09	20	20	20

8	20	20	20	0B	04	0C

9	20	20	1F	20	05	06

A	20	20	20	20	20	20

B	20	20	20	20	20	20

C	20	20	20	5E	07	7E

D	20	20	20	20	20	20

E	20	20	20	20	20	20

F	20	60	20	1E	20	20

#### Table 27: The Output Conversions Table (DCS=GSM 7 bit and AT+CSCS=“IRA”)

0

1

2

3

4

5

6

7

| 0 | 40 | 20 | 20 | 30 | A1 | 50 | BF | 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A3 | 5F | 21 | 31 | 41 | 51 | 61 | 71 |
| 2 | 24 | 20 | 22 | 32 | 42 | 52 | 62 | 72 |
| 3 | A5 | 20 | 23 | 33 | 43 | 53 | 63 | 73 |
| 4 | E8 | 20 | A4 | 34 | 44 | 54 | 64 | 74 |
| 5 | E9 | 20 | 25 | 35 | 45 | 55 | 65 | 75 |
| 6 | F9 | 20 | 26 | 36 | 46 | 56 | 66 | 76 |
| 7 | EC | 20 | 27 | 37 | 47 | 57 | 67 | 77 |
| 8 | F2 | 20 | 28 | 38 | 48 | 58 | 68 | 78 |
| 9 | C7 | 20 | 29 | 39 | 49 | 59 | 69 | 79 |
| A | 0D0A |  | 2A | 3A | 4A | 5A | 6A | 7A |
| B | D8 |  | 2B | 3B | 4B | C4 | 6B | E4 |
| C | F8 | C6 | 2C | 3C | 4C | D6 | 6C | F6 |
| D | 0D | E6 | 2D | 3D | 4D | D1 | 6D | F1 |
| E | C5 | DF | 2E | 3E | 4E | DC | 6E | FC |
| F | E5 | C9 | 2F | 3F | 4F | A7 | 6F | E0 |

#### Table 28: GSM Extended Characters

0	1	2	3	4	5	6	7

0	7C

1

2

3

4	5E

5

6

| 7 |  |  |
| --- | --- | --- |
| 8 | 7B |  |
| 9 | 7D |  |
| A |  |  |
| B |  |  |
| C |  | 5B |
| D |  | 7E |
| E |  | 5D |
| F | 5C |  |

Because the low 8 bit of UCS2 character is the same as the IRA character:

The conversion table of DCS=GSM 7 bit and AT+CSCS=“UCS2” is similar to AT+CSCS=“IRA”. The conversion table of fmt=GSM 7 bit and AT+CSCS=“GSM” is similar to AT+CSCS=“GSM”. The conversion table of fmt= GSM 7 bit and AT+CSCS=“IRA” is similar to AT+CSCS=“IRA”.

The conversion table of fmt=GSM 7 bit and AT+CSCS=“UCS2” is similar to AT+CSCS=“IRA”. The difference is the way of SMS text input or output. Please refer to Table 21 for more details.

## Release Cause Text List of AT+CEER

#### Table 29: Release Cause Text List of AT+CEER

CS Internal Cause

No cause information available (default)

Phone is offline

No service available

Network release, no reason given

Received incoming call

Client ended call

UIM not present

Access attempt already in progress

Access failure, unknown source

Concur service not supported by network

No response received from network

GPS call ended for user call

SMS call ended for user call

Data call ended for emergency call

Rejected during redirect or handoff

Lower-layer ended call

Call origination request failed

Client rejected incoming call

Client rejected setup indication

Network ended call

No funds available

No service available

Full service not available

Maximum packet calls exceeded

Video connection lost

Video protocol closed after setup

Video protocol setup failure

Internal error

CS Network Cause

Unassigned/unallocated number

No route to destination

Channel unacceptable

Operator determined barring

Normal call clearing

User busy

No user responding

User alerting, no answer

Call rejected

Number changed

Non selected user clearing

Destination out of order

Invalid/incomplete number

Facility rejected

Response to status enquiry

Normal, unspecified

No circuit/channel available

Network out of order

Temporary failure

Switching equipment congestion

Access information discarded

Requested circuit/channel not available

Resources unavailable, unspecified

Quality of service unavailable

Requested facility not subscribed

Incoming calls barred within the CUG

Bearer capability not authorized

Bearer capability not available

Service/option not available

Bearer service not implemented

ACM >= ACM max

Requested facility	not implemented

Only RDI bearer is available

Service/option not implemented

Invalid transaction	identifier value

User not member of CUG

Incompatible destination

Invalid transit network selection

Semantically incorrect message

Invalid mandatory information

Message non-existent/not implemented

Message type not compatible with state

IE non-existent/not implemented

Conditional IE error

Message not compatible with state

Recovery on timer expiry

Protocol error, unspecified

Interworking, unspecified

CS Network Reject

IMSI unknown in HLR

Illegal MS

IMSI unknown in VLR

IMEI not accepted

Illegal ME

GPRS services not allowed

GPRS and non GPRS services not allowed

MS identity cannot be derived

Implicitly detached

PLMN not allowed

Location area not allowed

Roaming not allowed

GPRS services not allowed in PLMN

No suitable cells in location area

MSC temporary not reachable

Network failure

MAC failure

Synch failure

Congestion

GSM authentication unacceptable

Service option not supported

Requested service option not subscribed

Service option temporary out of order

Call cannot be identified

No PDP context activated

Semantically incorrect message

Invalid mandatory information

Message type non-existent

Message type not compatible with state

Information element non-existent

Message not compatible with state

RR release indication

RR random access failure

RRC release indication

RRC close session indication

RRC open session failure

Low level failure

Low level failure no redial allowed

Invalid SIM

No service

Timer T3230 expired

No cell available

Wrong state

Access class blocked

Abort message received

Other causes

Timer T303 expired

No resources

Release pending

Invalid user data

PS Internal Cause

Invalid connection identifier

Invalid NSAPI

Invalid primary NSAPI

PDP establish timeout

Invalid field

SNDCP failure

RAB setup failure

No GPRS context

PDP activate timeout

PDP modify timeout

PDP inactive max timeout

PDP lower layer error

PDP duplicate

Access technology change

PDP unknown reason

CS PS Network Cause

LLC or SNDCP failure

Insufficient resources

Missing or unknown APN

Unknown PDP address or PDP type

User authentication failed

Activation rejected by GGSN

Activation rejected, unspecified

Service option not supported

Requested service option not subscribed

Service option temporary out of order

NSAPI already used (not sent)

Regular deactivation

QoS not accepted

Network failure

Reactivation required

Feature not supported

Semantic error in the TFT operation

Syntactical error in the TFT operation

Unknown PDP context

PDP context without TFT already activated

Semantic errors in packet filter

Syntactical errors in packet filter

Invalid transaction identifier

Semantically incorrect message

Invalid mandatory information

Message non-existent/not implemented

Message type not compatible with state

IE non-existent/not implemented

Conditional IE error

Message not compatible with state

Protocol error, unspecified